| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0633 |
| --- | --- |

| Title: | Crosscheck of JVET-O0355 (Non CE3: Simplified coding of chroma intra modes) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yucheng Sun Fangdong Chen Li Wang | Tel: Email: | +86 18105755698 sunyucheng@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

In this contribution, the crosscheck results of JVET-P0355 are reported. The patch is provided by BBC. To verify, a simulation was carried out using the provided software. For the software, the code review was done. For simulation results for 1 tests, it’s confirmed that the BD-rate results match with those in JVET- P0355. And the encoding and decoding run time is similar.

Verification

The software to test the techniques as introduced in JVET-P0355 [1] was provided by BBC. For the provided software, it was confirmed as following.

- The software is implemented based on VTM-6.0.

- The modification is to reduce the number of modes in chroma candidate list that are explicitly signalled from four to two.

- One test results are generated.

  - Anchor: VTM6.0

  - Test: Horizontal and vertical modes are removed from the chroma candidate list

- For simulation result for above test, the BD-rate results are the same as reported in [1] as reported below.

- For the run time for encoding and decoding, it’s similar to those in [1]

Experiment

JVET common test condition [2] is followed. For the simulations, same platform machines as below in PC cluster were used.

- Encode

  - OS: Red Hat 4.8.5

  - Compiler: gcc 4.8.5

  - CPU: Xeon E5-2680 v4

  - SIMD: AVX2

- Decode

  - OS: Red Hat 4.8.5

  - Compiler: gcc 4.8.5

  - CPU: Xeon E5-2680 v4

  - SIMD: AVX2

For a runtime measurement, the user time in the output log was used. For a random-access case and an all intra case, a parallel encoding is used. To measure encoding time in parallel encoding, the total of all user time for each frame were calculated.

Table 1.  Results of the proposed Test

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.73% | 0.74% | 97% | 100% |
| Class A2 | -0.04% | 0.83% | 0.92% | 96% | 100% |
| Class B | 0.02% | 0.65% | 0.66% | 98% | 100% |
| Class C | 0.02% | 0.35% | 0.50% | 97% | 100% |
| Class E | 0.04% | 0.92% | 0.94% | 98% | 100% |
| Overall | 0.00% | 0.67% | 0.73% | 97% | 100% |
| Class D | 0.02% | 0.46% | 0.43% | 96% | 97% |
| Class F | 0.07% | 0.46% | 0.60% | 98% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.17% | 0.29% | 98% | 99% |
| Class A2 | 0.02% | 0.44% | 0.43% | 98% | 99% |
| Class B | 0.00% | 0.53% | 0.47% | 98% | 100% |
| Class C | 0.00% | 0.37% | 0.24% | 98% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.40% | 0.36% | 98% | 100% |
| Class D | -0.06% | 0.08% | 0.38% | 98% | 102% |
| Class F | 0.05% | 0.27% | 0.37% | 98% | 101% |

Reference

- Saverio Blasi, Andre Seixas Dias, Gosala Kulupana, “Non CE3: Simplified coding of chroma intra modes,” JVET-P0355, Geneva, CH, 1–11 October 2019.

- F. Bossen,J. Boyce, X. Li, V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Geneva, CH, 19–27 March. 2019.

Patent rights declaration(s)

Hikvision Corporation does not have any current or pending patent rights relating to the technology described in this contribution.