| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0637 |
| --- | --- |

| Title: | CE6-related: On measuring reconstruction error of LFNST matrices |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xin Zhao | Tel: Email: | xinzzhao@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In this contribution, different methods for analyzing the “orthogonality” of LFNST kernels are studied together with their connections to coding performance.

# Introduction

An orthogonal matrix is a square matrix whose columns and rows are orthogonal unit vectors (i.e., orthonormal vectors) [1], i.e,

QT∙Q=Q∙QT=I,

where I is an identity matrix. In the scenario of video codec development, the orthogonality of primary transform kernels, which are square matrices, have been tuned for better orthogonality [2]. The orthogonality of primary transform is asserted to be important because primary transform cannot be avoided to achieve meaningful coding efficiency on natural video sequences, and better orthogonality leads to less transform reconstruction error which are typically more visible for low QP conditions [3].

However, for secondary transform in VVC, the larger LFNST matrices (16x48) are rectangular matrices since high-frequencies are zeroed out, which means LFNST matrix cannot be orthogonal matrix. In this contribution, instead of measuring orthogonality which is not clearly defined for rectangular matrix or non-invertible square matrix (zero out), a general term Transform Reconstruction Error (TRE) is used to describe the impact on the input signal after applying forward and inverse transform sequentially, which simulates the encode – decode process.

For secondary transform in VVC, i.e., LFNST, it is asserted in this contribution that, the “orthogonality” (or TRE) of secondary transform may be not so important considering two facts:

- The encoder always has the block-level option of not selecting LFNST.

  - In case LFNST yields a large reconstruction error caused by secondary transform process, the encoder always has the option not selecting secondary transform.

- The LFNST process zero-out higher frequency coefficients.

  - Due to the zero-out scheme, LFNST matrices are not invertible, which means LFNST matrices cannot be orthogonal, and therefore reconstruction error cannot be avoided.

In the following two subsections, the TRE of secondary transform is studied from two perspectives, one is the experimental perspective and the other is the analytical perspective.

## Experimental perspective

Due to complexity considerations, the zeroing out scheme is applied in secondary transform [4] to reduce 1) the memory cost for storing transform matrices, and 2) the worst-case and average multiplications per coefficient. The adopted design of secondary transform in VVC Draft D6 is the outcome of a compromise between transform precision (TRE) and coding performance. Through multiple cycles of experiments, it turns out that, by zeroing out even 75% of all primary coefficients, secondary transform still provides a promising trade-off between transform precision and coding performance.

Nevertheless, sacrificing transform precision of LFNST matrices does not give zero penalty on coding performance. In JVET-K0099 [5], two tests using different settings of R are reported, as described below,

Table 1 Two tests described in JVET-K0099.

| Test | Coding performance | Memory cost |
| --- | --- | --- |
| VTM + RST (R=32) | AI: -5.44% (Y), -5.13% (U), -5.40% (V) RA: -2.81% (Y), -3.75% (U), -4.07% (V) | 80.6 KB |
| VTM + RST (R=16) | AI: -4.97% (Y), -4.54% (U), -4.80% (V) RA: -2.53% (Y), -3.13% (U), -3.40% (V) | 53.8 KB |

The differences between above two tests are the secondary transform matrix size. In the top test (R=32), 32x64 secondary transform matrices are applied, and the below test (R=16), 16x64 secondary transform matrices are applied. It is asserted that a greater value of R yields a higher transform precision, thus lower TRE. However, by sacrificing the reconstruction accuracy (reduce from 32 to 16), around 27 KB memory can be saved, and multiplication operations can be halved, with a penalty of ~1/10 of the coding gain.

Evidenced by the above history of LFNST development, it is asserted in this contribution that, TRE has relative low impact on the coding performance for LFNST.

## Analytical perspective

Besides the experimental data that are available, in this section, the TRE of different LFNST approaches are also studied in pure analytical ways. The correlation between different analytical measurements and the actual coding performance is studied.

Basically, three analytical measurements have been applied in several input documents of this upcoming Geneva meeting [5][6][7], as describe below,

C1=AT・A - ININ									(1)

C2=A・AT - I16I16									(2)

MSE(A) =|| AAT - I ||F2N2,   SNR(T) = 20log10(1/MSE(A))			(3)

where A is the LFNST matrix (16x48) and I is the identity matrix.

To study the relationship between the above three measurements and the actual coding performance, by varying the R of LFNST matrix, i.e., further reduced Nx48 matrices are used where N≤16, both C1 and C2 measurements are evaluated as well as the coding gain of the associated LFNST on top of VTM-6.0. The comparisons are shown in Table 2. In Table 2, the average measurement values are reported for all LFNST matrices, and in Table 3, 4 and 5, detailed measurement values (for each LFSNT matrix) are reported for C1, C2 and SNR, respectively.

The BD-rate is measured using VTM6 with AI test condition defined in CTC [9], with --FramesToBeEncoded=50. The anchor is VTM6 and the test is VTM6 with Nx48 matrices, where N is specified in the first column of Table 2, and first row of Table 3, 4 and 5.

Table 2 Comparison on the C1, C2 and coding gain measurements.

| N | C1 [6] | C2 * 100 [6][7] | SNR [8] | BD-rate * 100 |
| --- | --- | --- | --- | --- |
| 16 | 1.46 | 4.21 | 49.64 | 1.15 |
| 15 | 1.46 | 3.97 | 49.56 | 1.12 |
| 14 | 1.45 | 3.72 | 49.54 | 1.10 |
| 13 | 1.45 | 3.45 | 49.56 | 1.05 |
| 12 | 1.41 | 3.15 | 49.66 | 1.00 |
| 11 | 1.42 | 2.83 | 49.84 | 0.94 |
| 10 | 1.40 | 2.58 | 49.74 | 0.89 |
| 9 | 1.41 | 2.38 | 49.57 | 0.82 |
| 8 | 1.40 | 2.20 | 49.41 | 0.73 |
| 7 | 1.37 | 1.92 | 49.40 | 0.59 |
| 6 | 1.33 | 1.61 | 49.62 | 0.50 |
| 5 | 1.30 | 1.42 | 49.44 | 0.38 |
| 4 | 1.25 | 1.14 | 49.59 | 0.26 |
| 3 | 1.20 | 0.84 | 49.71 | 0.17 |
| 2 | 1.13 | 0.53 | 51.32 | 0.08 |
| 1 | 1.06 | 0.23 | 55.61 | 0.08 |

It is noted that, for lower C1 or C2 values, or higher SNR values, which are assumed to indicate a more “orthogonal” matrix, the associated LFNST matrices can lead to even lower coding gains.

Table 3 Evaluation of different LFNST matrices using C1 criterion [6].

| N | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1.21 | 1.19 | 1.21 | 1.22 | 1.25 | 1.22 | 1.20 | 1.24 | 1.23 | 1.24 | 1.19 | 1.21 | 1.17 | 1.12 | 1.10 | 1.04 |
| 1 | 1.22 | 1.22 | 1.23 | 1.26 | 1.21 | 1.21 | 1.23 | 1.25 | 1.30 | 1.26 | 1.18 | 1.22 | 1.21 | 1.12 | 1.13 | 1.04 |
| 2 | 1.59 | 1.55 | 1.59 | 1.60 | 1.56 | 1.57 | 1.47 | 1.47 | 1.49 | 1.43 | 1.36 | 1.35 | 1.26 | 1.24 | 1.13 | 1.05 |
| 3 | 1.91 | 1.90 | 1.87 | 1.83 | 1.77 | 1.79 | 1.80 | 1.82 | 1.76 | 1.64 | 1.60 | 1.49 | 1.46 | 1.35 | 1.23 | 1.11 |
| 4 | 1.10 | 1.05 | 1.04 | 1.07 | 1.07 | 1.10 | 1.10 | 1.11 | 1.07 | 1.11 | 1.13 | 1.09 | 1.09 | 1.05 | 1.05 | 1.02 |
| 5 | 1.49 | 1.52 | 1.56 | 1.49 | 1.37 | 1.36 | 1.39 | 1.41 | 1.43 | 1.48 | 1.36 | 1.34 | 1.26 | 1.26 | 1.14 | 1.08 |
| 6 | 1.60 | 1.61 | 1.53 | 1.52 | 1.48 | 1.54 | 1.46 | 1.51 | 1.48 | 1.41 | 1.37 | 1.33 | 1.24 | 1.26 | 1.14 | 1.05 |
| 7 | 1.57 | 1.63 | 1.58 | 1.60 | 1.55 | 1.54 | 1.54 | 1.46 | 1.47 | 1.41 | 1.42 | 1.33 | 1.30 | 1.22 | 1.13 | 1.06 |
| Avg | 1.46 | 1.46 | 1.45 | 1.45 | 1.41 | 1.42 | 1.40 | 1.41 | 1.40 | 1.37 | 1.33 | 1.30 | 1.25 | 1.20 | 1.13 | 1.06 |

Table 4 Evaluation of different LFNST matrices using C2 criterion (amplified by 100) [6][7].

| N | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 4.32 | 4.05 | 3.68 | 3.48 | 3.04 | 2.84 | 2.51 | 2.02 | 1.99 | 1.77 | 1.48 | 1.31 | 0.96 | 0.73 | 0.45 | 0.10 |
| 1 | 3.89 | 3.61 | 3.41 | 3.15 | 2.93 | 2.70 | 2.42 | 2.16 | 2.07 | 1.80 | 1.57 | 1.22 | 1.25 | 1.07 | 0.87 | 0.59 |
| 2 | 4.21 | 3.93 | 3.70 | 3.51 | 3.38 | 3.16 | 2.91 | 2.80 | 2.44 | 2.09 | 1.69 | 1.67 | 1.31 | 0.64 | 0.13 | 0.04 |
| 3 | 4.58 | 4.40 | 4.11 | 3.77 | 3.42 | 2.86 | 2.76 | 2.80 | 2.61 | 2.31 | 2.10 | 1.65 | 1.29 | 1.06 | 0.62 | 0.12 |
| 4 | 4.08 | 3.91 | 3.77 | 3.57 | 3.12 | 2.74 | 2.44 | 2.34 | 1.97 | 1.68 | 1.44 | 1.36 | 1.19 | 0.79 | 0.60 | 0.23 |
| 5 | 4.24 | 4.14 | 3.79 | 3.33 | 2.98 | 2.67 | 2.50 | 2.29 | 2.35 | 1.91 | 1.38 | 0.99 | 0.72 | 0.67 | 0.36 | 0.49 |
| 6 | 4.38 | 4.10 | 3.89 | 3.57 | 3.46 | 3.14 | 2.65 | 2.41 | 2.15 | 2.13 | 1.66 | 1.51 | 1.38 | 1.07 | 0.94 | 0.08 |
| 7 | 3.95 | 3.64 | 3.38 | 3.24 | 2.89 | 2.56 | 2.44 | 2.22 | 1.99 | 1.70 | 1.52 | 1.61 | 1.04 | 0.73 | 0.24 | 0.21 |
| Avg | 4.21 | 3.97 | 3.72 | 3.45 | 3.15 | 2.83 | 2.58 | 2.38 | 2.20 | 1.92 | 1.61 | 1.42 | 1.14 | 0.84 | 0.53 | 0.23 |

Table 5 Evaluation of different LFNST matrices using “SNR” criterion [8].

| N | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 49.63 | 49.59 | 49.71 | 49.50 | 49.94 | 49.88 | 49.90 | 50.85 | 50.24 | 50.11 | 50.27 | 49.94 | 50.86 | 50.92 | 51.64 | 60.21 |
| 1 | 50.53 | 50.53 | 50.58 | 50.61 | 50.51 | 50.50 | 50.55 | 50.43 | 49.99 | 50.03 | 49.92 | 51.03 | 49.45 | 48.47 | 46.89 | 44.55 |
| 2 | 49.32 | 49.28 | 49.09 | 48.99 | 48.90 | 48.85 | 48.58 | 48.14 | 48.36 | 48.40 | 49.15 | 48.16 | 47.92 | 50.40 | 60.45 | 67.39 |
| 3 | 48.81 | 48.64 | 48.84 | 49.00 | 49.12 | 49.85 | 49.44 | 48.72 | 48.50 | 48.57 | 48.06 | 48.60 | 48.62 | 47.55 | 47.41 | 58.27 |
| 4 | 49.96 | 49.82 | 49.54 | 49.41 | 49.91 | 50.26 | 50.19 | 49.80 | 50.60 | 50.77 | 50.46 | 49.96 | 49.40 | 51.29 | 49.96 | 52.92 |
| 5 | 49.44 | 49.12 | 49.23 | 49.62 | 49.83 | 50.05 | 49.78 | 49.55 | 48.69 | 48.98 | 50.51 | 51.19 | 52.72 | 51.02 | 51.28 | 46.12 |
| 6 | 49.37 | 49.30 | 49.19 | 49.41 | 49.03 | 49.11 | 49.71 | 49.48 | 49.45 | 48.58 | 49.13 | 48.63 | 47.67 | 47.65 | 45.61 | 62.01 |
| 7 | 50.03 | 50.17 | 50.14 | 49.93 | 50.05 | 50.23 | 49.81 | 49.58 | 49.48 | 49.73 | 49.43 | 48.02 | 50.05 | 50.39 | 57.31 | 53.41 |
| Avg | 49.64 | 49.56 | 49.54 | 49.56 | 49.66 | 49.84 | 49.74 | 49.57 | 49.41 | 49.40 | 49.62 | 49.44 | 49.59 | 49.71 | 51.32 | 55.61 |

# Conclusions

In this contribution, the reconstruction error of LFNST matrices are studied through past experimental data (CE tests) and analytical approaches. It is asserted that, the C1, C2 and SNR measurements are not reliable as an indication on the coding performance of LFNST matrices.

# References

- https://en.wikipedia.org/wiki/Orthogonal_matrix.

- M. Budagavi,, A. Fuldseth,, G. Bjøntegaard, V. Sze, M. Sadafale, “Core Transform Design in the High Efficiency Video Coding (HEVC) Standard”, IEEE Trans. Circuits Systems Video Technol., vol.7, no.6, pp.1029-1041, Dec. 2013.

- A. Fuldseth, G. Bjøntegaard, M. Sadafale, M. Budagavi, “Transform design for HEVC with 16 bit intermediate data representation”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Doc. JCTVC-E243, 5th Meeting: Geneva, CH, 16-23 March, 2011.

- M. Koo, J. Lim, M. Salehifar and S. H. Kim, “CE6: Reduced Secondary Transform (RST) (CE6-3.1)”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-N0193, 14th JVET meeting, Geneva, 19–27 Mar. 2019.

- M. Salehifar, M. Koo, S. H. Kim and J. Lim, “CE 6.2.6: Reduced Secondary Transform (RST)”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-K0099, 11th Meeting: Ljubljana, SI, 10–18 July 2018.

- T. Zhou, T. Hashimoto, T. Ikai, “CE6-2.3a: Simplification with new LFNST transform basis”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-P0065, Geneva, CH, 1–11 October 2019.

- J. Gan, C. Rosewarne, “Non-CE6: On LFNST reduced kernels”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-P0259, Geneva, CH, 1–11 October 2019.

- H. E. Egilmez, “Crosscheck of CE6-2.2a and CE6-2.2d: An Orthogonality Analysis”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-P0594, Geneva, CH, 1–11 October 2019.

- F. Bossen, J. Boyce, K. Sühring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-N1010, 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

# Patent rights declaration(s)

Tencent does not have any current or pending patent rights relating to the technology described in this contribution.