| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0665 |
| --- | --- |

| Title: | Non-CE5: Spec fix for ALF filter and transpose index calculation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA, 92121, USA | Tel: Email: | +1(858)651-3483 nanh@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In section 8.8.5.3, the samples in a coding tree block (CTB) are filled before calculating the filter and transpose indices for the CTB and are not changed. However, if there is a boundary (virtual boundary or 360 boundary) in the CTB, some samples may have different values when used for blocks located at different sides of a boundary. This contribution is proposed to fix the problem.

# Introduction

In VVC draft 6, when calculating the filter and transpose indices for a coding tree block (CTB) in section 8.8.5.3, the samples in the whole CTB are first filled. Then the 2-D Laplacian values of in the CTB are calculated based on these filled samples. After that, the filter and transpose indices are derived at each 4x4 block level. However, when a boundary crosses inside the CTB, there is problem for sample and Laplacian values along the boundary.

For example, as shown in the figure below. There is a horizontal boundary in a CTB, two neighboring 4x4 blocks A and B are located at different sides of the boundary. When calculating the filter and transpose indices for block B, the values of the samples next to the boundary are shown in black. When calculating the filter and transpose indices for block A, for those same heighted samples for block B, different values (shown in red) should be used by repetitive padding from block A. However, based on VVC draft 6, those sample values used for block B (shown in black) are used for block A again, which is not correct.

# Proposed change

To fix this problem, to avoid the interaction between neighbouring blocks located at different sides of a boundary, pixel filling and Laplacian values calculation are performed at each 4x4 block level instead of being performed together at CTB level.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).