| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0667-v1 |
| --- | --- |

| Title: | AHG17/AHG15: Comments on aspect 1 of JVET-P0426. |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bappaditya Ray, A. K. Ramasubramonian, Geert Van der Auwera, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Email: | bray@qti.qualcomm.com aramasub@qti.qualcomm.com geertv@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes some comments on the solution to aspect 1 of JVET-P0426 which aligns various QP control parameters related to joint Cb-Cr residual (JCCR) coding and provides an alternative that assertedly does not contain any parsing dependency issues.

# Introduction

Aspect 1 of JVET-P0426, proposes to remove the signalling for various QP parameters that are related to joint Cb-Cr residual (JCCR) coding in the SPS and PPS when sps_joint_cbcr_enabled_flag is zero [2].

Two issues related to the reported solution of aspect 1 of JVET-P0426 are discussed here:

- Currently, at the SPS level, sps_joint_cbcr_enabled_flag is parsed after the parsing of chroma QPmapping table(s). So, the desired outcome of the proposed aspect (removing chroma QPmapping table for JCCR) may not be achieved unless the parsing order is modified.

- At the PPS level, parsing of PPS syntax elements pps_joint_cbcr_qp_offset and joint_cbcr_qp_offset_list[ i ] depends on the SPS syntax element sps_joint_cbcr_enabled_flag. However, one of the design principles of VVC is to avoid parameter set parsing dependency. In this case, PPS parameters depend on SPS parameter which violates this design principle.

# Proposal

Two items are proposed:

- We propose to parse sps_joint_cbcr_enabled_flag before parsing of chroma QP mapping table. Accordingly, the parsing order of sps_joint_cbcr_enabled_flag is modified.

- We add a flag pps_joint_cbcr_qp_offset_present_flag to control whether the QP offset for the joint Cb-Cr residuals is included in the chroma QP offset table and whether pps_joint_cbcr_qp_offset is signalled. As opposed to the approach of JVET-P0426, this approach does not have any parsing dependency.

The changes compared to JVET-O2001-vE [1] are marked below

| sps_max_luma_transform_size_64_flag | u(1) |
| --- | --- |
| sps_joint_cbcr_enabled_flag | u(1) |
| if( ChromaArrayType != 0 ) { |  |
| same_qp_table_for_chroma | u(1) |
| for( i = 0; i < same_qp_table_for_chroma ? 1 : (sps_joint_cbcr_enabled_flag)? 3: 2 ; i++ ) { |  |
| num_points_in_qp_table_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_points_in_qp_table_minus1[ i ]; j++ ) { |  |
| delta_qp_in_val_minus1[ i ][ j ] | ue(v) |
| delta_qp_out_val[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| sps_weighted_pred_flag | u(1) |
| sps_weighted_bipred_flag | u(1) |
| sps_sao_enabled_flag | u(1) |
| sps_alf_enabled_flag | u(1) |
| sps_transform_skip_enabled_flag | u(1) |
| if( sps_transform_skip_enabled_flag ) |  |
| sps_bdpcm_enabled_flag | u(1) |

| pps_cb_qp_offset | se(v) |
| --- | --- |
| pps_cr_qp_offset | se(v) |
| pps_joint_cbcr_qp_offset_present_flag | u(1) |
| if( pps_joint_cbcr_qp_offset_present_flag ) |  |
| pps_joint_cbcr_qp_offset | se(v) |
| pps_slice_chroma_qp_offsets_present_flag | u(1) |
| cu_chroma_qp_offset_enabled_flag | u(1) |
| if( cu_chroma_qp_offset_enabled_flag ) { |  |
| cu_chroma_qp_offset_subdiv | ue(v) |
| chroma_qp_offset_list_len_minus1 | ue(v) |
| for( i = 0; i <= chroma_qp_offset_list_len_minus1; i++ ) { |  |
| cb_qp_offset_list[ i ] | se(v) |
| cr_qp_offset_list[ i ] | se(v) |
| if( pps_joint_cbcr_qp_offset_present_flag ) |  |
| joint_cbcr_qp_offset_list[ i ] | se(v) |
| } |  |
| } |  |

pps_joint_cbcr_qp_offset_present_flag equal to 1 specifies that pps_joint_cbcr_qp_offset and joint_cbcr_qp_offset_list[ i ] are present in the PPS RBSP syntax structure. pps_joint_cbcr_qp_offset_present_flag equal to 0 specifies that pps_joint_cbcr_qp_offset and joint_cbcr_qp_offset_list[ i ] are not present in the PPS RBSP syntax structure. When ChromaArrayType is equal to 0 or sps_joint_cbcr_enabled_flag is equal to 0, the value of pps_joint_cbcr_qp_offset_presnt_flag shall be equal to 0.

pps_joint_cbcr_qp_offset specifies the offset to the luma quantization parameter Qp′Y used for deriving Qp′CbCr. The value of pps_joint_cbcr_qp_offset shall be in the range of −12 to +12, inclusive.. When pps_joint_cbcr_qp_offset_present_flag is equal to 0, pps_joint_cbcr_qp_offset is not present and is inferred to be equal to 0.

cb_qp_offset_list[ i ], cr_qp_offset_list[ i ], and joint_cbcr_qp_offset_list[ i ], specify offsets used in the derivation of Qp′Cb, Qp′Cr, and Qp′CbCr, respectively. The values of cb_qp_offset_list[ i ], cr_qp_offset_list[ i ], and joint_cbcr_qp_offset_list[ i ] shall be in the range of −12 to +12, inclusive. When pps_joint_cbcr_qp_offset_present_flag is equal to 0, joint_cbcr_qp_offset_list[ i ] is not present and is inferred to be equal to 0.

# References

[1] Bross et al., “Versatile Video Coding (Draft 6),” Joint Video Experts Team, doc. JVET-O2001, Gothenburg, SE, Jul. 2019.

[2] R. Chernyak et al.., “AHG17/AHG15: On quantization control parameters signalling”, Document of Joint Video Experts Team, JVET-P0426, Geneva, CH, Oct. 2019.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).