| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0670-v1 |
| --- | --- |

| Title: | Non-CE8: On num_palette_indices_minus1 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weijia Zhu, Li Zhang, Jizheng Xu 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-360-525-0535 weijia.zhu@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In the palette mode, the number of palette indices explicitly signalled or inferred for the current block minus 1 is coded which is denoted by num_palette_indices_minus1 in the specification. However, if the number is smaller than the current palette size plus escape value present flag (i.e., palette_escape_val_present_flag set to 1 if there are escape-coded symbols, otherwise, set to 0), it means either some palette entries are coded but not used by current block or palette_escape_val_present_flag was wrongly sent. On the other hand, the number shall be no greater than the number of samples in current block. To sovle these issues, in this document, two modifications are proposed: 1) the number minus current plattte size plus palette_escape_val_present_flag is signalled instead of num_palette_indices_minus1; 2) an encoder constraint is added that the signalled value shall be no greater than CU size minus (the current palette size plus palette_escape_val_present_flag). Following the CE8 test conditions, simulation results of proposed methods on top of JVET-P0472 are reported to be as follows:

CTC QPs = {22, 27, 32, 37}

Dual tree on:

TGM1080P: AI: -0.11%;RA: -0.11%; LDB: XXX%

TGM720P: AI: -0.04%; RA: -0.06%; LDB: -0.13%

Dual tree off:

TGM1080P: AI: XXX%; RA: -0.13%; LDB: XXX%

TGM720P: AI: -0.13%; RA: -0.10%; LDB: XXX%

# Introduction

In the palette mode, the number of palette indices explicitly signalled or inferred for the current block minus 1 is coded which is denoted by num_palette_indices_minus1 in the specification. However, if the number is smaller than the current palette size plus escape value present flag, it means either palette entries redudancy(some palette entries are coded but not used by current block) or palette_escape_val_present_flag was wrongly. Therefore, the number should be larger than the palette size plus the value of escape flag minus 1.

On the other hand, the decoder behavior is undefined if the number is greater than the number of samples in current block minus 1.

Therefore, if the current CU with size S (i.e., block width × block height) has N palette entries and its escaple value present flag is true, the number of palette indices explicitly signalled or inferred should be in the range [N+1, S – (N + 1)]. Otherwise, if the escape escaple value present flag is false, the number of palette indices explicitly signalled or inferred should be in the range [N, S – N].

# Proposed Method

This document proposes two modifications:

1) num_palette_indices is signalled to remove the redundant bits. It is set equal to the number of palette indices explicitly signalled or inferred minus (MaxPaletteIndex plus 1). The MaxPaletteIndex plus 1 is equal to (NumPredictedPaletteEntries + num_signalled_palette_entries + palette_escape_val_present_flag)

2) The num_palette_indices is constrained to be no greater than the CU size minus (MaxPaletteIndex plus 1). Please note that this modificaiton does not affect the coding performance.

# Simulation Results

The proposed method is evaluated with VTM-6.0. The simulation follows the CE 8 test condition [1]. The detailed results are given in Table 1-6. More detailed results could be found in the attached excel sheets.

Table 1.  Coding performance of the proposed scheme under AI configuration with dual tree on

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.11% | -0.10% | -0.09% | 100% | 99% |
| TGM 720p | -0.04% | -0.02% | -0.09% | 99% | 100% |
| Animation |  |  |  |  |  |
| Mixed content | -0.06% | -0.05% | -0.06% | 99% | 99% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall |  |  |  |  |  |

Table 2.  Coding performance of the proposed scheme under RA configuration with dual tree on

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.11% | -0.19% | -0.19% | 99% | 99% |
| TGM 720p | -0.06% | -0.10% | -0.04% | 99% | 99% |
| Animation | -0.02% | -0.07% | -0.09% | 99% | 99% |
| Mixed content | -0.03% | -0.14% | -0.06% | 100% | 99% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Overall | -0.05% | -0.11% | -0.09% | 99% | 99% |

Table 3.  Coding performance of the proposed scheme under LDB configuration with dual tree on

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p |  |  |  |  |  |
| TGM 720p | -0.13% | -0.20% | -0.23% | 101% | 95% |
| Animation |  |  |  |  |  |
| Mixed content | 0.06% | -0.07% | -0.17% | 100% | 99% |
| Camera-Captured content |  |  |  |  |  |
| Overall |  |  |  |  |  |

Table 4.  Coding performance of the proposed scheme under AI configuration with dual tree off

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p |  |  |  |  |  |
| TGM 720p | -0.13% | -0.21% | -0.21% | 100% | 98% |
| Animation |  |  |  |  |  |
| Mixed content | -0.08% | -0.21% | -0.16% | 100% | 99% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Overall |  |  |  |  |  |

Table 5.  Coding performance of the proposed scheme under RA configuration with dual tree off

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.13% | -0.20% | -0.20% | 100% | 100% |
| TGM 720p | -0.10% | -0.20% | -0.21% | 98% | 98% |
| Animation | -0.08% | -0.02% | -0.10% | 99% | 100% |
| Mixed content | -0.01% | 0.03% | -0.06% | 99% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Overall | -0.08% | -0.10% | -0.13% | 99% | 99% |

Table 6.  Coding performance of the proposed scheme under LDB configuration with dual tree off

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p |  |  |  |  |  |
| TGM 720p |  |  |  |  |  |
| Animation |  |  |  |  |  |
| Mixed content |  |  |  |  |  |
| Camera-Captured content |  |  |  |  |  |
| Overall |  |  |  |  |  |

# Text changes

The text changes below are on top of JVET-O2001-vE [2]. Changes are highlight by grey. Deleted texts are marked with strikethrough.

#### 7.3.8.6 Palette coding syntax

| palette_coding( x0, y0, cbWidth, cbHeight, startComp, numComps ) { | Descriptor |
| --- | --- |
| … |  |
| if( MaxPaletteIndex > 0 ) { |  |
| num_palette_indices_diff_minus1 | ae(v) |
| adjust = 0 |  |
| for( i = 0; i <= num_palette_indices_minus1NumPaletteIndices; i++ ) { |  |
| if( MaxPaletteIndex − adjust > 0 ) { |  |
| palette_idx_idc | ae(v) |
| PaletteIndexIdc[ i ] = palette_idx_idc |  |
| } |  |
| adjust = 1 |  |
| } |  |
| copy_above_indices_for_final_run_flag | ae(v) |
| palette_transpose_flag | ae(v) |
| } |  |
| if( treeType != DUAL_TREE_CHROMA && palette_escape_val_present_flag ) { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta_abs | ae(v) |
| if( cu_qp_delta_abs ) |  |
| cu_qp_delta_sign_flag | ae(v) |
| } |  |
| } |  |
| if( treeType != DUAL_TREE_LUMA && palette_escape_val_present_flag ) { |  |
| if( cu_chroma_qp_offset_enabled_flag && !IsCuChromaQpOffsetCoded ) { |  |
| cu_chroma_qp_offset_flag | ae(v) |
| if( cu_chroma_qp_offset_flag ) |  |
| cu_chroma_qp_offset_idx | ae(v) |
| } |  |
| } |  |
| remainingNumIndices = num_palette_indices_minus1 + 1NumPaletteIndices) |  |
| … |  |
| if ( CopyAboveIndicesFlag[ xC ][ yC ] = = 0 ) { |  |
| currNumIndices = num_palette_indices_minus1 + 1 NumPaletteIndices − remainingNumIndices |  |
| PaletteIndexMap[ xC ][ yC ] = PaletteIndexIdc[ currNumIndices ] |  |
| } |  |
| … |  |
| } |  |

num_palette_indices_diff_minus1 plus 1 (MaxPaletteIndex + 1) is the number of palette indices explicitly signalled or inferred for the current block.

NumPaletteIndices is set to (num_palette_indices_diff + MaxPaletteIndex + 1 ).

When num_palette_indices_diff_minus1 is not present, it is inferred to be equal to 0.

The value of num_palette_indices_diff shall be in the range of 0 to cbWidth* cbHeight – (MaxPaletteIndex + 1 ) inclusive.

copy_above_indices_for_final_run_flag equal to 1 specifies that the palette indices of the last positions in the coding unit are copied from the palette indices in the row above if horizontal traverse scan is used or the palette indices in the left column if vertical traverse scan is used. copy_above_indices_for_final_run_flag equal to 0 specifies that the palette indices of the last positions in the coding unit are copied from PaletteIndexIdc[ num_palette_indices_minus1 NumPaletteIndices - 1].

#### 9.5.3.13 Binarization process for num_palette_indices_diff _minus1

Input to this process is a request for a binarization for the syntax element num_palette_indices_diff _minus1, and MaxPaletteIndex.

Output of this process is the binarization of the syntax element.

The variables cRiceParam is derived as follows:

cRiceParam = 3 + ( ( MaxPaletteIndex + 1 )  >>  3 )		(9-26)

The variable cMax is derived from cRiceParam as:

cMax = 4  <<  cRiceParam		(9-27)

The binarization of the syntax element num_palette_indices_diff _minus1 is a concatenation of a prefix bin string and (when present) a suffix bin string.

For the derivation of the prefix bin string, the following applies:

- The prefix value of num_palette_indices_diff _minus1, prefixVal, is derived as follows:

prefixVal = Min( cMax, num_palette_indices_diff_minus1)	(9-28)

- The prefix bin string is specified by invoking the TR binarization process as specified in clause 9.3.3.3 for prefixVal with the variables cMax and cRiceParam as inputs.

When the prefix bin string is equal to the bit string of length 4 with all bits equal to 1, the suffix bin string is present and it is derived as follows:

- The suffix value of num_palette_indices_diff_minus1, suffixVal, is derived as follows:

suffixVal = num_palette_indices_diff_minus1– cMax	(9-29)

- The suffix bin string is specified by invoking the k-th order EGk binarization process as specified in clause 9.3.3.5 for the binarization of suffixVal with the Exp-Golomb order k set equal to cRiceParam + 1.

| Table 9-77 – Syntax elements and associated binarizations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Syntax structure | Syntax element |  | Binarization |  |  |  |  |  |  |
|  |  |  | Process |  | Input parameters |  |  |  |  |
| palette_coding( ) | num_palette_indices_diff _minus1 |  | 9.5.3.13 |  | MaxPaletteIndex |  |  |  |  |
| Table 9-82 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |  |  |  |  |
| Syntax element |  | binIdx |  |  |  |  |  |  |  |  |
|  |  | 0 |  | 1 |  | 2 | 3 | 4 | >= 5 |  |
| num_palette_indices_diff _minus1 |  | bypass |  | bypass |  | bypass | bypass | bypass | bypass |  |

# Conclusion

It is proposed to remove the redundancy of signalling of number of palette indices and add a constraint to the signalled syntax value. We recommend adopting the proposed methods to the next version of VVC draft.

# References

- X. Xu, Y.-H Chao, Y.-C Sun and J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools,” JVET-O2028, Jul. 2019

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, Jul. 2019

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).