| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0700 |
| --- | --- |

| Title: | Crosscheck of JVET-P0586 Aspect #1 ([AHG18][Non-CE5] Proposed cleanup of deblocking filter process) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Zhipin Deng Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | +1(858)999-7093 zhipin.deng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results of aspect #1 of JVET-P0586, where a cleanup on the deblocking filter boundary strength derivation is proposed. The simulation results reportedly match those provided by the proponents.

# Verification

In the aspect #1 of JVET- P0586 proposal [1], it is proposed to cleanup the boundary strength derivation for CUs with TS (Transform Skip) of the deblocking filter. We verified that

- The source code implements match with what was described in the proposal.

- Simulation results match with those provided in the proposal.

CTC testing [2] is performed with the provided source code based on VTM-6.0 [3]. The coding results of aspect #1 compared to VTM-6.0 anchor are summarized in Table 1.

Table 1. Results of aspect #1 compared to VTM-6.0 anchor

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 102% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 109% |
| Class E | 0.00% | 0.00% | 0.00% | 102% | 103% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 107% |
| Class F | 0.00% | 0.00% | 0.00% | 103% | 106% |
|  |  |  | Random access |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.04% | 101% | 101% |
| Class A2 | 0.00% | 0.08% | 0.03% | 101% | 100% |
| Class B | -0.01% | 0.06% | 0.02% | 102% | 101% |
| Class C | 0.00% | 0.05% | 0.03% | 102% | 106% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.05% | 0.03% | 101% | 102% |
| Class D | -0.03% | 0.04% | 0.02% | 102% | 105% |
| Class F | -0.02% | -0.06% | -0.02% | 102% | 104% |
|  |  |  | Low delay B |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.01% | -0.16% | 99% | 97% |
| Class C | -0.01% | -0.17% | 0.05% | 101% | 98% |
| Class E | -0.10% | 0.05% | 0.38% | 103% | 102% |
| Overall | -0.02% | -0.05% | 0.05% | 101% | 99% |
| Class D | -0.08% | 0.20% | 0.41% | 103% | 104% |
| Class F | 0.03% | 0.30% | 0.56% | 103% | 102% |

The BD rate differences match with the results from the proposal [1]. The run times are basically in accordance with that from the proposal. Note that timing results are not well comparable due to differences in the simulation environment.

# References

L. Pham Van, G. Van der Auwera, V. Seregin, H. Huang, A. K. Ramasubramonian, M. Karczewicz (Qualcomm), “	[AHG18][Non-CE5] Proposed cleanup of deblocking filter process”, JVET-P0586, October 2019, Geneva, CH.

S. Iwamura, S. Nemoto, A. Ichigaya (NHK), K. Andersson, R. Yu, J. Enhorn (Ericsson), “Versatile Video Coding (Draft 6)”, JVET-O2001, July 2019, Gothenburg, SE.

VTM-6.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0.