| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0739-v1 |
| --- | --- |

| Title: | CE5-related: Combination of JVET-P0165, P0556, and P0557 on simplifying CCALF |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Olena Chubach, Chen-Yen Lai, Ching-Yeh Chen, Tzu-Der Chuang, Yu-Wen Huang, Shaw-Min Lei Nan Hu, Jie Dong, Vadim Seregin, Marta Karczewicz | Email: | {jenny.lai, olena.chubach, chingyeh.chen, peter.chuang, yuwen.huang, shawmin.lei} @mediatek.com nanh@qti.qualcomm.com |
| Source: | MediaTek Inc., Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In CE5-2, the filtered result of cross-component adaptive loop filter (CCALF) is added on top of the chroma adaptive loop filter (ALF) output. This contribution provided several combined tests of CCALF simplifications.

- JVET-P0165 Method 1 + Method 2a + JVET-P0556

- JVET-P0165 Method 1 + Method 2a + JVET-P0557 Method 2

- JVET-P0165 Method 1 + Method 2a + JVET-P0557 Method 3

Simulation results with chroma gains shifted to luma are summarized as follows.

The YCbCr BD-rate (Y-BD-rate*0.8 + Cb-BD-rate*0.1 + Cb-BD-rate*0.1, according to AHG13) of the CE5-2.1 CCALF is -0.85% for RA.

(comparing “VTM6.0 ALF + simplified CCALF” to “VTM6.0 ALF”).

Combined Test 1: Simplified CCALF preserves xx.x% of CCALF coding gain.

Combined Test 2: Simplified CCALF preserves xx.x% of CCALF coding gain.

Combined Test 3: Simplified CCALF preserves xx.x% of CCALF coding gain.

# Introduction

The concept of CCALF [1] is to use luma samples to improve chroma samples, and the corresponding filter coefficients are derived and signalled. The filter shape of CCALF is shown in Fig. 1, where one square is one luma sample, the red square is a luma position of to-be-processed chroma sample, each number represents one filter coefficient, and different squares with the same number share one filter coefficient. In total, there are 18 filter taps with 14 filter coefficients, so we need 14 multiplications per chroma sample in CCALF.

Figure 1.  The filter shape in CCALF.

In addition, CCALF coefficients used for each chroma component of a slice are stored in a buffer corresponding to a temporal sublayer for future reuse. When CCALF is enabled, a reuse flag is signalled at slice to indicate whether the coefficients from the corresponding temporal buffer are used or not for each chroma component. If the reuse flag is not enabled, CCALF coefficients from APS will be used.

# Proposed method

To reduce the number of multiplications per pixel when applying CCALF and avoid the data dependence of CCALF coefficients signalling, we proposed to combine JVET-P0165 [2], JVET-P0556 [3], and JVET-P0557 [4].

There are three combined tests proposed in this contribution.

- JVET-P0165 Method 1 + Method 2a + JVET-P0556

- JVET-P0165 Method 1 + Method 2a + JVET-P0556 + JVET-P0557 Method 2

- JVET-P0165 Method 1 + Method 2a + JVET-P0556 + JVET-P0557 Method 3

In JVET-P0165, it is proposed to avoid application of both CCALF and chroma ALF and to further align CCALF filter shape with ALF filter designs in which all coefficients of the filter can have different values shown in Figure 2.

Figure 2.  The filter shape and filter coefficients in CCALF proposed from JVET-P0165.

In JVET-P0556, the temporal buffers for CCALF coefficients signalling are removed. A slice can only use CCALF coefficients from APS.

In JVET-P0557, the restrictions for CCALF filter coefficients are proposed. There are four methods proposed in JVET-P0557, but only Method 2 and Method 3 are tested in this contribution since Method 4 caused larger performance drop reported in JVET-P0557. In the Method 2 of JVET-P0557, only the following values: {-128, -64, -32, -16, -8, -4, -2, -1, 0, 1, 2, 4, 8, 16, 32, 64, 128} can be used, and they are signalled as {-8, -7, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5, 6, 7, 8} respectively before binarization in order to reduce the signalling overhead. First order exponential-Golomb (EG1) codes are used to signal the absolute values of the coefficients. Sign bits are signalled for nonzero coefficients. In the Method 3 of JVET-P0557, only the following values: {-64, -32, -16, -8, -4, -2, -1, 0, 1, 2, 4, 8, 16, 32, 64} can be used, which are signalled as {-7, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5, 6, 7} respectively before binarization in order to reduce the signalling overhead. Fixed length codes of 3 bits are used to signal the absolute values of the coefficients. Sign bits are signalled for nonzero coefficients.

# Simulation results

The combined tests have been implemented on VTM6.0 [5]. The simulations were performed following the JVET common test conditions (CTC) [6]. Table 1 shows simulation results of CCALF in CE5-2. Table 2 shows simulation results of JVET-P0165 Method 1 + Method 2a + JVET-P0556. Table 3 shows simulation results of JVET-P0165 Method 1 + Method 2a + JVET-P0556 + JVET-P0557 Method 2. Table 4 shows simulation results of JVET-P0165 Method 1 + Method 2a + JVET-P0556 + JVET-P0557 Method 3. The lambda and chroma QP changes (shifting chroma coding gains to luma) in CE5-2 are applied in the test. The run time data are inaccurate.

Table 1.  Results of CE5-2 CCALF

|  |  |  | AI Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.19% | 5.37% | 3.75% | 102% | 102% |
| Class A2 | -2.04% | -0.29% | -0.74% | 100% | 101% |
| Class B | -0.74% | -1.74% | 0.92% | 101% | 101% |
| Class C | -0.76% | 3.98% | 4.51% | 99% | 101% |
| Class E | -0.46% | -2.81% | 4.71% | 101% | 99% |
| Overall | -0.99% | 0.78% | 2.54% | 101% | 101% |
| Class D | -0.75% | 5.36% | 4.85% | 100% | 102% |
| Class F | -0.73% | -1.03% | -1.29% | 100% | 98% |
|  |  |  | RA Main10 |  |  |
|  |  |  | Over VTM6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.82% | 5.25% | 4.13% | 99% | 101% |
| Class A2 | -2.05% | -1.14% | -6.18% | 97% | 102% |
| Class B | -0.54% | -6.34% | -2.25% | 100% | 99% |
| Class C | -0.78% | 5.75% | 6.57% | 101% | 97% |
| Class E |  |  |  |  |  |
| Overall | -1.16% | 0.24% | 0.59% | 99% | 100% |
| Class D | -0.64% | 6.08% | 6.42% | 98% | 97% |
| Class F | -0.92% | 1.86% | 2.34% | 100% | 97% |
|  |  |  | LB Main10 |  |  |
|  |  |  | Over VTM6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.54% | -9.26% | -6.60% | 102% | 101% |
| Class C | -0.73% | 3.86% | 5.44% | 100% | 99% |
| Class E | -0.24% | -5.62% | 7.60% | 102% | 98% |
| Overall | -0.53% | -3.98% | 0.96% | 101% | 100% |
| Class D | -0.56% | 4.90% | 7.37% | 105% | 104% |
| Class F | -0.84% | -2.82% | -0.28% | 104% | 101% |

Table 2.  Results of JVET-P0165 Method 1 + Method 2a + JVET-P0556 (results to be updated)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.26% | 6.46% | 5.52% | 117% | 105% |
| Class A2 | -2.16% | 1.69% | 0.21% | 123% | 110% |
| Class B | -0.80% | -0.55% | 2.06% | 125% | 105% |
| Class C | -0.81% | 5.52% | 6.24% | 121% | 94% |
| Class E | -0.49% | -1.37% | 5.52% | 119% | 94% |
| Overall | -1.06% | 2.20% | 3.83% | 122% | 101% |
| Class D | -0.83% | 5.82% | 6.13% | 124% | 80% |
| Class F | -0.86% | 0.54% | 0.03% | 122% | 97% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.15% | 6.76% | 5.64% | 35% | #NUM! |
| Class A2 | -1.24% | -11.91% | -43.30% | 11% | #NUM! |
| Class B | -0.57% | -5.46% | -1.51% | 111% | 109% |
| Class C | -0.83% | 6.90% | 7.79% | 114% | 106% |
| Class E |  |  |  |  |  |
| Overall | -0.89% | -1.01% | -5.96% | 55% | #NUM! |
| Class D | -0.72% | 6.51% | 7.36% | 112% | 104% |
| Class F | -1.02% | 2.53% | 3.04% | 112% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | 0% | 10% |
| Class C | -0.85% | 6.24% | 8.32% | 114% | 108% |
| Class E | -0.05% | -5.48% | 6.97% | 106% | 92% |
| Overall | #VALUE! | #VALUE! | #VALUE! | 6% | 38% |
| Class D | -0.63% | 7.38% | 9.69% | 114% | 109% |
| Class F | #VALUE! | #VALUE! | #VALUE! | 23% | 55% |

Table 3.  Results of JVET-P0165 Method 1 + Method 2a + JVET-P0556 + JVET-P0557 Method 2 (results to be updated)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.27% | 6.65% | 5.78% | 115% | 101% |
| Class A2 | -2.03% | 0.39% | -16.57% | 43% | 72% |
| Class B | -0.81% | -0.23% | 2.42% | 120% | 102% |
| Class C | -0.83% | 5.57% | 6.30% | 122% | 95% |
| Class E | -0.50% | -0.99% | 5.66% | 120% | 95% |
| Overall | -1.04% | 2.18% | 1.22% | 101% | 94% |
| Class D | -0.84% | 5.78% | 6.02% | 124% | 81% |
| Class F | -0.86% | 0.70% | 0.15% | 122% | 97% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.94% | 5.28% | 6.07% | 3% | 28% |
| Class A2 | -0.53% | -9.99% | -42.60% | 3% | 27% |
| Class B | -0.54% | -5.09% | -1.48% | 111% | 93% |
| Class C | -0.81% | 6.75% | 7.56% | 115% | 109% |
| Class E |  |  |  |  |  |
| Overall | -0.69% | -0.84% | -5.78% | 28% | 60% |
| Class D | -0.69% | 5.87% | 6.94% | 113% | 107% |
| Class F | -1.03% | 2.44% | 2.93% | 113% | 103% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | 0% | 6% |
| Class C | -0.77% | 6.31% | 8.25% | 111% | 105% |
| Class E | -0.20% | -5.67% | 6.89% | 104% | 89% |
| Overall | #VALUE! | #VALUE! | #VALUE! | 4% | 31% |
| Class D | -0.57% | 6.86% | 8.84% | 114% | 107% |
| Class F | #VALUE! | #VALUE! | #VALUE! | 11% | 43% |

Table 4.  Results of JVET-P0165 Method 1 + Method 2a + JVET-P0556 + JVET-P0557 Method 3 (results to be updated)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.27% | 6.63% | 5.78% | 116% | 103% |
| Class A2 | -2.16% | 2.17% | 0.56% | 118% | 102% |
| Class B | -0.81% | -0.24% | 2.41% | 120% | 101% |
| Class C | -0.82% | 5.57% | 6.30% | 122% | 94% |
| Class E | -0.51% | -1.00% | 5.63% | 120% | 94% |
| Overall | -1.06% | 2.47% | 4.07% | 120% | 99% |
| Class D | -0.84% | 5.81% | 6.06% | 126% | 81% |
| Class F | -0.87% | 0.70% | 0.15% | 121% | 95% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.13% | 6.82% | 5.83% | 35% | #NUM! |
| Class A2 | -1.23% | -9.86% | -42.74% | 10% | #NUM! |
| Class B | -0.54% | -5.16% | -1.52% | 111% | #NUM! |
| Class C | -0.80% | 6.87% | 7.58% | 115% | 109% |
| Class E |  |  |  |  |  |
| Overall | -0.86% | -0.49% | -5.87% | 55% | #NUM! |
| Class D | -0.70% | 6.13% | 7.17% | 113% | 107% |
| Class F | -1.02% | 2.45% | 2.97% | 113% | 102% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | 0% | 8% |
| Class C | -0.78% | 6.39% | 8.41% | 113% | 106% |
| Class E | -0.20% | -6.36% | 6.78% | 104% | 91% |
| Overall | #VALUE! | #VALUE! | #VALUE! | 5% | 34% |
| Class D | -0.58% | 6.98% | 9.41% | 117% | 111% |
| Class F | #VALUE! | #VALUE! | #VALUE! | 23% | 55% |

# Conclusion

In this contribution, some combined tests are proposed. To simplify CCALF, it is proposed to reduce the number of multiplications per pixel, remove of temporal layer of CCALF filter coefficient, and further restrict the filter coefficients to some pre-defined values.

# References

- K. Misra, F. Bossen, and A. Segall, “CE5-2.1, CE5-2.2: Cross component adaptive loop filter,” Document of Joint Video Experts Team, JVET-P0080

- O. Chubach, C.-Y. Lai, C.-Y. Chen, T.-D. Chuang, Y.-W. Huang, and S.-M. Lei, “CE5-related: Simplified CCALF,” Document of Joint Video Experts Team, JVET-P0165

- N. Hu, J. Dong, V. Seregin, and M. Karczewicz, “CE5-related: Temporal buffer removal for cross component adaptive loop filter,” Document of Joint Video Experts Team, JVET-P0556

- N. Hu, J. Dong, V. Seregin, and M. Karczewicz, “CE5-related: Multiplication removal for cross component adaptive loop filter,” Document of Joint Video Experts Team, JVET-P0557

- VTM6.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).