| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0799-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-P0479: CE8-related: Restriction on context coded bins in palette |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Weijia Zhu 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-360-525-0535 weijia.zhu@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This document reports crosscheck results of JVET-P0479 (CE8-related: Restriction on context coded bins in palette). The results generated by us match those provided by the proponent.

# Introduction

In JVET-P0479, it is proposed to restrict the context coded bin number in palette.

# Simulation Results

Simulations were performed by using the code provided by the proponent. Simulation results are shown below. Please note that the complexity may be not reliable due to the heterogeneous clusters.

Table 1. The performance of JVET-P0479 under AI with dual tree on

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | 0.11% | 0.17% | 0.15% | 102% | 102% |
| TGM 720p | 0.05% | 0.09% | 0.14% | 101% | 101% |
| Animation | -0.01% | 0.01% | 0.01% | 101% | 101% |
| Mixed content | 0.04% | 0.08% | 0.10% | 102% | 102% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 102% | 102% |
| Overall | 0.05% | 0.08% | 0.09% | 101% | 102% |

Table 2. The performance of JVET-P0479 under RA with dual tree on

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | 0.03% | 0.05% | 0.04% | 102% | 103% |
| TGM 720p | 0.13% | 0.10% | 0.06% | 102% | 103% |
| Animation | 0.00% | 0.00% | -0.01% | 101% | 101% |
| Mixed content | 0.07% | 0.21% | 0.12% | 101% | 102% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 102% | 100% |
| Overall | 0.05% | 0.08% | 0.05% | 102% | 102% |

Table 3. The performance of JVET-P0479 under LDB with dual tree on

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | 0.08% | 0.01% | 0.05% | 102% | 101% |
| TGM 720p | 0.14% | -0.14% | 0.46% | 100% | 100% |
| Animation | 0.03% | 0.09% | 0.05% | 103% | 101% |
| Mixed content | -0.29% | -0.19% | -0.09% | 102% | 102% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 102% | 101% |
| Overall | 0.01% | -0.05% | 0.12% | 101% | 101% |

Table 4. The performance of JVET-P0479 under AI with dual tree off

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | 0.07% | 0.05% | 0.07% | 101% | 101% |
| TGM 720p | 0.03% | 0.10% | 0.05% | 101% | 99% |
| Animation | 0.02% | 0.06% | 0.06% | 101% | 100% |
| Mixed content | 0.04% | 0.03% | 0.05% | 103% | 102% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Overall | 0.04% | 0.06% | 0.05% | 101% | 101% |

Table 5. The performance of JVET-P0479 under RA with dual tree off

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | 0.11% | 0.05% | 0.17% | 102% | 102% |
| TGM 720p | 0.01% | 0.05% | 0.06% | 102% | 103% |
| Animation | 0.03% | -0.01% | 0.01% | 101% | 100% |
| Mixed content | -0.09% | 0.12% | 0.07% | 102% | 101% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Overall | 0.02% | 0.05% | 0.07% | 101% | 102% |

Table 6. The performance of JVET-P0479 under LDB with dual tree off

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.02% | 0.02% | 0.01% | 99% | 101% |
| TGM 720p | 0.09% | -0.03% | 0.64% | 102% | 102% |
| Animation | -0.02% | 0.02% | 0.00% | 100% | 101% |
| Mixed content | 0.16% | -0.07% | 0.01% | 101% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Overall | 0.04% | -0.01% | 0.16% | 101% | 101% |