| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0802-v1 |
| --- | --- |

| Title: | Non-CE3: PDPC spec and software mismatch |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hua Yang, Yuwen He, Hongyu Li 4 Research Way 3rd Floor, Princeton, NJ 08540, USA | Tel: Email: | +1-609-608-0479 Hua.Yang@InterDigital.com Yuwen.He@InterDigital.com Hongyu.Li@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

A spec and software mismatch is found on PDPC in VTM-6.0. In the spec, PDPC is applied to horizontal and vertical Intra prediction modes, while in the VTM-6.0 software, PDPC is not applied for these two Intra modes. We tested aligning the software to the spec, and the CTC result shows: +0.01%/+0.01%/-0.04% Y BD-rate change for AI/RA/LDB. A decision needs to be made on whether we should align the software to the spec or vice versa.

# Introduction

In the spec (Clause 8.4.5.2.14 of VTM-6.0 specification [1]) , for the horizontal and vertical Intra prediction modes (i.e. INTRA_ANGULAR18 and INTRA_ANGULAR50, respectively), PDPC (Position Dependent intra Prediction Combination) would be applied.

However, in the VTM-6.0 software [2], PDPC is not applied to these two Intra mode as specified in the spec, as can be seen in IntraPrediction::initPredIntraParams(.) in IntraPrediction.cpp.

# Simulation results

We tested with aligning the software to the spec, i.e. to apply PDPC to the horizontal and vertical Intra prediction modes, and the CTC result is as follows. Spec change to align with the software is also provided with the proposal.

Table 1. Performance of aligning the software to the spec

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.04% | 103% | 106% |
| Class A2 | 0.01% | -0.02% | -0.01% | 104% | 109% |
| Class B | 0.02% | 0.00% | -0.02% | 101% | 102% |
| Class C | 0.02% | -0.13% | 0.10% | 100% | 101% |
| Class E | -0.03% | -0.04% | -0.05% | 100% | 98% |
| Overall | 0.01% | -0.04% | 0.01% | 102% | 103% |
| Class D | -0.01% | 0.01% | 0.07% | 101% | 101% |
| Class F | 0.08% | -0.05% | -0.09% | 100% | 102% |
| Class TGM | 0.20% | 0.13% | 0.11% | 102% | 104% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.14% | 0.05% | 105% | 111% |
| Class A2 | 0.03% | 0.13% | 0.04% | 106% | 111% |
| Class B | 0.00% | 0.01% | -0.01% | 101% | 102% |
| Class C | 0.02% | -0.04% | -0.04% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.01% | 0.01% | 102% | 105% |
| Class D | -0.05% | -0.34% | 0.04% | 100% | 101% |
| Class F | 0.06% | 0.04% | 0.01% | 100% | 100% |
| Class TGM | 0.14% | 0.04% | 0.02% | 101% | 102% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.06% | -0.13% | 110% | 123% |
| Class C | -0.05% | 0.10% | -0.05% | 102% | 113% |
| Class E | -0.07% | -0.02% | -0.07% | 101% | 105% |
| Overall | -0.04% | 0.05% | -0.09% | 105% | 115% |
| Class D | 0.05% | -0.02% | -0.22% | 100% | 100% |
| Class F | 0.09% | 0.40% | 0.58% | 103% | 112% |
| Class TGM | -0.06% | 0.01% | 0.06% | 106% | 118% |

# Conclusion

A spec and software mismatch is found whether to apply PDPC on the horizontal and vertical Intra prediction modes. Simulation result aligning the software to the spec with applying PDPC on these two modes yields: +0.01%/+0.01%/-0.04% Y BD-rate change for AI/RA/LDB. A decision needs to be made on whether we should align the software to the spec or vice versa.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, July 2019, Gothenburg, Sweden.

- VTM-6.0 software https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).