| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0835-v1 |
| --- | --- |

| Title: | Cross-check result of JVET-P0625: Non-CE3: simplified MIP with power-of-two offset |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Z. Zhang | Tel: Email: | +46761499428 zhi.a.zhang@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This document reports the results of crosschecking JVET-P0625 proposed by Qualcomm. JVET-P0625 proposes to remove the MIP offset parameter table by using a MipSizeId dependent variable which is power of two. With the proposed method, the offset value is derived by left shift instead of multiplication of offset parameter from look-up table.

The update of the code had been reviewed and confirmed consistent with the proposal description in JVET-P0625.

The simulation has been conducted under CTC test condition and low-QP condition on top of VTM6.0. The simulation results match the results provided in JVET-P0625.

# Verification

Qualcomm had provided the source code and results of their proposed methods in JVET-P0625[1]. The simulation has been conducted under CTC test condition and low-QP condition on top of VTM6.0. The verification simulation results match the results in JVET-P0625.

Simulation results under CTC test condition:

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,05% | 0,04% | 0,09% | 101% | 100% |
| Class A2 | 0,02% | 0,04% | -0,03% | 100% | 100% |
| Class B | 0,02% | 0,03% | -0,02% | 102% | 99% |
| Class C | 0,02% | -0,16% | 0,00% | 103% | 102% |
| Class E | 0,05% | -0,04% | 0,06% | 102% | 103% |
| Overall | 0,03% | -0,02% | 0,01% | 102% | 101% |
| Class D | 0,00% | 0,08% | -0,06% | 101% | 101% |
| Class F | 0,01% | -0,03% | 0,04% | 103% | 102% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,03% | 0,04% | 0,05% | 102% | 101% |
| Class A2 | 0,02% | 0,00% | 0,07% | 100% | 102% |
| Class B | 0,00% | 0,09% | 0,09% | 102% | 99% |
| Class C | 0,02% | 0,04% | 0,10% | 102% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0,02% | 0,05% | 0,08% | 101% | 100% |
| Class D | -0,06% | -0,23% | -0,13% | 101% | 99% |
| Class F | 0,02% | -0,03% | 0,07% | 103% | 99% |

Simulation results under low-QP test condition:

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 (host timing without MD5) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,05% | 0,06% | 0,07% | 102% | 98% |
| Class A2 | 0,01% | 0,00% | 0,00% | 102% | 99% |
| Class B | 0,01% | -0,01% | -0,01% | 100% | 99% |
| Class C | 0,00% | 0,01% | 0,04% | 100% | 99% |
| Class E | 0,01% | 0,02% | 0,05% | 100% | 101% |
| Overall | 0,01% | 0,01% | 0,03% | 101% | 99% |
| Class D | 0,00% | -0,01% | 0,03% | 99% | 101% |
| Class F | 0,00% | 0,01% | 0,02% | 100% | 100% |

# Reference

[1]. JVET-P0625: Non-CE3: simplified MIP with power-of-two offset

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).