| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0836-v2 |
| --- | --- |

| Title: | Cross-check of JVET-P0353 on reference picture resampling filters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Vadim Seregin | Tel: Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents cross-check results for JVET-P0353, where downsampling filters are switched based on the scaling ratio for reference picture resampling (RPR). This cross-check report confirms the test results and specification changes provided in JVET-P0353.

# Introduction

In JVET-P0353, currently used motion interpolation filters are switched based on the scaling ratio for reference picture downsampling. It also introduces a 6-tap donwsampling filters for luma to be used for affine prediction when downsmapling of a reference picture is applied.

Filter sets are switched separately for horizontal and vertical directions based on the scaling ratio in the corresponding direction comparing to a threshold value. Threshold values are 20480 (1.25x) and 28672 (1.75x).

In total, two 8-tap luma filter sets and two 4-tap chroma filter sets are introduced for non-affine mode, and two 6-tap luma filter sets are introduced for affine mode.

Additionally, interpolation filtering is enabled for zero phase (xFrac = 0 or yFrac = 0) for scaling ratio >= 1.25x instead of performing samples copy.

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.10% | -0.36% | -0.14% | 98% | 98% |
| Class C | -3.59% | -1.88% | -1.81% | 98% | 92% |
| Class E | -2.99% | -0.18% | 0.17% | 98% | 99% |
| Overall | -2.40% | -0.82% | -0.62% | 98% | 96% |
| Class D | -3.92% | -1.50% | -1.58% | 99% | 98% |
| Class F | -13.43% | -10.44% | -10.89% | 94% | 95% |

1.5x PSNR1

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.77% | -0.52% | -0.27% | 98% | 98% |
| Class C | -2.59% | -1.92% | -1.90% | 98% | 92% |
| Class E | -2.21% | -1.84% | -1.10% | 98% | 99% |
| Overall | -1.74% | -1.32% | -1.02% | 98% | 96% |
| Class D | -2.55% | -1.43% | -1.85% | 99% | 98% |
| Class F | -9.34% | -9.85% | -9.84% | 94% | 95% |

1.5x PSNR2

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -3.16% | -1.73% | -1.67% | 98% | 100% |
| Class C | -6.65% | -4.37% | -4.10% | 98% | 94% |
| Class E | -9.71% | -4.54% | -3.71% | 99% | 100% |
| Overall | -5.96% | -3.31% | -2.99% | 98% | 98% |
| Class D | -6.11% | -3.41% | -3.42% | 99% | 96% |
| Class F | -17.43% | -14.64% | -15.66% | 94% | 94% |

2.0x PSNR1

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -2.16% | -1.24% | -1.22% | 98% | 100% |
| Class C | -4.07% | -2.53% | -2.46% | 98% | 94% |
| Class E | -6.19% | -3.80% | -3.45% | 99% | 100% |
| Overall | -3.80% | -2.31% | -2.19% | 98% | 98% |
| Class D | -3.30% | -2.60% | -2.60% | 99% | 96% |
| Class F | -10.37% | -10.21% | -9.98% | 94% | 94% |

2.0x PSNR2

# Additional tests

In the additional tests when RPR is applied, affine 6-tap luma filtering is using existed motion interpolation filters (Test 2), and not using alternative half pel filter on top of the test 2 (Test 3).

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.01% | -0.48% | -0.51% | 101% | 100% |
| Class C | -3.48% | -1.95% | -1.97% | 100% | 97% |
| Class E | -2.55% | -0.59% | 0.60% | 97% | 98% |
| Overall | -2.22% | -1.00% | -0.72% | 100% | 98% |
| Class D | -3.72% | -1.35% | -1.38% | 98% | 95% |
| Class F | -13.08% | -10.34% | -10.92% | 94% | 97% |

Test 2. 1.5x PSNR1

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.66% | -0.73% | -0.84% | 101% | 100% |
| Class C | -2.50% | -1.95% | -2.06% | 100% | 97% |
| Class E | -1.80% | -2.87% | -0.88% | 97% | 98% |
| Overall | -1.56% | -1.67% | -1.26% | 100% | 98% |
| Class D | -2.40% | -1.18% | -1.63% | 98% | 95% |
| Class F | -9.05% | -9.68% | -10.16% | 94% | 97% |

Test 2. 1.5x PSNR2

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -3.12% | -1.63% | -1.88% | 100% | 101% |
| Class C | -6.56% | -4.21% | -3.83% | 100% | 101% |
| Class E | -9.67% | -4.35% | -3.81% | 97% | 100% |
| Overall | -5.91% | -3.17% | -3.01% | 100% | 101% |
| Class D | -6.14% | -3.44% | -3.36% | 98% | 94% |
| Class F | -17.30% | -14.63% | -15.87% | 94% | 95% |

Test 2. 2.0x PSNR1

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -2.12% | -1.15% | -1.45% | 100% | 101% |
| Class C | -4.00% | -2.44% | -2.43% | 100% | 101% |
| Class E | -6.26% | -3.46% | -3.51% | 97% | 100% |
| Overall | -3.78% | -2.16% | -2.29% | 100% | 101% |
| Class D | -3.31% | -2.65% | -2.60% | 98% | 94% |
| Class F | -10.25% | -10.30% | -10.38% | 94% | 95% |

Test 2. 2.0x PSNR2

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.01% | -0.56% | -0.41% | 101% | 101% |
| Class C | -3.48% | -2.05% | -2.16% | 100% | 101% |
| Class E | -2.70% | -0.74% | 0.22% | 97% | 97% |
| Overall | -2.26% | -1.10% | -0.84% | 99% | 100% |
| Class D | -3.76% | -1.38% | -1.28% | 97% | 96% |
| Class F | -13.13% | -10.48% | -10.89% | 94% | 99% |

Test 3. 1.5x PSNR1

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.67% | -0.81% | -0.71% | 101% | 101% |
| Class C | -2.50% | -2.05% | -2.22% | 100% | 101% |
| Class E | -1.91% | -3.04% | -1.25% | 97% | 97% |
| Overall | -1.59% | -1.78% | -1.35% | 99% | 100% |
| Class D | -2.42% | -1.24% | -1.44% | 97% | 96% |
| Class F | -9.14% | -10.02% | -10.22% | 94% | 99% |

Test 3. 1.5x PSNR2

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -3.10% | -1.68% | -1.69% | 100% | 99% |
| Class C | -6.61% | -4.30% | -3.88% | 99% | 101% |
| Class E | -9.64% | -4.19% | -3.82% | 98% | 99% |
| Overall | -5.90% | -3.18% | -2.95% | 99% | 100% |
| Class D | -6.11% | -3.34% | -3.39% | 98% | 92% |
| Class F | -17.30% | -14.61% | -15.82% | 94% | 95% |

Test 3. 2.0x PSNR1

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -2.09% | -1.19% | -1.21% | 100% | 99% |
| Class C | -4.01% | -2.56% | -2.36% | 99% | 101% |
| Class E | -6.20% | -3.32% | -3.51% | 98% | 99% |
| Overall | -3.76% | -2.18% | -2.17% | 99% | 100% |
| Class D | -3.30% | -2.52% | -2.63% | 98% | 92% |
| Class F | -10.25% | -10.21% | -10.31% | 94% | 95% |

Test 3. 2.0x PSNR2