| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0846-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-P0105 (Non-CE5: Modified Chroma QP derivation for deblocking filter) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chia-Ming Tsai No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 chia-ming.tsai@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

The document reports the results of crosschecking JVET-P0105 proposed by Huawei. JVET-P0105 proposes a modified method of deriving the chroma QP for chroma deblocking. However, only partial results are verified in this document, and the BD-rate results match exactly with those in JVET-P0105.

Verification

Huawei provided the source code of their proposed method described in JVET-P0105 [1]. One verification was carried out by setting the following parameters in the provided code:

Test:

#define CHROMAQP_JCCR_AVMBKJ                           1

The test was conducted under the common test conditions (CTC) defined in [2] using the VTM6.0 software [3]. Table 1 shows the results of using VTM6.0 as the anchor and enabling the proposed methods in the test. The BD-rates of the test from our experimental results match exactly those in JVET-P0105. Please note that the runtime comparison may be not accurate.

Table 1. The results of test in JVET-P0105 using VTM6.0 as anchor

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | 0.06% | 99% | 99% |
| Class A2 | 0.01% | 0.11% | 0.02% | 99% | 99% |
| Class B | -0.01% | 0.09% | 0.06% | 99% | 104% |
| Class C | -0.01% | -0.03% | 0.01% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.04% | 0.04% | 99% | 101% |
| Class D | 0.00% | -0.01% | -0.12% | 100% | 100% |
| Class F | 0.01% | 0.01% | -0.02% | 99% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.02% | -0.17% | -0.09% | 100% | 100% |
| Class E | 0.00% | 0.26% | 0.22% | 99% | 98% |
| Overall |  |  |  |  |  |
| Class D | -0.01% | 0.08% | 0.31% | 100% | 97% |
| Class F |  |  |  |  |  |

References

[1] A. M. Kotra, S. Esenlik, B. Wang, H. Gao, and E. Alshina, “Non-CE5: Modified Chroma QP derivation for deblocking filter,” Document of Joint Video Experts Team, JVET-P0105.

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

[3] VTM6.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).