| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0847-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-P0106 (AHG16/CE5-related: Simplifications for cross component adaptive loop filter) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chia-Ming Tsai No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 chia-ming.tsai@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

The document reports the results of crosschecking JVET-P0106 proposed by Huawei. JVET-P0106 proposes complexity reduction of the cross-component adaptive loop filter (CCALF) by using a filter shape with 8 filter coefficients. However, only partial results are verified in this document, and the BD-rate results match exactly with those in JVET-P0106.

Verification

Huawei provided the source code of their proposed method described in JVET-P0106 [1]. One verification was carried out by setting the following parameters in the provided code:

Test:

#define AVMBKJ_REDUCE_FILTER                           1

The test was conducted under the common test conditions (CTC) defined in [2] using the VTM6.0 software [3]. Table 1 shows the results of using VTM6.0 as the anchor and enabling the proposed methods in the test. The BD-rates of the test from our experimental results match exactly those in JVET-P0106. Please note that the runtime comparison may be not accurate.

Table 1. The results of test in JVET-P0106 using VTM6.0 as anchor

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.20% | 5.86% | 4.34% | 101% | 102% |
| Class A2 | -2.03% | 0.69% | -0.35% | 99% | 102% |
| Class B | -0.76% | -1.10% | 1.52% | 100% | 101% |
| Class C | -0.79% | 4.73% | 5.29% | 100% | 100% |
| Class E | -0.50% | -2.10% | 4.94% | 100% | 101% |
| Overall | -1.01% | 1.49% | 3.09% | 100% | 101% |
| Class D | -0.77% | 5.38% | 5.34% | 99% | 99% |
| Class F | -0.75% | -0.41% | -0.95% | 100% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.82% | 5.73% | 4.60% | 100% | 102% |
| Class A2 | -2.02% | 0.32% | -5.35% | 99% | 104% |
| Class B | -0.55% | -6.05% | -1.56% | 100% | 102% |
| Class C | -0.79% | 6.31% | 7.09% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -1.16% | 0.88% | 1.22% | 100% | 102% |
| Class D | -0.64% | 5.45% | 6.60% | 101% | 100% |
| Class F | -0.92% | 2.21% | 2.75% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.71% | 5.53% | 7.22% | 100% | 96% |
| Class E | -0.21% | -5.65% | 7.69% | 101% | 98% |
| Overall |  |  |  |  |  |
| Class D | -0.60% | 5.64% | 8.27% | 101% | 97% |
| Class F | -0.90% | -1.81% | 0.46% | 100% | 98% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.52% | -6.78% | -4.55% | 100% | 98% |
| Class C | -0.69% | 6.07% | 7.78% | 100% | 97% |
| Class E | -0.04% | -6.28% | 8.32% | 102% | 97% |
| Overall | -0.46% | -2.37% | 2.78% | 101% | 98% |
| Class D | -0.48% | 6.12% | 7.99% | 101% | 97% |
| Class F | -0.90% | -1.44% | 0.07% | 101% | 98% |

References

[1] A. M. Kotra, S. Esenlik, B. Wang, H. Gao, and E. Alshina, “AHG16/CE5-Related: Simplifications for Cross Component Adaptive Loop Filter,” Document of Joint Video Experts Team, JVET-P0106.

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

[3] VTM6.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).