| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0849-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-P0251 (CE5-related: Simplified CCALF with 6 filter coefficients) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chia-Ming Tsai No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 chia-ming.tsai@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

The document reports the results of crosschecking JVET-P0251 proposed by Huawei. JVET-P0251 proposes to simplify the cross-component adaptive loop filter (CCALF) from 14 filter coefficients to 6 filter coefficients. However, only partial results are verified in this document, and the BD-rate results match exactly with those in JVET-P0251.

Verification

Huawei provided the source code of their proposed method described in JVET-P0251 [1]. Two verifications were carried out by setting the following parameters in the provided code:

Test 1:

//Start of JVET-P0251
#define SIMP_CCALF                      6 //number tap
#if SIMP_CCALF == 6
#define SIMP_C6_STEP1                  0 //0: test C6-1; 1: test C6-2
#endif
#define CCALF_MULTI_TYPE              0 //1: test C6-3, which selects C6-1 and C6-2 based on APS flag
//end

Test 2:

//Start of JVET-P0251
#define SIMP_CCALF                      6 //number tap
#if SIMP_CCALF == 6
#define SIMP_C6_STEP1                  1 //0: test C6-1; 1: test C6-2
#endif
#define CCALF_MULTI_TYPE              0 //1: test C6-3, which selects C6-1 and C6-2 based on APS flag
//end

The tests were conducted under the common test conditions (CTC) defined in [2] using the VTM6.0 software [3]. Table 1 and Table 2 show the results of using VTM6.0 as the anchor and enabling the proposed methods in the Test 1 and Test 2, respectively. The BD-rates of the test from our experimental results match exactly those in JVET-P0251. Please note that the runtime comparison may be not accurate.

Table 1. The results of Test 1 in JVET-P0251 using VTM6.0 as anchor

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | -3.22% | -3.70% | 104% | 104% |
| Class A2 | 0.14% | -8.00% | -8.37% | 106% | 109% |
| Class B | 0.09% | -9.82% | -7.51% | 103% | 104% |
| Class C | 0.08% | -3.10% | -3.07% | 102% | 103% |
| Class E | 0.13% | -9.10% | -3.36% | 98% | 102% |
| Overall | 0.11% | -6.80% | -5.34% | 102% | 104% |
| Class D | 0.10% | -2.32% | -2.93% | 101% | 103% |
| Class F | 0.08% | -2.62% | -2.19% | 96% | 98% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.15% | -5.21% | -4.82% | 108% | 104% |
| Class A2 | 0.25% | -9.72% | -13.60% | 105% | 102% |
| Class B | 0.20% | -15.69% | -12.23% | 101% | 99% |
| Class C | 0.10% | -4.00% | -3.53% | 99% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.17% | -9.29% | -8.70% | 103% | 102% |
| Class D | 0.09% | -4.09% | -4.46% | 99% | 103% |
| Class F | 0.10% | -2.31% | -1.98% | 99% | 104% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.14% | -6.94% | -5.48% | 103% | 102% |
| Class E | 0.15% | -16.43% | -6.24% | 101% | 99% |
| Overall |  |  |  |  |  |
| Class D | 0.18% | -7.67% | -5.90% | 101% | 102% |
| Class F | 0.16% | -5.04% | -3.77% | 100% | 105% |

Table 2. The results of Test 2 in JVET-P0251 using VTM6.0 as anchor

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | -3.08% | -3.32% | 104% | 103% |
| Class A2 | 0.13% | -7.54% | -8.31% | 103% | 104% |
| Class B | 0.08% | -10.00% | -7.45% | 103% | 104% |
| Class C | 0.08% | -3.11% | -3.08% | 100% | 100% |
| Class E | 0.13% | -9.07% | -2.87% | 100% | 104% |
| Overall | 0.10% | -6.75% | -5.17% | 102% | 103% |
| Class D | 0.07% | -2.34% | -2.61% | 98% | 101% |
| Class F | 0.14% | -4.89% | -5.20% | 98% | 103% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | -4.81% | -4.46% | 104% | 98% |
| Class A2 | 0.24% | -9.73% | -14.52% | 103% | 109% |
| Class B | 0.21% | -16.51% | -12.87% | 102% | 104% |
| Class C | 0.07% | -3.95% | -3.40% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.16% | -9.46% | -8.99% | 102% | 103% |
| Class D | 0.11% | -4.15% | -3.79% | 100% | 101% |
| Class F | 0.16% | -3.54% | -3.52% | 100% | 104% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.13% | -5.43% | -4.34% | 103% | 103% |
| Class E | 0.10% | -15.35% | -5.04% | 104% | 102% |
| Overall |  |  |  |  |  |
| Class D | 0.13% | -6.66% | -4.27% | 100% | 100% |
| Class F | 0.19% | -6.36% | -5.17% | 99% | 101% |

References

[1] Y. Zhao and H. Yang, “CE5-related: Simplified CCALF with 6 filter coefficients,” Document of Joint Video Experts Team, JVET-P0251.

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

[3] VTM6.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).