| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0852 |
| --- | --- |

| Title: | Crosscheck of JVET-P0173 (AHG16/Non-CE5: Cross component ALF simplification) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Kui Fan 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1 858 265 9942 kui.fan@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This document reports crosscheck results of JVET-P0173 (AHG16/Non-CE5: Cross component ALF simplification). The results generated by us match those provided by the proponent.

# Introduction

In JVET-P0173, it is proposed to simplify cross-component ALF (CCALF):

  - Reduce number of signaled CCALF coefficients. Number of coefficients equals to 6, 7, 8 are tested separately.

  - Add a clipping process to refinement value of CCALF to reduce on-chip memory and also allows more flexibility to control subjective quality.

# Simulation Results

Simulations were performed by using the code provided by the proponent. Simulation results are shown below. Please note that the complexity may be not reliable due to the heterogeneous clusters.

Table 1. The performance of JVET-P0173 (6 coeff w shift)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.22% | 6.10% | 5.31% | 102% | 101% |
| Class A2 | -2.03% | 1.48% | 0.26% | 99% | 102% |
| Class B | -0.77% | -0.56% | 2.73% | 99% | 102% |
| Class C | -0.81% | 5.03% | 5.80% | 99% | 103% |
| Class E | -0.52% | -1.81% | 5.49% | 100% | 104% |
| Overall | -1.02% | 1.92% | 3.89% | 100% | 103% |
| Class D | -0.81% | 5.56% | 6.29% | 98% | 102% |
| Class F | -0.77% | -0.16% | -0.85% | 101% | 104% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.87% | 6.89% | 6.19% | 100% | 102% |
| Class A2 | -2.04% | 1.74% | -4.37% | 99% | 101% |
| Class B | -0.55% | -4.40% | 0.83% | 101% | 102% |
| Class C | -0.80% | 7.25% | 8.07% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -1.18% | 2.19% | 2.79% | 100% | 101% |
| Class D | -0.64% | 6.21% | 7.39% | 99% | 101% |
| Class F | -0.98% | 2.69% | 2.97% | 100% | 103% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.55% | -4.39% | -1.22% | 100% | 102% |
| Class C | -0.81% | 9.47% | 9.92% | 100% | 99% |
| Class E | -0.22% | -3.03% | 10.08% | 101% | 103% |
| Overall | -0.55% | 0.57% | 5.31% | 100% | 101% |
| Class D | -0.59% | 8.04% | 9.66% | 102% | 103% |
| Class F | -0.97% | 0.77% | 2.43% | 103% | 99% |

Table 2. The performance of JVET-P0173 (7 coeff w shift)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.21% | 5.96% | 4.82% | 103% | 102% |
| Class A2 | -2.03% | 1.11% | -0.02% | 100% | 103% |
| Class B | -0.77% | -0.76% | 2.12% | 101% | 103% |
| Class C | -0.81% | 4.95% | 5.73% | 99% | 104% |
| Class E | -0.51% | -2.31% | 5.04% | 102% | 105% |
| Overall | -1.02% | 1.68% | 3.51% | 101% | 104% |
| Class D | -0.80% | 5.60% | 6.29% | 99% | 108% |
| Class F | -0.76% | -0.37% | -0.98% | 102% | 105% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.85% | 6.21% | 5.23% | 100% | 102% |
| Class A2 | -2.02% | 0.79% | -4.95% | 100% | 102% |
| Class B | -0.53% | -5.29% | -0.97% | 101% | 102% |
| Class C | -0.82% | 6.68% | 7.70% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | -1.17% | 1.42% | 1.79% | 101% | 102% |
| Class D | -0.67% | 5.98% | 7.60% | 100% | 98% |
| Class F | -0.95% | 2.54% | 2.85% | 102% | 104% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.53% | -6.10% | -4.14% | 99% | 103% |
| Class C | -0.74% | 7.53% | 8.80% | 99% | 99% |
| Class E | -0.12% | -5.17% | 9.28% | 102% | 104% |
| Overall | -0.50% | -1.32% | 3.53% | 100% | 102% |
| Class D | -0.51% | 6.87% | 9.30% | 101% | 103% |
| Class F | -0.90% | 0.07% | 1.95% | 102% | 101% |

Table 3. The performance of JVET-P0173 (8 coeff w shift)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.21% | 5.82% | 4.54% | 101% | 100% |
| Class A2 | -2.03% | 0.82% | -0.14% | 98% | 102% |
| Class B | -0.76% | -1.27% | 1.90% | 101% | 103% |
| Class C | -0.80% | 4.63% | 5.17% | 98% | 105% |
| Class E | -0.50% | -2.80% | 4.99% | 100% | 101% |
| Overall | -1.01% | 1.32% | 3.24% | 100% | 103% |
| Class D | -0.79% | 5.46% | 5.66% | 100% | 104% |
| Class F | -0.75% | -0.44% | -1.09% | 100% | 105% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.84% | 6.05% | 4.89% | 100% | 102% |
| Class A2 | -2.03% | 0.65% | -5.13% | 101% | 104% |
| Class B | -0.54% | -5.37% | -1.39% | 101% | 102% |
| Class C | -0.78% | 6.62% | 7.49% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | -1.16% | 1.31% | 1.49% | 101% | 102% |
| Class D | -0.68% | 6.31% | 7.50% | 101% | 105% |
| Class F | -0.96% | 2.40% | 2.68% | 101% | 102% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.51% | -6.32% | -4.62% | 99% | 102% |
| Class C | -0.80% | 7.60% | 8.53% | 100% | 100% |
| Class E | -0.29% | -4.88% | 8.79% | 101% | 106% |
| Overall | -0.55% | -1.32% | 3.12% | 100% | 102% |
| Class D | -0.58% | 6.67% | 9.82% | 102% | 104% |
| Class F | -0.93% | -0.33% | 1.41% | 102% | 102% |

Table 4. The performance of JVET-P0173 (6 coeff wo shift)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | -2.80% | -2.76% | 103% | 101% |
| Class A2 | 0.13% | -7.17% | -8.05% | 101% | 103% |
| Class B | 0.07% | -9.61% | -7.01% | 101% | 103% |
| Class C | 0.08% | -3.20% | -2.77% | 100% | 102% |
| Class E | 0.13% | -9.35% | -2.79% | 101% | 101% |
| Overall | 0.10% | -6.60% | -4.83% | 101% | 102% |
| Class D | 0.07% | -2.72% | -2.09% | 101% | 105% |
| Class F | 0.14% | -5.81% | -6.21% | 101% | 105% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | -4.37% | -3.59% | 102% | 102% |
| Class A2 | 0.24% | -9.11% | -14.21% | 101% | 102% |
| Class B | 0.18% | -15.93% | -11.52% | 102% | 102% |
| Class C | 0.09% | -3.74% | -3.07% | 103% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.16% | -9.00% | -8.22% | 102% | 102% |
| Class D | 0.14% | -4.70% | -3.68% | 102% | 100% |
| Class F | 0.17% | -4.00% | -3.88% | 103% | 103% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.17% | -17.58% | -14.83% | 100% | 103% |
| Class C | 0.09% | -3.68% | -3.16% | 100% | 99% |
| Class E | 0.14% | -13.71% | -4.03% | 101% | 101% |
| Overall | 0.13% | -11.98% | -8.24% | 100% | 101% |
| Class D | 0.13% | -5.65% | -3.80% | 102% | 103% |
| Class F | 0.04% | -6.96% | -4.99% | 103% | 103% |

Table 5. The performance of JVET-P0173 (7 coeff wo shift)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | -2.96% | -3.14% | 103% | 100% |
| Class A2 | 0.13% | -7.52% | -8.30% | 101% | 103% |
| Class B | 0.08% | -9.81% | -7.56% | 103% | 103% |
| Class C | 0.08% | -3.29% | -2.83% | 103% | 102% |
| Class E | 0.14% | -9.80% | -3.26% | 103% | 103% |
| Overall | 0.10% | -6.84% | -5.18% | 103% | 102% |
| Class D | 0.08% | -2.72% | -2.07% | 103% | 108% |
| Class F | 0.15% | -5.98% | -6.21% | 101% | 104% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | -4.88% | -4.64% | 102% | 103% |
| Class A2 | 0.26% | -9.88% | -14.72% | 102% | 104% |
| Class B | 0.21% | -16.74% | -12.99% | 102% | 102% |
| Class C | 0.09% | -4.15% | -3.37% | 104% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.17% | -9.64% | -9.10% | 103% | 103% |
| Class D | 0.12% | -4.70% | -3.77% | 103% | 100% |
| Class F | 0.20% | -4.16% | -4.05% | 103% | 104% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.17% | -19.04% | -17.39% | 101% | 103% |
| Class C | 0.09% | -5.47% | -4.26% | 101% | 99% |
| Class E | 0.14% | -15.85% | -4.55% | 101% | 103% |
| Overall | 0.14% | -13.72% | -9.80% | 101% | 102% |
| Class D | 0.13% | -6.25% | -3.99% | 101% | 99% |
| Class F | 0.27% | -7.73% | -5.98% | 103% | 102% |

Table 6. The performance of JVET-P0173 (8 coeff wo shift)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | -3.09% | -3.43% | 104% | 102% |
| Class A2 | 0.13% | -7.76% | -8.40% | 102% | 104% |
| Class B | 0.08% | -10.18% | -7.75% | 102% | 103% |
| Class C | 0.09% | -3.59% | -3.34% | 100% | 100% |
| Class E | 0.15% | -10.22% | -3.30% | 101% | 103% |
| Overall | 0.11% | -7.13% | -5.42% | 102% | 102% |
| Class D | 0.08% | -2.77% | -2.63% | 103% | 108% |
| Class F | 0.15% | -5.99% | -6.31% | 102% | 104% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | -4.87% | -4.73% | 102% | 102% |
| Class A2 | 0.24% | -10.08% | -14.94% | 102% | 103% |
| Class B | 0.19% | -16.81% | -13.46% | 103% | 102% |
| Class C | 0.09% | -4.33% | -3.67% | 103% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.16% | -9.75% | -9.40% | 103% | 102% |
| Class D | 0.07% | -4.63% | -3.74% | 104% | 101% |
| Class F | 0.20% | -4.27% | -4.17% | 104% | 104% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.19% | -19.33% | -17.45% | 101% | 102% |
| Class C | 0.14% | -5.44% | -4.45% | 102% | 102% |
| Class E | 0.15% | -16.13% | -5.20% | 102% | 103% |
| Overall | 0.16% | -13.90% | -10.05% | 101% | 102% |
| Class D | 0.11% | -6.38% | -3.90% | 102% | 101% |
| Class F | 0.21% | -7.91% | -6.68% | 103% | 102% |

Table 7. The performance of JVET-P0173 (clipping wo shift)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.13% | -3.49% | -4.07% | 106% | 104% |
| Class A2 | 0.11% | -8.45% | -8.83% | 102% | 104% |
| Class B | 0.10% | -10.36% | -8.36% | 102% | 103% |
| Class C | 0.13% | -4.18% | -3.90% | 101% | 102% |
| Class E | 0.19% | -10.03% | -3.45% | 101% | 103% |
| Overall | 0.13% | -7.47% | -5.92% | 102% | 103% |
| Class D | 0.12% | -2.68% | -3.25% | 102% | 105% |
| Class F | 0.17% | -5.73% | -5.97% | 103% | 107% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | -5.66% | -5.26% | 102% | 103% |
| Class A2 | 0.20% | -11.45% | -15.47% | 101% | 103% |
| Class B | 0.22% | -17.44% | -13.97% | 102% | 103% |
| Class C | 0.08% | -5.01% | -4.25% | 103% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.16% | -10.57% | -9.93% | 102% | 102% |
| Class D | 0.12% | -4.46% | -4.46% | 104% | 102% |
| Class F | 0.20% | -4.73% | -4.55% | 104% | 103% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.18% | -21.36% | -18.88% | 101% | 103% |
| Class C | 0.12% | -8.30% | -6.67% | 102% | 103% |
| Class E | 0.02% | -16.23% | -6.51% | 103% | 103% |
| Overall | 0.12% | -15.72% | -11.72% | 102% | 103% |
| Class D | 0.23% | -7.88% | -6.36% | 102% | 102% |
| Class F | 0.20% | -10.65% | -7.65% | 104% | 103% |