| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0855 |
| --- | --- |

| Title: | CE1 related: Dynamic range analysis in reference picture resampling process |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET or Output document approved by JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E. Alshina Riesstr. 25, Munich, 80992,Germany | Tel: Email: | +491624302163 elena.alshina@huawei.com |
| Source: | Huawei Technologies Düsseldorf, GmbH |  |  |

_____________________________

# Abstract

In VVC draft 6, reference picture resampling process uses existed motion compensation filters. In CE1 and related contributions, depending on the scaling ratio alternative fixed filter coefficients for reference picture down-sampling have been tested. In this contribution, dynamic range of re-sampling process is analysed for different filters in CE1 and CE1 related contributions. Except, JVET-P0083 all analysed filters with predefined coefficients have dynamic range not higher than HEVC motion compensation interpolation process (this module can be re-used for re-sampling with just filter coefficients change). But non-restricted interpolation filter coefficients allowed by syntax proposed in JVET-P0593 might result in overflow of 16 bits temporal buffer. Additional restriction to the sum of interpolation filter positive coefficients to be not greater than 96 ensures dynamic range to be the same as HEVC motion compensation interpolation. Among all analysed re-sampling filters the one in JVET-P0390 demonstrates the best performance in CE1 test conditions.

# Introduction

Fractional position interpolation processes described in sections 8.5.6.3.2 Luma sample interpolation filtering process and 8.5.6.3.4 Chroma sample interpolation process of VVC draft 6. Resampling process is essentially 2D separable filter with 7 bits (6 bits magnitude + 1 bit sign) coefficients. De-scaling shift after first stage of interpolation (the variable shift1) is set equal to Min( 4, BitDepth − 8 ), which ensures 16 bits depth of temporal buffer.

Dynamic range accumulated durign interpolation depends on filter coefficients. Re-sampling interpolation filters proposed CE1 and related contribution use up to 12 taps. Dynamic range for those filters is analysed in this contribution.

# Dynamic range analysis

2D separable interpolation filter process is depicted on Fig. 1.

Fig. 1 2D separable interpolation filter

Here

Reg1(j,i)=k=KMinKMaxfk(px)∙sigj,i+k; Temp1(j,i)=Reg1(j,i)>>shift1

sigj,i is reference picture signal in position [i,j], inputMin=0≤sigj,i≤2d-1=inputMax, d is internal bit-depth

fk(px) are interpolation filter coefficients for fractional position px.

If sum of interpolation filter coefficients

norm=k=KMinKMaxfk(px),

and sums of positive and negative coefficients of interpolation filter

SumPos=k=KMinKMax(fkpx>0? fkpx:0) ; SumNeg=norm-SumPos

are known then dynamic range of register during first stage of interpolation is

| -a | +b | -c | +d | +f | -e | +h | -j | - Max |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| inputMin | inputMax | inputMin | inputMax | inputMax | inputMin | inputMax | inputMin |  |
| -a | +b | -c | +d | +f | -e | +h | -j | - Min |
| inputMax | inputMin | inputMax | inputMin | inputMin | inputMax | inputMin | inputMax |  |

MinReg1=SumNeg∙inputMax+SumPos∙inputMin≤Reg1j,i=k=KMinKMaxfkpx∙sigj,i+k≤SumPos∙inputMax+SumNeg∙inputMin= MaxReg1

and bit-depth of register is

BDReg1=celling( log2(MaxReg1- MinReg1+1))+1      (Eq. 1)

Similar for temporal buffer

MinTemp1=(MinReg1≫shift1)≤Temp1j,i≤ MaxReg1≫shift1=MaxTemp1

and

BDTemp1=celling( log2(MaxTemp1- MinTemp1+1))+1    (Eq. 2)

So, dynamic range of Register and Temporal buffer are determined by signal (1) bit-depth (d =BitDepthY), sum of positive coefficients of interpolation filter, normalization factor and de-scaling shift 1.

Table 1 show dynamic range for Luma motion compensation interpolation filters in VVC draft 6 (the worst case from veiw point dynamic range accumulation is 1/2-pel filter).

Table 1. Dynamic range analysis for re-sampling process with different filters in CE1 and CE1 related contributions.

| Filter | Length | Norm | SumPos | BDReg1 | shift 1 | BD Temp1 | BD-rate (PSNR1) | BD-rate (PSNR2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Luma MC filter | 8 | 64 | 88 | d + 8 | d– 8 | 16 | 0.0% | 0.0% |
| JVET-P0083, 2:1 | 12 | 128 | 152 | d + 9 | d– 8 | 17 | 6.5% | 4.0% |
| JVET-P0083, 1,5:1 | 12 | 128 | 156 | d + 9 | d– 8 | 17 | 3.1% | 2.2% |
| JVET-P0088, 2:1 | 8 | 64 | 70 | d + 8 | d– 8 | 16 | 4.5% | 2.9% |
| JVET-P0088, 1,5:1 | 8 | 64 | 72 | d + 8 | d– 8 | 16 | 2.2% | 1.9% |
| JVET-P0088, 4:1 | 8 | 64 | 64 | d + 7 | d– 8 | 15 | 9.5% | 4.6% |
| JVET-P0353, 2:1 | 8 | 64 | 70 | d + 8 | d– 8 | 16 | 4.4% | 2.8% |
| JVET-P0353, 1,5:1 | 8 | 64 | 74 | d + 8 | d– 8 | 16 | 2.1% | 1.7% |
| JVET-P0390, 2:1 | 8 | 64 | 70 | d + 8 | d– 8 | 16 | 5.3% | 2.9% |
| JVET-P0628, 2:1 | 8 | 64 | 68 | d + 8 | d– 8 | 16 | 2.1% | 0.3% |
| JVET-P0628, 1,5:1 | 8 | 64 | 70 | d + 8 | d– 8 | 16 |  |  |
| JVET-P0628, 4:1 | 8 | 64 | 64 | d + 7 | d– 8 | 15 |  |  |

Except, CE1-3 (JVET-P0083) all proponents keep normalization factor (64) and filter length (8) aligned with Luma motion compensation interpolation in VVC draft 6.

For JVET-P0353 version 2 uploaded at date 2019-09-30 22:49:16 was used for analysis.

SumPos parameter for all proposed filters (except CE1-3) is smaller than SumPos = 88 Luma motion compensation interpolation in VVC draft 6, so dynamic range of Register and Temporal buffer same or even lover and for motion compensation interpolation in VVC draft 6. It means motion compensation module can be re-used with just modified filter coefficients and adding non-trivial interpolation for integer-pel position.

Excel file which performs dynamic range calculation in 2D separable interpolation filter process is attached to this contribution and can be useful for filter assessment during BoG or/and future CEs.

Among all filters proposed for scaling ratio 2:1 the one in JVET-P0390 demonstrates the best performance (5.3% in PSNR1; 2.9% in PSNR2).

# Signalled coefficients

Considering variety of re-sampling filters tested in CE1 and related contributions JVET-P0593 proposes signaling for re-sampling filter coefficients. Proposed in JVET-P0593 syntax assumes luma uses 8-taps interpolation filter, Chroma uses 4-taps, both have Normalization factor 64 (6 bits for interpolation filter coefficients magnitude).

But only normalization factor is ensured by syntax proposed in JVET-P0593, SumPos value which affects dynamic range is not restricted.

For example, both VVC draft 6 1/2 interpolation {-1 4 -11 40 40 -11 4 -1} (SumPos = 88) and {-64 64 -64 96 96 -64 64 -64 } (SumPos =320) are allowed by JVET-P0593 syntax; both filters perform 1/2 interpolation, both have normalization factor (sum of coefficients) 64. But according to according to Eq. 1 and Eq.2 dynamic range of second filter is 3 bits higher and temporal buffer would overflow.

Table 2. Dynamic range analysis for re-sampling process with different filters allowed by JVET-P0593 .

| Filter | Length | Norm | SumPos | BDReg1 | shift 1 | BDTemp1 |
| --- | --- | --- | --- | --- | --- | --- |
| {-1 4 -11 40 40 -11 4 -1} | 8 | 64 | 88 | d + 8 | d– 8 | 16 |
| {-64 64 -64 96 96 -64 64 -64 } | 8 | 64 | 320 | d + 11 | d– 8 | 19 |

By combining Eq. 1 and Eq. 2 we can prove that 16 bit depth of temporal buffer can be ensured for filters with normalization factor 64 id SumPos is not greater than 96:

SumPos≤96;norm=64;shift1=d-8→ BDTemp1=16 bits

| Filter | Length | Norm | SumPos | BDReg1 | shift 1 | BDTemp1 |
| --- | --- | --- | --- | --- | --- | --- |
| {-1 4 -11 40 40 -11 4 -1} | 8 | 64 | 88 | d + 8 | d– 8 | 16 |
| {-8 8 -8 40 40 -8 8 -8} | 8 | 64 | 96 | d + 11 | d– 8 | 16 |

In JVET-P0593 largest positive coefficients of interpolation filter is not signalled but derived from other coefficients and normalization factor. Simple clipping of this coefficients to adjust SumPos to be not greater than 96 would ensure dynamic range in re-sampling process.

# References

- D. Luo, V. Seregin, W. Wan, JVET-O2021 Description of Core Experiment 1 (CE1): Reference picture resampling filters Gothenburg 2019

- J. Luo, M. Sarwer, Y. Ye, (Alibaba) CE1-3: Reference picture resampling filters, JVET-P0083

- J. Samuelsson, S. Deshpande, A. Segall (Sharp Labs of America), CE1-1: RPR downsampling filter JVET-P0088

- J. Luo, Y. Ye, M. Sarwer (Alibaba), CE1-related: Reference picture resampling filters , JVET-P0353

- A. Alshin, J. Boyce, P. Frolov, V. Aristarkhov (Intel), CE1-related: Anti-aliasing motion compensation interpolation downsampling filters for reference picture resampling, JVET-P0390

- Pankaj Topiwala, Madhu Krishnan, Wei Dai(FastVDO), CE1-2: RPR Resampling Filters, JVET-P0628

- V. Seregin, M. Coban, M. Karczewicz (Qualcomm), AHG8/CE1-related: Signalling filter coefficients for resampling,JVET-P0593

# Conclusions

Reusing motion compensation interpolation design for re-sampling filters essentially reduces implementation cost for reference picture resampling support. Except, JVET-P0083 all analysed filters with predefined coefficients have dynamic range not higher than HEVC motion compensation interpolation process (this module can be re-used for re-sampling with just filter coefficients change). But non-restricted interpolation filter coefficients allowed by syntax proposed in JVET-P0593 might result in overflow of 16 bits temporal buffer. Additional restriction to the sum of interpolation filter positive coefficients to be not greater than 96 ensures dynamic range. Among all analysed re-sampling filters the one in JVET-P0390 demonstrates the best performance in CE1 test conditions.

# Patent rights declaration(s)

Huawei Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).