| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0922-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-P0511 on Non-CE3: ISP extension to avoid small Chroma intra blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Suhong Wang, Yueming Wang, Xiaozheng Zheng | Tel: Email: | suhong.wang@pku.edu.cn |
| Source: | Peking University, SZ DJI Technology Co., Ltd. |  |  |

_____________________________

- Abstract

This contribution is a cross-check report of JVET-P0511. It is confirmed that the cross-checked results are identical with the proponent’s results except the simulation time is unreliable.

Introduction

Brief technical description of JVET-P0511

In this contribution, an alternative method to avoid small chroma intra blocks is proposed, which needs less software and spec text changes. It includes two steps. In the first step, 4x4, 4x8, and 8x4 intra CU is disallowed in single tree so that 2x2, 2x4, and 4x2 chroma intra CBs is avoided. In the second step, ISP is extended to support binary tree split (BT) for 8x8, 4x16, and 16x4 CUs, and one flag is added to indicate whether current luma CU is split into 2 partitions or 4 partitions. Same as ISP in VTM-6, the extended ISP is only applied to luma component.

Experimental Results

The simulation is conducted using VTM-6.0 with the test condition described in JVET-N1010. The encoding time is unreliable.

For the finished tests, the cross-checked results are identical with the proponent’s results other than the encoding time.

Table 1 Experimental Results of JVET-P0511

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | 0.03% | 109% | 106% |
| Class A2 | 0.01% | -0.01% | 0.01% | 94% | 100% |
| Class B | 0.01% | 0.01% | -0.03% | 147% | 125% |
| Class C | 0.00% | -0.12% | 0.04% | 97% | 95% |
| Class E | 0.00% | 0.00% | 0.01% | 150% | 134% |
| Overall | 0.01% | -0.03% | 0.01% | 119% | 112% |
| Class D | -0.02% | 0.02% | -0.03% | 102% | 101% |
| Class F | 0.27% | 0.04% | 0.09% | 98% | 105% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.07% | 0.05% | -0.04% | 103% | 107% |
| Class C | 0.23% | 0.09% | 0.03% | 102% | 106% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.16% | -0.18% | -0.01% | 101% | 102% |
| Class F | 0.21% | 0.20% | 0.27% | 102% | 97% |

Conclusion

It is confirmed that the cross-checked results are identical with the proponent’s results for the finished tests except the running time is not reliable in the cross-checking.