| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0934 |
| --- | --- |

| Title: | Crosscheck of JVET-P0281: Corrections on parameter calculation for PROF and BDOF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Chen 9276 Scranton Road, Suite 300 San Diego, CA 92121, USA | Tel: Email: | 1-858-210-4855 Wei.Chen@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution reports the cross-checked results for JVET-P0281. The simulation results do not change CTC simulation results, which match with that reported by the proponents. The time complexity is not crosschecked because our grid was too busy to be accurate.

# Introduction

JVET- P0281 [1] proposes following corrections and clean-ups for both PROF and BDOF.

Method I. Remove of bit-depth factor on parameter calculation: In current PROF and BDOF design, the bit-width of factors on parameter calculation depends on the input bit-depth. However, bit-width of the motion vector difference and some parameters are decided according to the input bit-depth even though it has no correlation with bit-depth, and gradient has fixed bit-width even though it has high correlation. Therefore, bit-depth factors on parameter calculation are removed in this proposal.

Method II. Minimize the truncation error: The gradient and motion vector difference are used to derive the refinement offset. But, right-shift operation for normalization of the refinement offset is performed just for gradient. This operation makes gradient parameter to have the truncation error. Therefore, methods to reduce truncation error are introduced.

Method III. Prevent the 16-bit overflow: The range of the prediction sample is within 16- bit range, but the range of the refinement offset exceeds the 16-bit. In this proposal, range of the refinement offset is restricted to prevent the 16-bit overflow.

In this contribution, test 2, test 3 and test 4 of JVET- P0281 are crosschecked, where in test 2 and test 3, proposed Method II and Method III for PROF is tested, while in test 4, clipping range of the refinement offset for PROF in Method III is [-4096, 4095], which is aligned to clipping range of BDOF.

# Simulation results

The simulations were performed using VTM-6.0 with those changes proposed in [1], according to common test conditions. The encoding results compared to VTM-6.0 anchor are summarized in Table 1, Table 2 and Table 3.

Table 1.  Crosschecked test 2 of JVET- P0281

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM 6.0 |  |  |  |  | Over VTM 6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.03% | -0.01% | 107% | 114% | - | - | - | - | - |
| Class A2 | 0.01% | 0.04% | 0.07% | 108% | 114% | - | - | - | - | - |
| Class B | -0.01% | 0.11% | -0.02% | 102% | 102% | -0.02% | 0.13% | -0.01% | 113% | 125% |
| Class C | 0.01% | 0.03% | 0.02% | 101% | 100% | -0.03% | 0.09% | 0.03% | 103% | 118% |
| Class E | - | - | - | - | - | -0.05% | 0.09% | 0.14% | 103% | 109% |
| Overall | 0.00% | 0.06% | 0.01% | 104% | 106% | -0.03% | 0.11% | 0.04% | 107% | 119% |
| Class D | -0.02% | -0.05% | -0.05% | 101% | 102% | -0.04% | 0.62% | 0.35% | 101% | 101% |
| Class F | 0.00% | 0.03% | -0.02% | 101% | 101% | 0.09% | 0.24% | 0.30% | 106% | 113% |

Table 2.  Crosschecked test 3 of JVET- P0281

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM 6.0 |  |  |  |  | Over VTM 6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 106% | 114% | - | - | - | - | - |
| Class A2 | 0.00% | 0.00% | 0.00% | 108% | 114% | - | - | - | - | - |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 103% | 0.00% | -0.01% | -0.01% | 112% | 125% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 101% | 0.00% | 0.00% | 0.00% | 103% | 118% |
| Class E | - | - | - | - | - | 0.00% | 0.00% | 0.00% | 102% | 110% |
| Overall | 0.00% | 0.00% | 0.00% | 103% | 107% | 0.00% | 0.00% | 0.00% | 107% | 119% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 102% | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class F | 0.01% | 0.01% | 0.01% | 100% | 100% | 0.07% | 0.20% | 0.00% | 104% | 114% |

Table 3.  Crosschecked test 4 of JVET- P0281

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM 6.0 |  |  |  |  | Over VTM 6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.00% | 106% | 114% | - | - | - | - | - |
| Class A2 | 0.00% | -0.01% | -0.01% | 107% | 114% | - | - | - | - | - |
| Class B | -0.01% | -0.01% | -0.01% | 101% | 103% | 0.00% | 0.15% | -0.04% | 112% | 125% |
| Class C | 0.00% | -0.01% | -0.03% | 100% | 100% | 0.00% | 0.01% | -0.04% | 103% | 117% |
| Class E | - | - | - | - | - | 0.02% | 0.11% | 0.18% | 102% | 110% |
| Overall | 0.00% | -0.01% | -0.01% | 103% | 106% | 0.00% | 0.09% | 0.02% | 106% | 118% |
| Class D | 0.00% | -0.02% | 0.01% | 101% | 102% | 0.05% | 0.08% | 0.07% | 100% | 101% |
| Class F | 0.01% | -0.02% | 0.01% | 100% | 101% | 0.12% | 0.05% | 0.07% | 105% | 113% |

# Reference

- N. Park, J. Nam, H. Jang, J. Lim, S. Kim, “Corrections on parameter calculation for PROF and BDOF”, JVET-P0281, Geneva, CH, 1–11 October 2019.

# Patent rights declaration(s)

InterDigital Communications Inc. does not have any current or pending patent rights relating to the technology described in this contribution.