| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0981 |
| --- | --- |

| Title: | Cross-check of JVET-P0611 (Non-CE5/AHG-11: Boundary strength harmonization for BDPCM, TS, Palette and IBC) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | K. Andersson | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution verifies that the software matches the description and that the software gives the results provided in JVET-P0611 [1]. Visual quality was not investigated but it can be noted that there is a risk that the proposal introduces blocking artifacts since deblocking is by passed if one of the blocks on the side of the boundary uses transform skip or BDPCM or IBC or palette and the other block uses transform and has non-zero coefficients.

# Introduction

JVET-P0611 [1] propose to modify the boundary strength determination in the VVC draft [2]. If any of the following cases are used on one side of the block boundary, BDPCM, transform skip, transform bypass, IBC or palette, the boundary strength for luma is set to 0. If IBC, palette or transform bypass is used on one side of the block boundary for chroma the boundary strength for chroma is set to 0.

# Verification of software

It is verified that the software matches the proposal except for that the software also set boundary strength to 0 when transform bypass is used on one side of the boundary which not is part of the specification text for the proposal. The proponent has clarified that it is not the intention of the proposal.

# Observation

It is possible that the proposal can introduce blocking artifacts if one of the blocks on the boundary uses transform skip or BDPCM or IBC or palette and the other block uses transform and has non-zero coefficients, since deblocking is skipped in this case.

# Results

The performance for the proposal is shown below in comparison to VTM-6.0 [3] using CTC [4] and also in comparison to TGM test class. The BDR impact is minor. Encoding time is measured on simulations run on a cluster with machines with same capability. Decoding time is measured by running test and anchor on same machine without yuv output. Encoding and decoding time is similar as for the anchor. There is a slight difference in BDR for TGM class due to the cross-check was using 600 frames of Flying graphics where the CTC says it should be 300 frames as also the proponent has used.

|  | All intra Over VTM-6.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,10% | 0,00% | 0,00% | 106% | 101% |
| Class A2 | -0,01% | 0,00% | 0,00% | 106% | 100% |
| Class B | -0,01% | 0,00% | 0,00% | 107% | 101% |
| Class C | -0,04% | 0,00% | 0,01% | 100% | 103% |
| Class E | 0,00% | 0,00% | 0,00% | 101% | 103% |
| Overall | -0,03% | 0,00% | 0,00% | 104% | 102% |
| Class D | -0,01% | -0,01% | -0,01% | 105% | 101% |
| Class F | -0,01% | 0,02% | 0,03% | 108% | 100% |
| TGM | -0,27% | -0,01% | -0,01% | 101% | 97% |

|  | Random access Over VTM-6.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,03% | 0,07% | 0,00% | 103% | 97% |
| Class A2 | 0,02% | 0,06% | 0,05% | 102% | 98% |
| Class B | -0,02% | 0,05% | 0,02% | 101% | 98% |
| Class C | -0,04% | 0,09% | -0,04% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0,02% | 0,07% | 0,01% | 102% | 99% |
| Class D | -0,06% | -0,10% | -0,01% | 102% | 101% |
| Class F | 0,01% | 0,04% | -0,02% | 103% | 101% |
| TGM | -0,25% | -0,15% | -0,12% | 100% | 101% |

|  | Low-delay B Over VTM-6.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0,02% | -0,04% | -0,03% | 103% | 101% |
| Class C | -0,01% | 0,02% | 0,22% | 106% | 98% |
| Class E | 0,03% | 0,16% | 0,42% | 103% | 101% |
| Overall | 0,01% | 0,03% | 0,17% | 104% | 100% |
| Class D | -0,04% | 0,52% | 0,33% | 103% | 102% |
| Class F | 0,18% | 0,04% | 0,75% | 106% | 100% |
| TGM | -0,39% | -0,19% | -0,14% | 100% | 98% |

# References

- Bappaditya Ray, Geert Van der Auwera, Marta Karczewicz, “Non-CE5/AHG-11: Boundary strength harmonization for BDPCM, TS, Palette and IBC”, JVET-P0611, Geneva, CH, October 2019.

- B. Bross, J. Chen, and S. Liu, "Versatile Video Coding (Draft 6)", Document of Joint Video Experts Team, JVET-O2001.

- VTM-6.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations”, Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).