| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P1028 |
| --- | --- |

| Title: | AHG18: CE7.1.2d aligned with JVET-P0072 and JVET-P0298 methods for TS only based lossless coding JVET-P0606 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongtao Wang Yung-Hsuan Chao Muhammed Coban Marta Karczewicz | Tel: Email: | +1 858 658 3973 hongtaow@qti.qualcomm.com yunghsua@qti.qualcomm.com mcoban@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution combines CCB count check method of P0072, and level mapping removal in last coding pass of P0298 along with 1.75bin/coefficient CCB limit with CE7-1.2d alternative scheme and reports performance for TS based lossless coding.

# Proposal

CE7-1.2d alternative scheme [2] performs context coded bin (CCB) count within the coding passes. In this proposal, checking of CCB counts at the end of each coded coded bin group as used in CE7-1.3b(alt) scheme [3] is used, hence the CCB count is checks after first pass group of 2 bins and 2nd pass group of 6 bins. In addition to that, in the last coding pass, for baseLevel 0 region, level mapping existing in current TS residual coding scheme is not used as proposed in JVET-P0298 [4]. CCB count is reduced to 1.75bin per coefficient. The proposed TS residual coding schemet is applied to TS based lossless coding scheme proposed in JVET-P0606 [5].

# Results

The proposed scheme is implemented on top of VTM-6.0 software that includes TS chroma support [5]. The simulations were carried out under the JVET common test conditions as well as lossless coding configuration. BDPCM (luma) is enabled for all classes for lossless configuration.

The lossless performance of the proposed scheme is shown in Table 1. Anchor used is the AHG18 SW VTM-6.0 anchor.

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM6 | 0 |  | VTM6 | 0 |  | VTM6 | 0 |  |
| Class A1 | 2.2 | 2.2 | -0.13% | 2.3 | 0.0 | #DIV/0! |  |  |  |
| Class A2 | 1.7 | 1.8 | -0.35% | 1.9 | 0.0 | #DIV/0! |  |  |  |
| Class B | 2.1 | 2.1 | -1.10% | 2.3 | 2.2 | 1.80% | 2.3 | 0.0 | #DIV/0! |
| Class C | 2.1 | 2.1 | -0.90% | 2.6 | 2.5 | 3.08% | 2.6 | 0.0 | #DIV/0! |
| Class D | 1.9 | 2.0 | -2.73% | 2.8 | 2.7 | 2.70% | 2.7 | 2.7 | 2.93% |
| Class E | 2.9 | 3.0 | -0.79% |  |  |  | 3.3 | 0.0 | #DIV/0! |
| Class F | 5.5 | 5.8 | -4.03% | 39.4 | 41.7 | -2.13% | 60.7 | 127.0 | #DIV/0! |
| TGM | 12.3 | 13.2 | -6.29% | 112.6 | 123.2 | -7.50% | 142.4 | 205.5 | #DIV/0! |
| Overall | 2.2 | 2.2 | -0.72% | 2.3 | 1.4 | #DIV/0! | 2.6 | 0.0 | #DIV/0! |
| Enc Time[%] | 64% |  |  | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | 100% |  |  | #NUM! |  |  | #NUM! |  |  |

Table 1: Lossless performance of proposed scheme compared to AHG18 VTM-6.0 anchor.

CTC results for the proposed scheme is shown in Table 2 below.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.00% | -0.01% | -0.04% | 100% | 99% |
| Class C | 0.01% | -0.08% | 0.08% | 101% | 98% |
| Class E | -0.01% | 0.02% | 0.00% | 100% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | 0.01% | -0.07% | 101% | 101% |
| Class F | 0.07% | -0.04% | -0.06% | 100% | 102% |
| TGM | 0.16% | 0.11% | 0.10% | 100% | 99% |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.01% | 0.01% | 0.05% | 100% | 99% |
| Class C | 0.02% | 0.06% | 0.07% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.08% | -0.21% | -0.05% | 101% | 102% |
| Class F | 0.05% | -0.04% | 0.07% | 100% | 100% |
| TGM | 0.08% | 0.02% | 0.05% | 100% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.01% | -0.13% | -0.06% | 105% | 102% |
| Class E | -0.09% | -0.06% | 0.26% | 99% | 102% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | -0.31% | -0.01% | 100% | 101% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 2: Average Bjøntegaard delta rate savings of the proposed method relative to VTM-6.0 anchor for low QP conditions.

Bin to bit ratio analysis is conducted according to CE7 testing conditions [5]. For CTC conditions, the bin to bit ratio analysis is provided in Table 3.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Gothenburg, SE, July 2019.

- M. Karczewicz, H. Want, Y.-H Chao, M. Coban, “CE7: Modifications to transform-skip residual coding (CE7-1.2)”, JVET-P0079, Geneva, CH, October 2019.

- M. G. Sarwer, R. -L. Liao, Y. Ye, “CE7-1.3: Simplification of transform-skip residual coding”, JVET-P0072, Geneva, CH, October 2019.

- Y. Kato, K. Abe, T. Toma, “CE7-related: Unification of CCB check method and bypass coding between two residual coding modes”, JVET-P0298, Geneva, CH, October 2019.

- B. Bross, T. Nguyen, H. Schwarz, D. Marpe, T. Wiegand (HHI), M. Karczewicz, Y.-H. Chao, H. Wang, M. Coban, “AHG18: Enabling lossless coding with minimal impact on VVC design”, JVET-P0606, Geneva, CH, October 2019.

- https://vcgit.hhi.fraunhofer.de/VVCSoftware_VTM

- “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, CH, March 2019.

- H. Schwarz, M. Coban, C. Auyeung, “Description of Core Experiment 7 (CE 7): Quantization and coefficient coding,” JVET-O2027, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Gothenburg, SE, July 2019.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).