| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P1034-v1 |
| --- | --- |

| Title: | CE7-related: Improved coding of user defined quantization matrices |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Olena Chubach, Chen-Yen Lai, Ching-Yeh Chen, Tzu-Der Chuang, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei Philippe de Lagrange, Fabrice Le Léannec, Edouard François, Karam Naser | Email: | {olena.chubach, jenny.lai, chingyeh.chen, peter.chuang, cw.hsu, yuwen.huang, shawmin.lei}@mediatek.com philippe.delagrange@interdigital.com |
| Source: | MediaTek Inc., InterDigital |  |  |

_____________________________

# Abstract

In this contribution, an improved predictive coding of user defined quantization matrices (QMs) is proposed. The current design in VVC Draft 6 allows either referencing one of the previously coded QMs having the same sizeId as the current QM or DPCM coding of elements within the current QM. This contribution suggests simple changes on top of the current design to improve coding efficiency as follows.

- Allow referencing a previously coded QM with the same base size as the current QM.

- Allow coding element-to-element differences between the current QM and the reference QM.

- Keep the original DPCM coding of elements within the current QM.

- Use a single matrix identifier scalingListId which combines matrixId and sizeId.

The number of bits required for each of the three methods is computed, and the method requiring minimum number of bits will be chosen for coding of the current QM. The proposed method was implemented on top of VTM6.1.

# Introduction

VVC, having a larger number of block sizes compared to HEVC, requires coding of more QMs. In VVC Draft 6, when user defined QMs are specified, current coding procedure is limited to the two methods:

- Reference one of the previously decoded QMs if it is equal to the current to-be-coded QM and has the same sizeId.

- Apply DPCM coding of elements within the current to-be-coded QM.

# Proposed method

This contribution targets at improving coding efficiency for user defined QMs while keeping the design simple for understanding. It is proposed to:

- Allow referencing a previously coded QM with the same base size as the current QM.

- Allow coding element-to-element differences between the current QM and the reference QM.

- Keep the original DPCM coding of elements within the current QM.

- Use a single matrix identifier scalingListId which combines matrixId and sizeId.

The number of bits required for each of the three methods is computed, and the method requiring minimum number of bits will be chosen for coding of the current QM.

In addition, it is proposed to use a single matrix identifier scalingListId which combines matrixId and sizeId in the syntax signalling.

## Allow referencing a previously coded QM

Table 1 shows an example of selected QMs for block sizes 8x8 and 16x16. As it can be seen, all three QMs are equal. In case of VVC Draft 6 design, QM for INTRA8X8_CHROMAU can be decoded by copying a previously decoded QM for INTRA8X8_LUMA. However, QM for INTRA16X16_LUMA cannot be decoded in the same way, since it has a different sizeId from that of 8x8, and prediction between QMs with different sizeIds is not allowed in VVC Draft 6.

Table 1. Selected QMs for block sizes 8x8 and 16x16 (HEVC default)

| INTRA8X8_LUMA = | INTRA8X8_CHROMAU = | INTRA16X16_LUMA = |
| --- | --- | --- |
| 16, 16, 16, 16, 17, 18, 21, 24 16, 16, 16, 16, 17, 19, 22, 25 16, 16, 17, 18, 20, 22, 25, 29 16, 16, 18, 21, 24, 27, 31, 36 17, 17, 20, 24, 30, 35, 41, 47 18, 19, 22, 27, 35, 44, 54, 65 21, 22, 25, 31, 41, 54, 70, 88 24, 25, 29, 36, 47, 65, 88, 115 | 16, 16, 16, 16, 17, 18, 21, 24 16, 16, 16, 16, 17, 19, 22, 25 16, 16, 17, 18, 20, 22, 25, 29 16, 16, 18, 21, 24, 27, 31, 36 17, 17, 20, 24, 30, 35, 41, 47 18, 19, 22, 27, 35, 44, 54, 65 21, 22, 25, 31, 41, 54, 70, 88 24, 25, 29, 36, 47, 65, 88, 115 | 16, 16, 16, 16, 17, 18, 21, 24 16, 16, 16, 16, 17, 19, 22, 25 16, 16, 17, 18, 20, 22, 25, 29 16, 16, 18, 21, 24, 27, 31, 36 17, 17, 20, 24, 30, 35, 41, 47 18, 19, 22, 27, 35, 44, 54, 65 21, 22, 25, 31, 41, 54, 70, 88 24, 25, 29, 36, 47, 65, 88, 115 |

In this proposal, it is suggested to permit referencing QMs with the same base size, while reference QM sizeId can be smaller than or equal to the current QM sizeId. This way, INTRA8X8_LUMA can be used for decoding QM for INTRA16X16_LUMA, which allows reducing bits needed for coding user defined QMs, especially for cases when same base QMs are used across different block sizes.

## Allow coding element-to-element differences

Inter QM prediction mode implies using any of the previously coded QM for decoding of the current QM. For that, a flag is sent to indicate whether inter QM prediction mode is enabled. Then, the reference sizeId and matrixId specified in Tables 2 and 3 are coded together with the delta QM. The delta QM is equal to the difference between the current QM and the reference QM. DPCM is applied for coding delta QM.

Table 2. Size identifier (VTM6.1)

| Luma | Chroma | sizeId |
| --- | --- | --- |
| - | - | 0 |
| - | 2x2 | 1 |
| 4x4 | 4x4 | 2 |
| 8x8 | 8x8 | 3 |
| 16x16 | 16x16 | 4 |
| 32x32 | 32x32 | 5 |
| 64x64 | - | 6 |

Table3. Matrix type identifier (VTM6.1)

| CuPredMode | cIdx (Colour component) | matrixId |
| --- | --- | --- |
| MODE_INTRA | 0 (Y) | 0 |
| MODE_INTRA | 1 (Cb) | 1 |
| MODE_INTRA | 2 (Cr) | 2 |
| MODE_INTER | 0 (Y) | 3 |
| MODE_INTER | 1 (Cb) | 4 |
| MODE_INTER | 2 (Cr) | 5 |

In cases when the QM used for prediction does not have a separate DC value but the current QM has a separate DC value, the top-left value of the reference QM is used to predict the separate DC value of the current QM. As shown in Table 4, INTER8X8_CHROMAV can still be used for predicting of INTRA16X16_LUMA with INTRA16X16_LUMA_DC=2, and the prediction for this value is the top-left element of INTER8X8_CHROMAV which is equal to 5.

Table 4. Selected QMs for block sizes 8x8 and 16x16 (asymmetric7)

| INTER8X8_CHROMAV = | INTRA16X16_LUMA = |
| --- | --- |
| 5, 5, 14, 32, 44, 52, 59, 62 5, 10, 17, 34, 50, 61, 68, 70 8, 12, 25, 37, 57, 70, 84, 91 10, 17, 34, 52, 73, 84, 95, 104 17, 26, 52, 66, 88, 98, 113, 116 23, 32, 57, 73, 97, 107, 118, 122 26, 35, 62, 84, 102, 118, 124, 125 30, 35, 64, 89, 107, 120, 124, 125 | 2, 2, 5, 11, 19, 21, 24, 24 2, 3, 7, 16, 25, 29, 31, 32 2, 4, 8, 21, 39, 44, 47, 47 3, 6, 11, 26, 53, 61, 63, 65 4, 9, 17, 34, 62, 74, 77, 77 6, 13, 22, 47, 71, 79, 84, 86 9, 16, 25, 53, 70, 83, 87, 88 9, 17, 30, 56, 76, 84, 88, 90 |

Although current default QMs are all flat (with the default value 16), the default QMs can also be used for predicting current matrices. No additional flags are needed for this. However, since the default QMs are all flat and do not bring any visual improvements, it is still suggested to remove them and support only user defined QMs.

## Keep the original DPCM coding

When both previous methods are either impossible or inefficient, the original DPCM coding of elements within the current QM can be applied. This mode already exists in VVC Draft 6.

## Use a single matrix identifier

It is proposed to introduce a single matrix identifier scalingListId which is a combined identifier for sizeId and matrixId from Table 2 and Table 3, respectively. The scalingListId is defined as shown in Table 4.

Table 4. Definition of scalingListId

| INTRA | Y |  | 4 | 10 | 16 | 22 | 28 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Cb | 0 | 5 | 11 | 17 | 23 |  |
|  | Cr | 1 | 6 | 12 | 18 | 24 |  |
| INTER, IBC | Y |  | 7 | 13 | 19 | 25 | 29 |
|  | Cb | 2 | 8 | 14 | 20 | 26 |  |
|  | Cr | 3 | 9 | 15 | 21 | 27 |  |
| Block size: max(width,height) |  | 2 | 4 | 8 | 16 | 32 | 64 |
| Decoded QM coefficients |  | 2x2 | 4x4 | 8x8 | 8x8 + DC |  |  |

## Additional optimizations

Additional optimizations are made in order to remove some redundant signalling:

- Skip signalling the sizeId of the reference QM for the QMs with scalingListId equal to 0, 4, and 10.

- The number of bits required in this case will be compared versus the number of bits after DPCM coding, and the method requiring minimum number of bits will be chosen.

- Delta values are signalled for scalingListId when one reference scaling list is required.

## Final scheme design

After applying proposed updates, the coding scheme is as follows:

1

1

0

scalingListId

scalingListId

copyFlag

res DPCM

direct DPCM

predFlag

0

1

1

0

scalingListId

scalingListId

copyFlag

res DPCM

direct DPCM

predFlag

0

If copyFlag is true, then one of the previously decoded quantization matrices is exactly the same as the current QM and only the scalingListId has to be decoded. If copyFlag is false, a predFlag ha to be decoded. If predFlag is true, then scalingListId is decoded. If scalingListId is equal to the scalingListId of the current QM, then a corresponding default (flat) QM is used for decoding a current QM, otherwise (scalingListId is not equal to the scalingListId of the current QM), then a QM with the signaled scalingListId is used for decoding of the current. Otherwise, if predFlag is false, then a direct DPCM coding of the current QM is used.

It should be noted that the elements within orange area are newly introduced decoding processes.

The final decision is made in the end: the number of bits required for each of the three methods is computed and the method requiring minimum number of bits will be chosen for coding of the current QM.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).