| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P2023-v1 |
| --- | --- |

| Title: | Description of Core Experiment 3 (CE3): Lossless Coding |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | CE Description |  |  |
| Author(s) or Contact(s): | T. Nguyen T. C. Ma A. Nalci | Email: | tung.nguyen@hhi.fraunhofer.de tsung-chuanma@kwai.com analci@qti.qualcomm.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Introduction

The purpose of this core experiment is to explore the coding efficiency and complexity impact of proposed algorithms for lossless coding, and if applicable, their impact on the Common Test Conditions (CTC). This core experiment evaluates the impact of:

- BDPCM and high-level techniques for lossless coding

- Regular and TS residual coding (RRC, TSRC) for lossless coding

- Combination of modifications to the category A and B

# Participants List

| Affiliation | Person, Email address |
| --- | --- |
| Fraunhofer HHI | Tung Nguyen tung.nguyen@hhi.fraunhofer.de Benjamin Bross benjamin.bross@hhi.fraunhofer.de |
| Qualcomm | Alican Nalci analci@qti.qualcomm.com Yung-Hsuan Chao yunghsua@qti.qualcomm.com Hongtao Wang hongtaow@qti.qualcomm.com Muhammed Coban mcoban@qti.qualcomm.com |
| MediaTek | Tzu-Der Chuang peter.chuang@mediatek.com Zhi-Yi Lin Zhi-Yi.Lin@mediatek.com |
| Kwai | Tsung-Chuan Ma tsung-chuanma@kwai.com Yi-Wen Chen yiwenchen@kwai.com Xiaoyu Xiu xiaoyuxiu@kwai.com Hong-jheng Jhu jhuhong-jheng@kwai.com Xianglin Wang xianglinwang@kwai.com |
| LG | Jungah Choi jungah.choi@lge.com Hyeongmun Jang hm.jang@lge.com Jaehyun Lim jaehyun.lim@lge.com Seung Hwan Kim seunghwan3.kim@lge.com Mehdi Salehifar mehdi.salehifar@lge.com |
| Alibaba | Yan Ye yan.ye@alibaba-inc.com Ruling Liao ruling.lrl@alibaba-inc.com Jiancong Luo jiancong.luo@alibaba-inc.com Mohammed Sarwer m.sarwer@alibaba-inc.com |
| Sony | Takeshi Tsukuba takeshi.tsukuba@sony.com Masaru Ikeda masaru.ikeda@sony.com Yoichi Yagasaki yoichi.yagasaki@sony.com |

# Proposals under Test

The following proposals are considered by this CE.

|  | Proponent | Document | Tool description |
| --- | --- | --- | --- |
| 1 | Qualcomm | JVET-P0082 | BDPCM for lossless |
| 2 | MediaTek | JVET-P0147 | BDPCM for lossless |
| 3 | MediaTe Qualcom Kwai Alibaba | JVET-P0148 | Disabling state transition for RRC and lossless |
| 4 | Qualcomm | JVET-P0258 | Disabling state transition for RRC and lossless |
| 5 | Alibaba | JVET-P0463 | Modified residual coding for lossless |
| 6 | LGE | JVET-P0466 | Modified residual coding for lossless |
| 7 | Qualcomm | JVET-P0485 | Modified residual coding for lossless |
| 8 | Kwai MediaTek | JVET-P0525 | RRC/TSRC selection for lossless |
| 9 | Kwai | JVET-P0528 | Scanning for lossless |
| 10 | LGE | JVET-P0574 | BDPCM residual coding for lossless |
| 11 | Qualcomm | JVET-P0559 | Rice parameter derivation for lossless |
| 12 | Sony | JVET-P0584 | BDPCM for lossless |
| 13 | Qualcomm | JVET-P1028 | Modified residual coding for lossless |

# Test Description

## Software, reference configurations, and test conditions

All tools have to be integrated into the VTM-7.0 software (outcome of the Geneva meeting). Results have to be reported for the VTM-7.0 software and the common test conditions for lossless, which has BDPCM disabled for camera captured and screen content sequences. The lossless mode can be enabled at the encoder by using the configuration parameter CostMode=lossless.

All tools are tested against the VTM-7.0 anchor (same set of tools, except the tool to be tested).

For tests in category B and C targeting modifications to either RRC or TSRC, results have to be provided for two ranges of QP values, using the CTC configurations (HashME, IBC, and luma BDPCM enabled for screen content sequences):

- Standard QPs (37, 32, 27, 22): AI, RA, and LD configurations as specified in CTC (all frames);

- Low QPs (17, 12, 7, 2): AI, RA, and LD configurations with 100 frames only (i.e., by using the encoder option “--FramesToBeEncoded=100”).

For all categories, additional results for screen content sequences (class TGM with YUV 4:2:0 format) are mandatory, using the same configuration for class F CTC (HashME and IBC enabled).

## Coding Performance Measurements

The measurement relies on the compression ratio relative to the uncompressed raw data. A bit-rate saving or overhead is then calculated by comparing the compression ratio of the test relative to the VTM-7.0 anchor.

For the lossy test conditions, the impact on bitrate/PSNR using provided data (4 points, BD-rate) is measured. Proponents are encouraged to provide supplementary results.

## Complexity Considerations

To provide an indicative assessment of complexity, the following numbers should be provided:

- Encoding time (compared to the reference configurations);

- Decoding time (compared to the reference configuration).

In addition, proponents are encouraged to provide the following (but not limited to) information for complexity and entropy throughput analysis:

- Number of context models

- Number of context-coded and bypass bins in average case and the worst case

- Table size of scanning paths

- Analysis of the frequency of context switches

- Analysis of the dependency between two consecutive bins

- Analysis of entropy parsing dependency (whether bits to bins conversion depends on complex reconstruction and/or pixel processing process)

- Analysis of entropy de-binarization dependency (whether bins to symbols conversion depends on complex reconstruction and/or pixel processing process)

# Test Plan

The core experiment is split into the following tests:

## Category 1: BDPCM and high-level approaches

|  | Tester | Tool | Cross checker |
| --- | --- | --- | --- |
| CE3-1.1 | A. Nalci (Qualcomm) | JVET-P0082: Enabling Luma BDPCM for lossless coding (using TSRC) |  |
| CE3-1.2 | A. Nalci (Qualcomm) Z.-Y. Lin (MediaTek) | JVET-P0082: Luma and Chroma BDPCM for lossless coding (using TSRC) |  |
| CE3-1.3 | A. Nalci (Qualcomm) M. Sarwer (Alibaba) H. Jang (LGE) | JVET-P0082/JVET-P0463/JVET-P0574: Luma BDPCM for lossless coding with RRC (no changes to RRC) |  |
| CE3-1.4 | A. Nalci (Qualcomm) Z.-Y. Lin (MediaTek) | JVET-P0082: Luma and Chroma BDPCM for lossless coding with RRC (no changes to RRC) |  |
| CE3-1.5 | T.-C. Ma (Kwai) Z.-Y. Lin (MediaTek) | Residual coding selection signaling for lossless VVC (JVET-P0525) |  |
| CE3-1.6 | Z.-Y. Lin (MediaTek) T.-C. Ma (Kwai) A. Nalci (Qualcomm) M. Sarwer (Alibaba) | JVET-P0148/JVET-P0258: A PPS flag to select either RRC (without state transition) or TSRC |  |
| CE3-1.7 | T. Tsukuba (Sony) T.-C. Ma (Kwai) Z.-Y. Lin (MediaTek) | JVET-P0584/JVET-P0525: A SPS flag to select transform coefficient coding and TS coding with luma BDPCM mode enabled |  |
| CE3-1.8 | T. Tsukuba (Sony) T.-C. Ma (Kwai) Z-Y. Lin (MediaTek) | JVET-P0584/JVET-P0525: A SPS flag to select transform coefficient coding and TS coding with both luma and chroma BDPCM mode enabled |  |
| CE3-1.9 | J. Choi (LGE) | JVET-P0466: BDPCM and IBC with TSRC for lossless coding |  |

## Category 2: RRC/TSRC modifications

|  | Tester | Tool | Cross checker |
| --- | --- | --- | --- |
| CE3-2.1 +CTC+LQP | H. Wang (Qualcomm) | JVET-P0559: Modified TSRC for lossless |  |
| CE3-2.2 +CTC+LQP | Y.-H. Chao (Qualcomm) | JVET-P1028: Modified TSRC for lossless |  |
| CE3-2.3 | T. -C. Ma (Kwai) | On residual scanning order for lossless coding (JVET-P0528) |  |
| CE3-2.4 | T.-C. Ma (Kwai) Z.-Y. Lin (MediaTek) A. Nalci (Qualcomm) M. Sarwer (Alibaba) | JVET-P0148/JVET-P0258: Using RRC for lossless without state transition |  |

## Category 3: Combined Tests with BDPCM and RRC/TSRC modifications

|  | Tester | Tool | Cross checker |
| --- | --- | --- | --- |
| CE3-3.1 | A. Nalci (Qualcomm) | JVET-P0082/JVET-P0463/JVET-P0574: Luma BDPCM for lossless coding with RRC (disable state transition in RRC) |  |
| CE3-3.2 | A. Nalci (Qualcomm) | JVET-P0082: Luma and Chroma BDPCM for lossless coding with RRC (disable state transition in RRC) |  |
| CE3-3.3 | H. Wang (Qualcomm) | Modified TSRC + BDPCM luma/chroma |  |
| CE3-3.4 | H. Wang (Qualcomm) | Modified TSRC* + BDPCM luma/chroma |  |
| CE3-3.5 | Y.-H. Chao (Qualcomm) | Modified TSRC** + BDPCM luma/chroma |  |

# Timeline

General timeline for core experiments (as agreed by JVET).

| T1 | 3 weeks after meeting | - Final CE description uploaded |
| --- | --- | --- |
| T2 | 2 weeks after VTM-7 release [Oct. X] | - Proponents completed initial integration of tools (in individual branch) - Cross checkers can start initial study of tools / software (Note: Proponents can continue to modify the software until T3) |
| T3 | 3 weeks before next JVET meeting or T2 + 1 week (whichever is later) [Dec. X] | - Proponents froze changes to software branches - Proponents tag software and announce on JVET reflector - The names of the cross checkers and the list of specific tests for each CE tool are announced on JVET reflector - Proponents provide full test results - Cross checkers start final cross checks |
| T4 | regular document deadline – 1 week [Jan. X] | - CE contribution documents including specification text and complete test results are uploaded to the JVET document repository |

Test results and cross check results are reported to CE coordinators (for inclusion into CE report).

# Detailed Test Description

TBD

## CEX-1.1 to CEX-1.5 JVET-P0082/JVET-P0147/JVET-P0574 AHG:18 BDPCM for lossless (Qualcomm, MediaTek)

In this test it is proposed to enable BDPCM mode for lossless coding for both natural sequences and screen content. We would specifically test benefits of (1) Luma BDPCM for transform skip residual coding (TSRC), (2) Luma and Chroma BDPCM for TSRC, (3) Luma BDCM for regular residual coding  (RRC) (4) Luma and Chroma BDPCM for RRC, (5) only RRC coding for lossless without BDPCM. An SPS level high-level flag would be defined to achieve RRC for BDPCM.

## CEX-1.6 and CEX-1.7: JVET-P0147 AHG18: BDPCM in lossless coding (MediaTek)

It is proposed to improve the block-wise differential pulse code modulation (BDPCM) mode in lossless coding. To further improve BDPCM in lossless coding, two methods are proposed. In Method 1 (CEX-1.6), BDPCM is applied to both luma and chroma, and BDPCM is enabled in all sequences. In Method 2 (CEX-1.7), unlike always applying transform skip (TS) coefficient coding to BDPCM blocks in VTM6.0, a flag is signaled in the picture parameter set (PPS) to indicate whether non-TS coefficient coding or TS coefficient coding is applied to BDPCM blocks.

## Disabling state transition for RRC and lossless (JVET-P0148/JVET-P0258, MediaTek/Qualcomm/Kwai/Alibaba)

DQ comprises two parts. The first part is using different quantizers in quantization, and the second part is Q-state dependent coefficient coding. When DQ is enabled, a Q-state transition is performed according to each coefficient’s parity bit. During the coefficient coding, both the context variable set for each coefficient’s significant flag and the Rice parameter derivation are dependent on the Q-state. After the sign flags in one coding group are decoded, the coefficient levels are converted to quantized coefficients. These quantized coefficients will be further dequantized in the inverse quantization stage. However, when a block is coded in lossless mode, the conversion from coefficient levels to quantized coefficients will make lossless coding impossible since the original residuals are changed by the conversion. Therefore, it is proposed to test disabling the DQ if RRC is applied for lossless VVC. Since the cu_transquant_bypass_mode is not used in CE and the CE mandate is using high-level syntax approach, one PPS flag is added to indicate whether the TS or the non-TS coefficient coding is used.

Two tests are performed. In Test-1, it always uses transformed coefficient coding without DQ state transition. In Test-2, one PPS flag is added to select the TS or the non-TS coefficient coding.

## Residual coding selection signaling for lossless VVC (JVET-P0525, Kwai/MediaTek)

During this (Geneva) meeting, since the current TSRC still performs significantly worse for camera content but better for screen content, it is proposed to test the performance of signaling 1-bit syntax to select which coding scheme (i.e., RRC or TSRC) is applied under lossless coding. Three sub-tests are defined in this CE test, they are:

- Sub-Test 1: CU level signaling to select RRC/TSRC

- Sub-Test 2: TB level signaling to select RRC/TSRC

- Sub-Test 3: Implicitly select RRC/TSRC by transform_skip_flag

## On residual scanning order for lossless coding (JVET-P0528, Kwai)

The residual samples from intra prediction located at the bottom-right area of the block tend to have larger amplitude than those located at the top-left area, because of the distance between the reconstructed pixels that are used for prediction and the predicted pixels. When regular residual coding is applied for lossless residual coding, rotating the residual blocks (i.e. flipping the blocks along both horizontal and vertical direction) would place the statistically large amplitude residue at the top left area of the TU, which is more suitable for regular coefficient coding. In this CE, it is proposed to test the rotation of residual blocks if RRC is applied for lossless residual.

## CEX-2.1 to CEX-2.3 Combination of JVET-P0258 with CEX1.3: Disabling dependent quantization state transitions for lossless for RRC (Qualcomm)

Purpose of CEX-2.1 is to test bit-rate savings for lossless when we dependent quantization related Q-state transitions is disabled for lossless coding (e.g. when RRC mode is used for lossless coding with a high-level SPS flag). Tests CEX-2.2 and CEX-2.3 furthermore test whether the gain is additive and improves the the RRC BDPCM results presented in CEX-1.4 and CEX-1.5.

## CEX-2.4 to CEX-2.12 JVET-P0559: Modified RRC and TSRC residual coding for lossless (Qualcomm)

The purpose of test CEX-2.4 is to test possible bit-rate savings for lossless coding when RRC or TSRC is used and the modified TSRC method in JVET-P0559 is selected as a candidate. Tests CEX-2.5 to CEX-2.10 test whether this gain is additive on top of the BDPCM tests (RRC based) detailed in CEX1.3, CE CEX1.4, CEX1.5, CEX2.1 CEX2.2, or CEX2.3. Likewise, in tests CEX-2.11 and CEX-2.12 we use the modified TSRC method of JVET-P0559 and use TSRC coding for lossless.

## CEX-2.13 to CEX-2.15 JVET-P1028 Modified TSRC residual coding for lossless (Qualcomm)

The purpose of test CEX-2.13 is to test possible bit-rate savings for lossless coding when TSRC is used and the modified TSRC method in JVET-P1028 is selected as a candidate. Tests CEX-2.14 to CEX-2.15 test whether this gain is additive on top of the BDPCM tests (TSRC based) detailed in CEX1.1, and CEX1.2.

## SPS flag to signal residual coding method (JVET-P0463, Alibaba)

In this test, an SPS level control of selecting residual coding method of TS and BDPCM mode will be tested. Following is the semantic of the sps_transform_residual_coding_flag .

- sps_transform_residual_coding_flag equal to 1 specifies that if the transform_skip_flag is equal to 1  to the current block at the location (x0,y0) , then the current  block at the location (x0, y0) uses transform residual coding.

- sps_transform_residual_coding_flag equal to 0 specifies that if the transform_skip_flag is equal to 1  to the current luma block at the location (x0,y0) , then the current  block at the location (x0, y0) uses transform-skip residual coding.

## BDPCM for Luma and Chroma in lossless coding (JVET-P0584/JVET-P0525, Sony/Kwai/MediaTek)

In this test, a high-level control (e.g. SPS flag) to indicate that TS coding and transform coefficient coding are applied to BDPCM and TS mode (other than BDPCM), respectively, for lossless coding will be tested. The test will be conducted for luma only and both luma & chorma BDPCM mode, respectively.

## BDPCM and IBC with TSRC for lossless coding (JVET-P0466, LGE)

VVC has two residual coding methods: regular residual coding (RRC) and transform skip residual coding (TSRC). In this CE, it is proposed to use TSRC for the BDPCM mode and the IBC mode, and other coding modes are coded by RRC for lossless coding.