| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0044 |
| --- | --- |

| Title: | AHG9/AHG12: Simplification of slice index with subpictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takahiro Nishi Virginie Drugeon | Tel: Email: | +49-6103-766240 nishi.takahiro@jp.panasonic.com virginie.drugeon@eu.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

It is suggested to simplify the indexing of slices by aligning the picture level slice index to the subpicture level slice index, thereby removing the need to map the subpicture level slice index to the picture level slice index.

# Introduction

In VVC draft 7, there are two indexes for slices: the picture level slice index and the sub-picture level slice index. The slice_address signalled in the slice header is the slice index within the sub-picture. Since the slice index within the whole picture may be completely different, an additional array is required to map the subpicture level slice index k in subpicture i to the picture level slice index: SliceSubpicToPicIdx[ i ][ k ].

Such a mapping is required for example in the partitioning case of Figure 1, whereby the array SliceSubpicToPicIdx is calculated as follows:

- SliceSubpicToPicIdx[0] = [0,1,2,3,4,6]

- SliceSubpicToPicIdx[1] = [5,7]

Figure 1: Picture divided in 12 tiles, 8 slices and 2 subpictures

However, the picture level slice index is only required for signalling purposes and has no influence on decoding order. Indeed, in the example of Figure 1, the slices 0, 1, 2, 3, 4 and 6 have to be sent before the slices 5 and 7 because of the following constraint related to subpictures: “For any two subpictures subpicA and subpicB, when the subpicture index of subpicA is less than that of subpicB, any coded slice NAL unit of subPicA shall precede any coded slice NAL unit of subPicB in decoding order.”

Therefore, the usefulness of having a signalling order such as the one from Figure 1 seems doubtful. For the purpose of clarity of the specification, it would be much better to remove the need for a picture level slice index, which could be easily derived from the subpicture level slice index. This would also ensure that the “picture level slice index” is following the decoding order of the slices. The partitioning from Figure 1 would still be allowed, but the signalling order would become the one from Figure 2.

Figure 2: Picture divided in 12 tiles, 8 slices and 2 subpictures with slices indexed according to their decoding order

# First possible implementation

It is proposed to remove the array SliceSubpicToPicIdx, which is currently only used to derive the variable picLevelSliceIdx in the slice header semantics. Instead the variable picLevelSliceIdx can be very easily derived as follows for each slice of subpicture SubPicIdx:

picLevelSliceIdx = slice_address 
for( j = 0; j < SubPicIdx; j++ )
	picLevelSliceIdx += NumSlicesInSubpic[ j ]

# Second possible implementation

Alternatively, the two-dimensional array SliceSubpicToPicIdx could be replaced by a one-dimensional array TopLeftSliceIdxInSubpic that specifies the picture level index of the first slice of each subpicture. The array TopLeftSliceIdxInSubpic can be derived as follows:

for( j = 0; j  <=  sps_num_subpics_minus1; j++ )
	NumSlicesInSubpic[ j ] = 0
for( i = 0; i  <=  num_slices_in_pic_minus1; i++ ) {
	posX = CtbAddrInSlice[ i ][ 0 ] % PicWidthInCtbsY * CtbSizeY 
	posY = CtbAddrInSlice[ i ][ 0 ] / PicWidthInCtbsY * CtbSizeY 
	for( j = 0; j  <=  sps_num_subpics_minus1; j++ ) {
		if( ( posX  >=  subpic_ctu_top_left_x[ j ] * CtbSizeY )  &&	
				( posX < ( subpic_ctu_top_left_x[ j ] + subpic_width_minus1[ j ] + 1 ) * CtbSizeY )  &&
				( posY  >=  subpic_ctu_top_left_y[ j ] * CtbSizeY )  &&
				( posY < ( subpic_ctu_top_left_y[ j ] + subpic_height_minus1[ j ] + 1 ) * CtbSizeY ) ) {
			if( NumSlicesInSubpic[ j ]  = =  0 )
				TopLeftSliceIdxInSubpic[ j ] = i 
			NumSlicesInSubpic[ j ]++
		}
	}
}

The variable picLevelSliceIdx can then be very easily derived as follows for each slice of subpicture SubPicIdx:

if( rect_slice_flag ) {
	picLevelSliceIdx = TopLeftSliceIdxInSubpic[ SubPicIdx ] + slice_address
	NumCtuInCurrSlice = NumCtuInSlice[ picLevelSliceIdx ]
	for( i = 0; i < NumCtuInCurrSlice; i++ )
		CtbAddrInCurrSlice[ i ] = CtbAddrInSlice[ picLevelSliceIdx ][ i ]	
}

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).