| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0070-v2 |
| --- | --- |

| Title: | CE3-2.5: Residual coding selection signaling for lossless coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tsung-Chuan Ma, Hong-Jheng Jhu, Xiaoyu Xiu, Yi-Wen Chen, Xianglin Wang 8910 University Center Dr., Suite 400, San Diego, CA 92122, USA Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan Alican Nalci, Hongtao Wang, Muhammed Coban, Marta Karczewicz 5775 Morehouse Dr., San Diego, CA, 92121, USA | Email: | {tsung-chuanma, yiwenchen, xiaoyuxiu, jhuhong-jheng, xianglinwang}@kwai.com {zhi-yi.lin, peter.chuang, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com analci@qti.qualcomm.com |
| Source: | Kwai Inc., MediaTek Inc., Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the results of Core Experiment (CE) 3-2.5 [1]. In this CE test, three subtests are proposed to select regular residual coding (RRC) or transform-skip residual coding (TSRC) at low-level since the cu_transquant_bypass_mode is not used in CE. In the first test, CU level flag is signaled to indicate which residual coding scheme is selected signaling is performed at the beginning of a coding unit. In the second test, TB level is signaled to indicate which residual coding scheme is selected. In the third test, an alternative CU level flag is signaled in cu_residual function instead of the beginning of  coding unit after signaling root CBF.

Lossless results are reported as below:

Test 1: CU level signal for lossless VVC (signal in coding unit)

CTC: -5.22% AI. -5.58% RA, -4.96% LDB,

Class F: -2.43% AI, -3.49% RA, -3.40% LDB

Class TGM: -2.93% AI, -3.31% RA, -3.22% LDB

EncT: 96% AI, 99% RA, 99% LDB

DecT: 104% AI, 98% RA, 105% LDB

Test 2: TB level signal for lossless VVC

CTC: -5.21% AI, -5.57% RA, -4.95% LDB

Class F: -2.43% AI, -3.45% RA, -3.20% LDB

Class TGM: -3.26% AI, -3.37% RA, -3.43% LDB

EncT: 95% AI, 99% RA, 101% LDB

DecT: 104% AI, 96% RA, 105% LDB

Test 3: CU level signal for lossless VVC (signal in cu_residual)

CTC: -5.22% AI. -5.58% RA, -4.96% LDB,

Class F: -2.49% AI, -3.64% RA, -3.50% LDB

Class TGM: -3.35% AI, -3.79% RA, -3.89% LDB

EncT: 94% AI, 100% RA, 98% LDB

DecT: 104% AI, 100% RA, 107% LDB

# Introduction

During this (Geneva) meeting, since the current TSRC still performs significantly worse for camera content but better for screen content, it is proposed to test the performance of signaling 1-bit syntax to select which coding scheme (i.e., RRC or TSRC) is applied under lossless coding. Two sub-tests are defined in this CE test, they are:

- CU level signaling to select RRC/TSRC (2 variants presented herein)

- TB level signaling to select RRC/TSRC

# Implementation

Two high-level syntax, one PPS and one slice level, are signaled to indicate the selection of TSRC/RRC is enabled. In sub-test 1, a CU level syntax is signaled if PPS is enabled; otherwise, such CU level syntax is inferred to 0. In sub-test 2, a TB level syntax is signaled if PPS is enabled; otherwise, such TB level syntax is inferred to 0. In sub-test 3, signaling of the CU level syntax is done after signaling the root CBF for a CU.

# Simulation Results

In subtest1, it is implemented based on the VTM-7.0 reference software [2] and tested using test condition and evaluation criteria that are used in CE3 [1] are applied.

## CU level signaling for lossless VVC (signaling in coding unit)

Table 1: Results of Subtest1 for lossless VVC

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM7.0 | TEST |  | VTM7.0 | TEST |  | VTM7.0 | TEST |  |
| Class A1 | 2.2 | 2.4 | -5.41% | 2.2 | 2.4 | -6.24% |  |  |  |
| Class A2 | 1.6 | 1.7 | -8.85% | 1.7 | 4.1 | -8.12% |  |  |  |
| Class B | 2.2 | 2.3 | -3.99% | 2.3 | 2.4 | -3.83% | 2.3 | 2.4 | -3.85% |
| Class C | 1.9 | 2.0 | -4.95% | 2.4 | 2.6 | -5.35% | 2.4 | 2.6 | -5.28% |
| Class D | 1.9 | 2.0 | -4.61% | 2.8 | 2.9 | -5.24% | 2.7 | 2.9 | -5.22% |
| Class E | 2.8 | 2.9 | -3.80% |  |  |  | 3.1 | 3.3 | -6.41% |
| Class F | 5.3 | 5.4 | -2.43% | 33.7 | 34.6 | -3.49% | 50.7 | 51.7 | -3.40% |
| TGM | 11.8 | 12.2 | -2.93% | 107.1 | 110.8 | -3.31% | 124.9 | 129.0 | -3.22% |
| Overall | 2.1 | 2.2 | -5.22% | 2.2 | 2.8 | -5.58% | 2.6 | 2.7 | -4.96% |
| Enc Time[%] | 96% |  |  | 99% |  |  | 99% |  |  |
| Dec Time[%] | 104% |  |  | 98% |  |  | 105% |  |  |

In subtest2, it is implemented based on the VTM-7.0 reference software [2] and tested using test condition and evaluation criteria that are used in CE3 [1] are applied.

## TB level signaling for lossless VVC

Table 2: Results of Subtest2 for lossless VVC

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM7.0 | TEST |  | VTM7.0 | TEST |  | VTM7.0 | TEST |  |
| Class A1 | 2.2 | 2.4 | -5.40% | 2.2 | 2.4 | -6.24% |  |  |  |
| Class A2 | 1.6 | 1.7 | -8.85% | 1.7 | 4.1 | -8.12% |  |  |  |
| Class B | 2.2 | 2.3 | -3.99% | 2.3 | 2.4 | -3.83% | 2.3 | 2.4 | -3.84% |
| Class C | 1.9 | 2.0 | -4.94% | 2.4 | 2.6 | -5.34% | 2.4 | 2.6 | -5.27% |
| Class D | 1.9 | 2.0 | -4.59% | 2.8 | 2.9 | -5.20% | 2.7 | 2.9 | -5.20% |
| Class E | 2.8 | 2.9 | -3.79% |  |  |  | 3.1 | 3.3 | -6.39% |
| Class F | 5.3 | 5.4 | -2.43% | 33.7 | 34.6 | -3.45% | 50.7 | 51.5 | -3.20% |
| TGM | 11.8 | 12.2 | -3.26% | 107.1 | 111.1 | -3.37% | 124.9 | 129.9 | -3.43% |
| Overall | 2.1 | 2.2 | -5.21% | 2.2 | 2.8 | -5.57% | 2.6 | 2.7 | -4.95% |
| Enc Time[%] | 95% |  |  | 99% |  |  | 101% |  |  |
| Dec Time[%] | 104% |  |  | 96% |  |  | 105% |  |  |

## CU level signaling for lossless VVC (signaling in cu_residual)

Table 3: Results of Subtest3 for lossless VVC

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM7.0 | TEST |  | VTM7.0 | TEST |  | VTM7.0 | TEST |  |
| Class A1 | 2.2 | 2.4 | -5.41% | 2.2 | 2.4 | -6.24% |  |  |  |
| Class A2 | 1.6 | 1.7 | -8.85% | 1.7 | 1.9 | -8.12% |  |  |  |
| Class B | 2.2 | 2.3 | -3.99% | 2.3 | 2.4 | -3.83% | 2.3 | 2.4 | -3.85% |
| Class C | 1.9 | 2.0 | -4.95% | 2.4 | 2.6 | -5.35% | 2.4 | 2.6 | -5.28% |
| Class D | 1.9 | 2.0 | -4.61% | 2.8 | 2.9 | -5.24% | 2.7 | 2.9 | -5.22% |
| Class E | 2.8 | 2.9 | -3.80% |  |  |  | 3.1 | 3.3 | -6.41% |
| Class F | 5.3 | 5.4 | -2.49% | 33.7 | 34.7 | -3.64% | 50.7 | 51.8 | -3.50% |
| TGM | 11.8 | 12.2 | -3.35% | 107.1 | 111.4 | -3.79% | 124.9 | 130.3 | -3.89% |
| Overall | 2.1 | 2.2 | -5.22% | 2.2 | 2.3 | -5.58% | 2.6 | 2.7 | -4.96% |
| Enc Time[%] | 94% |  |  | 100% |  |  | 98% |  |  |
| Dec Time[%] | 104% |  |  | 100% |  |  | 107% |  |  |

# References

- T. Nguyen T. C. Ma A. Nalci, “Description of Core Experiment 3 (CE3): Lossless Coding”, JVET-P2023, 16th meeting, Geneva, CH, 1–11 October 2019

- VTM-7.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0.

- B. Bross, J. Chen, S. Liu and Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-P2001-vE, the 16th JVET meeting, Geneva, CH, 1–11 October 2019

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).