| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0071 |
| --- | --- |

| Title: | CE3-2.10: Luma BDPCM for lossless coding with RRC and residual rotation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tsung-Chuan Ma, Hong-Jheng Jhu, Xiaoyu Xiu, Yi-Wen Chen, Xianglin Wang 8910 University Center Dr., Suite 400, San Diego, CA 92122, USA Alican Nalci, Hongtao Wang, Hilmi E. Egilmez Muhammed Coban, Marta Karczewicz 5775 Morehouse Dr., San Diego, CA, 92121, USA | Email: | {tsung-chuanma, yiwenchen, xiaoyuxiu, jhuhong-jheng, xianglinwang}@kwai.com analci@qti.qualcomm.com |
| Source: | Kwai Inc., Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the results of Core Experiment (CE) 3-2.10 [1]. In this CE test, it is proposed to test the combination of CE3-1.4 and CE3-2.3 [1].

Lossless results are reported as below:

CTC: -7.01% AI. -6.06% RA, -5.63% LDB

Class F: -5.52% AI, -4.04% RA, -3.76% LDB

Class TGM: -2.21% AI, -0.96% RA, -0.81% LDB

EncT (all class): 107% AI, 102% RA, 103% LDB

DecT (all class): 97% AI, 99% RA, 108% LDB

Additional result is provided. For this additional test, the anchor is CE3-2.3 and the test is CE 3-2.10.

Lossless results are reported as below:

CTC: -0.26% AI. -0.11% RA, -0.09% LDB,

Class F: 0.23% AI, 0.35% RA, -0.03% LDB

Class TGM: -0.61 % AI, -0.24% RA, -0.95 % LDB

EncT (all class): 104% AI, 102% RA, 104% LDB

DecT (all class): 105% AI, 103% RA, 105% LDB

# Introduction

In current VVC [3] for lossless coding, it is forced to apply transform-skip residual coding while it is not optimal for lossless coding. In CE3-2.3, it is proposed to apply regular residual coding and enable BDPCM for luma for both natural and screen content. In CE3-1.4, it is proposed to apply residual rotation since it helps the residual coding performance. In this CE test, both CE3-1.4 and CE3-2.3 are applied together.

# Simulation Results

In this test, it is implemented based on the VTM-7.0 reference software [2] and tested using test condition and evaluation criteria that are used in CE3 [1] are applied.

Table 1: Results of test for VVC lossless

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM7.0 | TEST |  | VTM7.0 | TEST |  | VTM7.0 | TEST |  |
| Class A1 | 2.2 | 2.4 | -5.98% | 2.2 | 2.4 | -6.65% |  |  |  |
| Class A2 | 1.6 | 1.8 | -10.03% | 1.7 | 1.9 | -8.43% |  |  |  |
| Class B | 2.2 | 2.3 | -5.75% | 2.3 | 2.5 | -4.37% | 2.3 | 2.4 | -4.40% |
| Class C | 1.9 | 2.1 | -6.86% | 2.4 | 2.6 | -5.95% | 2.4 | 2.6 | -5.89% |
| Class D | 1.9 | 2.1 | -7.85% | 2.8 | 2.9 | -5.86% | 2.7 | 2.9 | -5.69% |
| Class E | 2.8 | 3.0 | -7.33% |  |  |  | 3.1 | 3.4 | -7.33% |
| Class F | 5.3 | 5.7 | -6.28% | 33.7 | 35.0 | -4.56% | 50.7 | 51.3 | -3.86% |
| TGM | 11.8 | 12.0 | -1.78% | 107.1 | 107.7 | -0.73% | 124.9 | 123.8 | 0.48% |
| Overall | 2.1 | 2.3 | -7.01% | 2.2 | 2.4 | -6.06% | 2.6 | 2.7 | -5.63% |
| Enc Time[%] | 107% |  |  | 102% |  |  | 103% |  |  |
| Dec Time[%] | 97% |  |  | 99% |  |  | 108% |  |  |

In this additional test, it is implemented based on the VTM-7.0 reference software [2] and tested using test condition and evaluation criteria that are used in CE3 [1] are applied. The anchor is CE3-2.3. The test is CE3-2.10.

Table 2: Results of test for VVC lossless with CE3-2.3 as anchor

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | CE3-2.3 | TEST |  | CE3-2.3 | TEST |  | CE3-2.3 | TEST |  |
| Class A1 | 2.4 | 2.4 | -0.18% | 2.4 | 2.4 | -0.14% |  |  |  |
| Class A2 | 1.8 | 1.8 | -0.05% | 1.9 | 1.9 | -0.03% |  |  |  |
| Class B | 2.3 | 2.3 | -0.18% | 2.5 | 2.5 | -0.08% | 2.4 | 2.4 | -0.07% |
| Class C | 2.1 | 2.1 | -0.47% | 2.6 | 2.6 | -0.18% | 2.6 | 2.6 | -0.15% |
| Class D | 2.1 | 2.1 | -0.37% | 2.9 | 2.9 | -0.09% | 2.9 | 2.9 | -0.06% |
| Class E | 3.0 | 3.0 | -0.43% |  |  |  | 3.4 | 3.4 | -0.04% |
| Class F | 5.7 | 5.6 | 0.23% | 34.9 | 34.6 | 0.35% | 51.2 | 51.7 | -0.03% |
| TGM | 12.0 | 12.1 | -0.61% | 107.6 | 108.1 | -0.24% | 124.0 | 126.2 | -0.95% |
| Overall | 2.3 | 2.3 | -0.26% | 2.4 | 2.4 | -0.11% | 2.7 | 2.7 | -0.09% |
| Enc Time[%] | 104% |  |  | 102% |  |  | 104% |  |  |
| Dec Time[%] | 105% |  |  | 103% |  |  | 105% |  |  |

# Modification for Working Draft

For spec change, please refer to CE3-1.4 and CE3-2.3.

# References

- T. Nguyen T. C. Ma A. Nalci, “Description of Core Experiment 3 (CE3): Lossless Coding”, JVET-P2023, 16th meeting, Geneva, CH, 1–11 October 2019

- VTM-7.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0.

- B. Bross, J. Chen, S. Liu and Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-P2001-vE, the 16th JVET meeting, Geneva, CH, 1–11 October 2019

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).