| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0074 |
| --- | --- |

| Title: | CE5-related: Performance of CE5 anchor with 8-bit dynamic range for CC-ALF coefficients |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Nan Hu, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA, 92121, USA | Tel: Email: | +1(858)651-3483 nanh@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In JVET-P1008, a cross component adaptive loop filter (CC-ALF) is tested to refine chroma components by using luma samples. The shape of a filter is 3x4. Each filter has 8 filter coefficients and 8 taps. Every coefficient has a 6-bit dynamic range and is signalled with a 6-bit code. This is also used in CE5 anchor. In this contribution, the performance of 8-bit dynamic range for filter coefficients is tested under common test conditions.

The average BD rate for the tests versus CE5 anchor is as follows:

| Results over CE5 anchor (SDR) | All Intra |  |  | Random Access |  |  | Low Delay B |  |  | Low Delay P |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| 8-bit dynamic range | 0.02% | -0.08% | 0.12% | 0.00% | 0.05% | 0.13% | -0.02% | 0.31% | 0.70% | 0.01% | 0.33% | 0.56% |

Compared with CE5 anchor, similar encoding and decoding time is observed.

# Experimental results

Experiments are performed on top of CE5 anchor by changing the macro CCALF_COEFF_BITDEPTH from 6 to 8.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0-CE5Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.02% | -0.12% | 100% | 100% |
| Class A2 | 0.01% | -0.63% | -0.41% | 99% | 100% |
| Class B | 0.02% | 0.00% | 0.09% | 100% | 100% |
| Class C | 0.03% | 0.14% | 0.22% | 101% | 100% |
| Class E | 0.01% | 0.01% | 0.79% | 100% | 100% |
| Overall | 0.02% | -0.08% | 0.12% | 100% | 100% |
| Class D | 0.00% | 0.45% | 0.15% | 99% | 100% |
| Class F | 0.03% | 0.05% | 0.09% | 100% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0-CE5Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.12% | -0.01% | 100% | 100% |
| Class A2 | 0.02% | -0.29% | -0.10% | 100% | 100% |
| Class B | 0.00% | 0.20% | 0.15% | 100% | 101% |
| Class C | -0.01% | 0.26% | 0.40% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.05% | 0.13% | 100% | 101% |
| Class D | -0.01% | 0.34% | 0.32% | 100% | 100% |
| Class F | 0.02% | 0.12% | 0.20% | 100% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0-CE5Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.07% | 0.37% | 100% | 101% |
| Class C | 0.01% | 0.34% | 0.37% | 100% | 99% |
| Class E | -0.06% | 0.92% | 1.70% | 100% | 99% |
| Overall | -0.02% | 0.31% | 0.70% | 100% | 100% |
| Class D | -0.03% | 0.92% | 0.03% | 100% | 100% |
| Class F | -0.02% | 0.48% | 0.61% | 101% | 100% |
|  |  |  | Low delay P Main10 |  |  |
|  |  |  | Over VTM-7.0-CE5Anchor |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.17% | 0.35% | 100% | 97% |
| Class C | 0.00% | 0.24% | 0.50% | 101% | 100% |
| Class E | 0.00% | 0.71% | 0.98% | 100% | 101% |
| Overall | 0.01% | 0.33% | 0.56% | 100% | 99% |
| Class D | 0.02% | -0.12% | -0.07% | 101% | 101% |
| Class F | -0.05% | 0.12% | 0.35% | 101% | 103% |