| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0080-v3 |
| --- | --- |

| Title: | CE3-1.5: Rice parameter derivation of transform skip residual coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer, Ruling Liao, Yan Ye, Jiancong Luo 525 Almanor Ave, Sunnyvale, CA | Tel: Email: | +1 (408) 858-6780 m.sarwer@alibaba-inc.com |
| Source: | Alibaba Global |  |  |

_____________________________

# Abstract

This contribution reports the simulation results of CE3-1.5. In CE3-1.5, the rice parameter of abs_remainder of transform skip residuals is derived from the top and left neighbors. Following results are reported as compared to VTM-7.0.

- Lossless test conditions:

- Overall: -2.52% (AI), -0.82% (RA), -0.91% (LB) bit rate saving

- Class TGM: -3.40% (AI), -5.36% (RA), -5.67% (LB) bit rate saving

- CTC test conditions:

- Overall: -0.01% (AI), 0.00% (RA), -0.03% (LB) luma BD-rate

- Class TGM: -0.37% (AI), -0.17% (RA), -0.25% (LB) luma BD-rate

- Low QP test conditions:

- Overall: -0.06% (AI), -0.02% (RA), -0.02% (LB) luma BD-rate

- Class TGM: -1.35% (AI), -1.85% (RA), -1.20% (LB) luma BD-rate

# Introduction

In VVC 7, the transform skip (TS) residual levels of a coefficient group (CG) are coded in three passes over the scan positions as follows:

- Pass 1: context coding

  - sig_coeff_flag

  - coeff_sign_flag

  - greater 1 flag (abs_level_gtx_flag[0])

  - parity (par_level_flag) flag

- Pass 2: context coding

  - greater than 3 flag (abs_level_gtx_flag[1])

  - greater than 5 flag (abs_level_gtx_flag[2])

  - greater than 7 flag (abs_level_gtx_flag[3])

  - greater than 9 flag (abs_level_gtx_flag[4])

- Pass 3: by-pass coding: Rice-Golomb coding of remaining absolute level (abs_remainder)

The syntax element abs_remainder is binarized with a fixed rice parameter ( cRiceParam = 1) value [1].

# Proposed method

In this CE test, the rice parameter is derived from the sum of absolute values of the top and left neighboring levels. Assume, the variable locSumAbs is the sum of absolute values of top and left levels. The rice parameter is derived as follows:

- locSumAbs = min(locSumAbs, 31)

- the rice parameter (cRiceParam) is derived from the look-up table. The look-up table used in this CE test is same as the look-up table used in regular residual coding and tabulated in table 1.

Table 1: look-up table to select the Rice parameter

| locSumAbs | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cRiceParam | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 |
| locSumAbs | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| cRiceParam | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 |

# Simulation results

The proposed method is implemented on top of VTM-7.0. All of the tests are conducted under test conditions described in CE3 [2]. Table 2 shows the results of CE3-1.5 under lossless test conditions where BDPCM mode is disabled for all sequences. The results of CE3-1.5 under CTC and low QP test conditions are tabulated in Table 3 and Table 4, respectively.

Table 2: results of CE3-1.5 under lossless test conditions

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM-7.0 | CE3-1.5 |  | VTM-7.0 | CE3-1.5 |  | VTM-7.0 | CE3-1.5 |  |
| Class A1 | 2.2 | 2.3 | -1.27% | 2.2 | 2.3 | -1.14% |  |  |  |
| Class A2 | 1.6 | 1.7 | -4.41% | 1.7 | 1.7 | -2.34% |  |  |  |
| Class B | 2.2 | 2.2 | -1.25% | 2.3 | 2.3 | 0.74% | 2.3 | 2.3 | 0.81% |
| Class C | 1.9 | 2.0 | -2.81% | 2.4 | 2.5 | -1.37% | 2.4 | 2.5 | -1.22% |
| Class D | 1.9 | 2.0 | -3.12% | 2.8 | 2.8 | -1.55% | 2.7 | 2.8 | -1.41% |
| Class E | 2.8 | 2.9 | -3.63% |  |  |  | 3.1 | 3.2 | -3.37% |
| Class F | 5.3 | 5.5 | -2.67% | 33.7 | 34.5 | -2.38% | 50.7 | 51.8 | -2.28% |
| TGM | 11.8 | 12.3 | -3.40% | 107.1 | 114.8 | -5.36% | 124.9 | 135.0 | -5.67% |
| Overall | 2.1 | 2.2 | -2.52% | 2.2 | 2.2 | -0.82% | 2.6 | 2.6 | -0.91% |
| Enc Time[%] | 98% |  |  | 99% |  |  | 101% |  |  |
| Dec Time[%] | 104% |  |  | 107% |  |  | 108% |  |  |

Table 3: results of CE3-1.5 under CTC test conditions

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | 0.07% | 95% | 98% |
| Class A2 | 0.00% | -0.02% | -0.02% | 96% | 98% |
| Class B | 0.00% | 0.06% | -0.06% | 96% | 97% |
| Class C | -0.03% | -0.01% | 0.00% | 96% | 98% |
| Class E | 0.00% | -0.03% | -0.05% | 96% | 97% |
| Overall | -0.01% | 0.00% | -0.02% | 96% | 97% |
| Class D | -0.05% | 0.01% | 0.00% | 96% | 100% |
| Class F | -0.15% | -0.19% | 0.02% | 101% | 102% |
| Class SCC | -0.37% | -0.27% | -0.30% | 101% | 102% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | 0.01% | 97% | 98% |
| Class A2 | -0.05% | -0.02% | 0.07% | 97% | 98% |
| Class B | 0.02% | 0.07% | 0.02% | 98% | 97% |
| Class C | -0.01% | -0.09% | -0.01% | 98% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | 0.02% | 98% | 98% |
| Class D | -0.04% | 0.06% | 0.26% | 98% | 98% |
| Class F | -0.08% | 0.00% | -0.04% | 101% | 102% |
| Class SCC | -0.17% | -0.20% | -0.17% | 101% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.20% | 0.04% | 97% | 99% |
| Class C | -0.02% | -0.10% | -0.28% | 98% | 94% |
| Class E | -0.07% | 0.35% | -0.17% | 96% | 94% |
| Overall | -0.03% | -0.03% | -0.12% | 97% | 96% |
| Class D | 0.02% | 0.10% | -0.92% | 97% | 95% |
| Class F | -0.07% | -0.28% | 0.05% | 102% | 103% |
| Class SCC | -0.25% | -0.24% | -0.30% | 102% | 104% |

Table 4: results of CE3-1.5 under low QP test conditions

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.04% | -0.04% | 100% | 101% |
| Class A2 | 0.00% | -0.01% | -0.01% | 100% | 101% |
| Class B | -0.02% | -0.04% | -0.09% | 99% | 99% |
| Class C | -0.07% | -0.12% | -0.10% | 100% | 98% |
| Class E | -0.17% | -0.30% | -0.38% | 99% | 99% |
| Overall | -0.06% | -0.10% | -0.12% | 99% | 99% |
| Class D | -0.05% | -0.12% | -0.10% | 99% | 94% |
| Class F | -1.18% | -0.77% | -0.73% | 100% | 100% |
| Class SCC | -1.35% | -1.22% | -1.20% | 101% | 97% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.04% | -0.04% | 99% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class B | 0.00% | -0.03% | -0.04% | 100% | 100% |
| Class C | -0.03% | -0.04% | -0.06% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.03% | -0.04% | 99% | 100% |
| Class D | -0.06% | -0.04% | -0.07% | 99% | 95% |
| Class F | -1.52% | -0.70% | -0.80% | 100% | 98% |
| Class SCC | -1.85% | -1.01% | -1.03% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.02% | 0.00% | 100% | 99% |
| Class C | -0.01% | -0.03% | 0.00% | 100% | 102% |
| Class E | -0.05% | -0.01% | -0.06% | 100% | 103% |
| Overall | -0.02% | -0.02% | -0.02% | 100% | 101% |
| Class D | -0.06% | -0.01% | -0.12% | 99% | 96% |
| Class F | -0.53% | -0.46% | -0.65% | 100% | 101% |
| Class SCC | -1.20% | -0.95% | -0.94% | 101% | 101% |

# Conclusion

In the proposed method, rice parameter of abs_remainder of transform skip residual coding is derived from the top and left neighboring levels. In lossless test condition, in overall, the proposed method can achieve -2.52%(AI), -0.82% (RA) and -0.91% (LB) coding gain.

# References

Y.-W. Chen, X. Xiu, T.-C. Ma, H.-J. Jhu, X. Wang, “ Non-CE8: Transform skip residual coding simplification”, JVET-P0562, Geneva, Oct 2019

T. Nguyen, T. C. Ma, A. Nalci, “ Description of Core Experiment 3 (CE3): Lossless Coding”, JVET-P2023

# Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).