| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0095 |
| --- | --- |

| Title: | CE5-2.2: Multiplication removal for CCALF with coefficient range in [-8, 8] |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guichun Li, XiangLi, Xin Zhao, Liang Zhao, Yixin Du, Shan Liu Nan Hu, J. Dong, V. Seregin, M. Karczewicz | Tel: Email: | +1-650-798-3300 guichunli@tencent.com nanh@qti.qualcomm.com |
| Source: | Tencent & Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This is a report of the core experiment of CE5-2.2, removing multiplications for CCALF and limit the coefficient values to {-8, -4, -2, -1, 0, 1, 2, 4, 8}. The proposed methods were implemented on top of CE5 anchor software, and simulations were conducted with common test conditions. It is reported that the proposed method has overall performance as following.

- Over VTM-7, BD rate changes are 0.11%, -7.27%, -5.83% for AI, 0.17%, -10.38%, -9.86% for RA, 0.14%，-15.63%，-11.80% for LDB.

- Over CE5 Anchor, BD rate changes are -0.02%, 0.35%, 0.21% for AI, 0.01%, 0.36%, 0.23% for RA, -0.03%， 0.04%， -0.05% for LDB.

# Introduction

The CCALF was previously proposed in JVET-P1008. And it was implemented on top of VTM-7.0 to be used as CE5 anchor.

This method provides a CC-ALF design that has the following properties:

- Filter shape is a 3x4 diamond with 8 unique coefficients

- Filter coefficient dynamic range is limited to 6-bit signed

- Filter selection to performed at the CTU level with support for a maximum of 4 filters

- Symmetric line selection is used at virtual boundary (to better harmonize with ALF)

- Temporal layer coefficient buffers are not used (to better harmonize with the APS concept)

- Residual output is clipped to -2BitDepthC-1 to 2BitDepthC-1-1, inclusive.

An overview of the CC-ALF process is shown in Figure C-1.  As can be seen in the Figure, CC-ALF uses a linear filter to filter luma sample values and generate a residual correction for the chroma channels from the co-located filtered output.  An illustration of the filter shape is provided in Figure C-2.

Fig. C-1: Placement of CC-ALF with respect to other loop filters

Fig. C-2: Filter support in CE Anchor

It was previously proposed in JVET-P0666 to remove multiplication for CCALF, filter coefficients are restricted to have values: {-8, -4, -2, -1, 0, 1, 2, 4, 8}. A bit is first signalled to indicate whether it is a zero coefficient. If not, a fixed length code of 2 bits is used to signal the absolute value of the coefficient followed by a sign bit.

# Experimental Results

The proposed method was implemented based on CE5 anchor software. Simulations were conducted following JVET common test conditions. VTM-7.0 was used as anchor, and CE5 anchor software is used as a second anchor.

Bit streams for subjective tests were also provided and uploaded to the FTP server of CE5.

## Results of CE5-2.2

As shown in Table 3.1, it was observed that compared with VTM-7.0 anchor, the BD-rate change of CE5-2.2 method is 0.14%, -7.64%, -6.53% for AI with 100% Enc. time, 104% Dec. time, 0.14%, -9.85%, -9.40% for RA with 101% Enc. time, 103% Dec. time, 0.14%，-13.08%，-9.57% for LB with 100% Enc. time, 105% Dec. time.

Compared with CE5 anchor, the BD-rate change of CE5-2.2 is -0.03%, 0.74%, 0.60% for AI with 100% Enc. time, 100% Dec. time, 0.00%, 0.35%, 0.45% for RA with 100% Enc. time, 100% Dec time, 0.02%，-0.08%，-0.10% for LB, with 100% Enc. time, 99% Dec. time.

Table 3.1 Simulation Results of CE5-2.2

|  | AI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  | Over CE5 Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.15% | -4.40% | -4.56% | 100% | 103% | -0.02% | 0.59% | 0.80% | 100% | 100% |
| Class A2 | 0.10% | -6.80% | -10.60% | 101% | 106% | -0.01% | 1.23% | 1.89% | 100% | 101% |
| Class B | 0.13% | -12.36% | -10.00% | 100% | 104% | -0.02% | 1.05% | 0.64% | 100% | 100% |
| Class C | 0.14% | -3.86% | -3.63% | 100% | 106% | -0.05% | 0.06% | -0.04% | 100% | 102% |
| Class E | 0.19% | -8.88% | -2.54% | 100% | 103% | -0.06% | 0.81% | -0.10% | 100% | 99% |
| Overall | 0.14% | -7.64% | -6.53% | 100% | 104% | -0.03% | 0.74% | 0.60% | 100% | 100% |
| Class D | 0.09% | -3.55% | -2.24% | 100% | 103% | -0.01% | -0.16% | -0.54% | 100% | 101% |
| Class F | 0.11% | -5.60% | -5.72% | 100% | 105% | -0.05% | 0.29% | 0.34% | 100% | 101% |
|  | RA |  |  |  |  |  |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  | Over CE5 Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | -5.50% | -5.02% | 101% | 102% | -0.01% | 0.23% | 1.13% | 100% | 99% |
| Class A2 | 0.14% | -9.72% | -14.60% | 101% | 105% | 0.03% | 1.40% | 1.60% | 100% | 100% |
| Class B | 0.19% | -17.07% | -13.53% | 100% | 103% | 0.02% | 0.31% | -0.05% | 100% | 100% |
| Class C | 0.11% | -4.19% | -3.62% | 100% | 103% | -0.02% | -0.31% | -0.30% | 101% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.14% | -9.85% | -9.40% | 101% | 103% | 0.00% | 0.35% | 0.45% | 100% | 100% |
| Class D | 0.08% | -4.33% | -3.35% | 100% | 104% | -0.02% | -0.50% | -0.48% | 100% | 101% |
| Class F | 0.13% | -3.84% | -3.91% | 100% | 103% | -0.03% | 0.09% | -0.07% | 101% | 99% |
|  | LB |  |  |  |  |  |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  | Over CE5 Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.16% | -19.00% | -16.84% | 100% | 105% | 0.00% | 0.31% | 0.49% | 100% | 100% |
| Class C | 0.14% | -5.41% | -4.21% | 101% | 106% | 0.05% | -0.14% | -0.43% | 101% | 99% |
| Class E | 0.10% | -13.43% | -4.61% | 100% | 103% | 0.00% | -0.65% | -0.64% | 100% | 99% |
| Overall | 0.14% | -13.08% | -9.57% | 100% | 105% | 0.02% | -0.08% | -0.10% | 100% | 99% |
| Class D | 0.09% | -5.74% | -3.95% | 100% | 103% | 0.01% | -1.26% | -0.92% | 100% | 100% |
| Class F | 0.13% | -7.77% | -6.13% | 100% | 106% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Conclusion

It was observed that compared with CE5 anchor, the BD-rate change of CE5-2.2 is -0.03%, 0.74%, 0.60% for AI with 100% Enc. time, 100% Dec. time, 0.00%, 0.35%, 0.45% for RA with 100% Enc. time, 100% Dec time, 0.02%，-0.08%，-0.10% for LB, with 100% Enc. time, 99% Dec. time.

It is asserted that all the multiplications of the filter coefficient are replaced by shifting, which reduced the complexity of CCALF filter.

It is proposed to adopt the proposed methods as part of CCALF design.

# Reference

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-N1010 Marrakech, MA, Mar 2019.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-6.0

- X. Li, N. Hu, et. Al, “Non-CE5: On deblocking on affine internal prediction sub-block edges”, Document of Joint Video Experts Team (JVET), JVET-P0666 Geneva, CH, Oct 2019.

- C.-Y. Chen, A. Segall, “Description of Core Experiment 5 (CE5): Cross-component adaptive loop filtering”, Document of Joint Video Experts Team (JVET), JVET-P2025 Geneva, CH, Oct 2019.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).