| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0101-v1 |
| --- | --- |

| Title: | On Transform Skip in JointCbCr mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba Masaru Ikeda Yoichi Yagasaki Teruhiko Suzuki 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-50-3750-2779 Takeshi.Tsukuba@sony.com Masaru.Ikeda@sony.com Yoichi.Yagasaki@sony.com Teruhiko.S@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

In VVC WD, transform skip for chroma can be used regardless of JointCbCr mode or not. However, in VTM, transform skip for chroma is restricted in JointCbCr mode; In JointCbCr mode, transform type equal to DCT2 is only evaluated while transform type equal to DCT2 or TS is evaluated in non-JointCbCr mode.

To solve the above miss-alignment of transform skip for chroma between in JointCbCr mode, this contribution proposes two variants: proposal#1) align SW with WD by reusing transform type in non-JointCbCr mode for JointCbCr mode (encoder-only change), and proposal#2) align WD with SW by omitting transform_skip_flag for chroma in JointCbCr mode (normative change).

It is reported that proposal#1 achieves average BD-rate savings of (0.00%, -0.06%, -0.04%) for AI, (0.00%, -0.06%, -0.01 %) for RA, and (-0.02 %, -0.23%, -0.32 %) for LB at normal QP; Especially, for SCC sequence, BD-rate savings of (-0.06%, -0.37%, -0.41%) for AI, (-0.06%, -0.40%, -0.34%) for RA, and (-0.04%, -0.34%, -0.36%) for LB.

It is reported that proposal#2 achieves average BD-rate savings of (0.00%, 0.01%, 0.00%) for AI, (0.00%, -0.04%, 0.03%) for RA, and (0.00%, -0.17%, -0.17%) for LB at normal QP; Especially, for SCC sequence, BD-rate savings of (-0.03%, -0.03%, -0.04%) for AI, (-0.05%, -0.08%, -0.04%) for RA, and (-0.13%, -0.23%, -0.21%) for LB.

Introduction

In the VVC WD [1], signaling condition of transform skip for chroma is defined as below:

// for Cb

| if( tu_cbf_cb[ xC ][ yC ] && treeType != DUAL_TREE_LUMA ) { |  |
| --- | --- |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 1 ] && wC <= MaxTsSize && hC <= MaxTsSize && !cu_sbt_flag ) |  |
| transform_skip_flag[ xC ][ yC ][ 1 ] | ae(v) |

// for Cr

| if( tu_cbf_cr[ xC ][ yC ] && treeType != DUAL_TREE_LUMA && !( tu_cbf_cb[ xC ][ yC ] && tu_joint_cbcr_residual_flag[ xC ][ yC ] ) ) { |  |
| --- | --- |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 2 ] && wC <= MaxTsSize && hC <= MaxTsSize && !cu_sbt_flag ) |  |
| transform_skip_flag[ xC ][ yC ][ 2 ] | ae(v) |

In other words, transform skip for chroma can be used regardless of JointCbCr mode or not. However, in VTM, transform skip for chroma is restricted in JointCbCr mode; In JointCbCr mode, transform type equal to DCT2 is only evaluated while transform type equal to DCT2 or TS is evaluated in non-JointCbCr mode.

Proposal

To solve the above miss-alignment of transform skip for chroma between WD and SW, this contribution proposes two variants: proposal#1) align SW with WD by reusing transform type in non-JointCbCr mode for JointCbCr mode (encoder-only change), and proposal#2) align WD with SW by omitting transform_skip_flag for chroma in JointCbCr mode (normative change)

Proposal#1: Align SW with WD by reusing transform type in non-JointCbCr mode for JointCbCr mode

To align SW with WD, transform type for codedCIdx in JointCbCr mode is set to best transform type for codedCIdx in non-JointCbCr mode without full search for transform skip for chroma:

if bdpcmModeChroma:

mtsIdx[codedCIdx] in JointCbCr mode = MTS_SKIP 
else:

mtsIdx[codedCIdx] in JointCbCr mode = bestMtsIdx[ codedCIdx ] in non-JointCbCr mode

, where codedCIdx = (TuCResMode[ xTbY ][ yTbY ]==1 || TuCResMode[ xTbY ][ yTbY ]==2) ? 1 : 2.

Proposal#2: Align WD with SW by omitting transform_skip_flag for chroma in JointCbCr mode

To align WD with SW, signaling transform_skip_flag for chroma is normatively restricted when JointCbCr mode (TuCResMode[x0][y0]>0) is used as below:

// Interaction with transform skip for chroma and JointCbCr mode

// for Cb

| if( tu_cbf_cb[ xC ][ yC ] && treeType != DUAL_TREE_LUMA ) { |  |
| --- | --- |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 1 ] && wC <= MaxTsSize && hC <= MaxTsSize && !cu_sbt_flag && TuCResMode[x0][y0]==0 ) |  |
| transform_skip_flag[ xC ][ yC ][ 1 ] | ae(v) |

// for Cr

| if( tu_cbf_cr[ xC ][ yC ] && treeType != DUAL_TREE_LUMA && !( tu_cbf_cb[ xC ][ yC ] && tu_joint_cbcr_residual_flag[ xC ][ yC ] ) ) { |  |
| --- | --- |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 2 ] && wC <= MaxTsSize && hC <= MaxTsSize && !cu_sbt_flag ) |  |
| transform_skip_flag[ xC ][ yC ][ 2 ] | ae(v) |

Simulation Results

The proposed changes were implemented on top of VTM-7.0 and following simulations were conducted under common test condition [2] and low QP condition [3]:

- Test1: VTM-7.0 vs Proposal#1 at normal QP

- Test2: VTM-7.0 vs Proposal#2 at normal QP

Table 3-1 to 3-2 shows the detail results of each Test, respectively.

Table 3-1: Results of Test1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | 0.04% | 99% | 100% |
| Class A2 | -0.01% | -0.06% | -0.06% | 102% | 103% |
| Class B | 0.00% | -0.04% | -0.04% | 100% | 100% |
| Class C | 0.00% | -0.13% | -0.08% | 101% | 102% |
| Class E | 0.00% | -0.03% | -0.04% | 102% | 103% |
| Overall | 0.00% | -0.06% | -0.04% | 101% | 101% |
| Class D | -0.02% | -0.17% | -0.07% | 103% | 102% |
| Class F | 0.02% | -0.33% | -0.28% | 97% | 98% |
| Class SCC | -0.06% | -0.37% | -0.41% | 98% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.06% | -0.06% | 103% | 104% |
| Class A2 | -0.01% | -0.12% | -0.02% | 102% | 102% |
| Class B | 0.01% | 0.00% | 0.01% | 100% | 100% |
| Class C | 0.00% | -0.08% | 0.01% | 102% | 104% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.06% | -0.01% | 102% | 102% |
| Class D | -0.03% | -0.58% | -0.13% | 104% | 105% |
| Class F | 0.00% | -0.33% | -0.40% | 99% | 100% |
| Class SCC | -0.06% | -0.40% | -0.34% | 99% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.32% | -0.26% | 100% | 102% |
| Class C | -0.03% | -0.43% | -0.35% | 101% | 102% |
| Class E | 0.00% | 0.19% | -0.39% | 103% | 103% |
| Overall | -0.02% | -0.23% | -0.32% | 101% | 102% |
| Class D | 0.00% | 0.24% | -0.65% | 104% | 104% |
| Class F | -0.01% | -0.80% | 0.12% | 103% | 103% |
| Class SCC | -0.04% | -0.34% | -0.36% | 100% | 100% |

Table 3-2: Results of Test2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | 0.02% | 101% | 102% |
| Class A2 | -0.01% | -0.02% | -0.02% | 101% | 102% |
| Class B | 0.00% | 0.03% | -0.01% | 98% | 101% |
| Class C | 0.01% | 0.01% | -0.01% | 100% | 102% |
| Class E | 0.00% | 0.01% | 0.03% | 101% | 103% |
| Overall | 0.00% | 0.01% | 0.00% | 100% | 102% |
| Class D | 0.01% | -0.01% | -0.03% | 103% | 104% |
| Class F | -0.01% | 0.01% | -0.01% | 97% | 99% |
| Class SCC | -0.03% | -0.03% | -0.04% | 97% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.15% | 0.00% | 102% | 103% |
| Class A2 | -0.01% | -0.04% | 0.10% | 102% | 103% |
| Class B | 0.00% | 0.03% | 0.04% | 100% | 101% |
| Class C | 0.03% | -0.04% | -0.04% | 102% | 104% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.04% | 0.03% | 101% | 102% |
| Class D | 0.00% | -0.22% | -0.16% | 103% | 104% |
| Class F | -0.02% | 0.02% | -0.02% | 100% | 101% |
| Class SCC | -0.05% | -0.08% | -0.04% | 100% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.41% | -0.17% | 99% | 101% |
| Class C | 0.00% | -0.24% | -0.18% | 100% | 102% |
| Class E | 0.02% | 0.35% | -0.15% | 103% | 102% |
| Overall | 0.00% | -0.17% | -0.17% | 100% | 101% |
| Class D | -0.02% | 0.13% | -0.26% | 104% | 103% |
| Class F | 0.01% | -0.28% | -0.14% | 99% | 101% |
| Class SCC | -0.13% | -0.23% | -0.21% | 99% | 101% |

Conclusion

It is recommended to adopt either of proposed method into the next VTM/VVC WD.

Reference

[1] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 7), ” JVET-P2001, Oct. 2019

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Apr. 2019

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

WD changes (proposal#2)

- 7.3.9.5 Coding unit syntax

| if( sps_joint_cbcr_enabled_flag && ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && ( tu_cbf_cb[ xC ][ yC ] ¦ ¦ tu_cbf_cr[ xC ][ yC ] ) ) ¦ ¦ ( tu_cbf_cb[ xC ][ yC ] && tu_cbf_cr[ xC ][ yC ] ) ) && chromaAvailable ) |  |
| --- | --- |
| tu_joint_cbcr_residual_flag[ xC ][ yC ] | ae(v) |

tu_joint_cbcr_residual_flag[ x0 ][ y0 ] specifies whether the residual samples for both chroma components Cb and Cr are coded as a single transform block. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture.

tu_joint_cbcr_residual_flag[ x0 ][ y0 ] equal to 1 specifies that the transform unit syntax includes the transform coefficient levels for a single transform block from which the residual samples for both Cb and Cr are derived. tu_joint_cbcr_residual_flag[ x0 ][ y0 ] equal to 0 specifies that the transform coefficient levels of the chroma components are coded as indicated by the syntax elements tu_cbf_cb[ x0 ][ y0 ] and tu_cbf_cr[ x0 ][ y0 ].

When tu_joint_cbcr_residual_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

Depending on tu_joint_cbcr_residual_flag[ x0 ][ y0 ], tu_cbf_cb[ x0 ][ y0 ], and tu_cbf_cr[ x0 ][ y0 ], the variable TuCResMode[ x0 ][ y0 ] is derived as follows:

- If tu_joint_cbcr_residual_flag[ x0 ][ y0 ] is equal to 0, the variable TuCResMode[ x0 ][ y0 ] is set equal to 0;

- Otherwise, if tu_cbf_cb[ x0 ][ y0 ] is equal to 1 and tu_cbf_cr[ x0 ][ y0 ] is equal to 0, the variable TuCResMode[ x0 ][ y0 ] is set equal to 1;

- Otherwise, if tu_cbf_cb[ x0 ][ y0 ] is equal to 1, the variable TuCResMode[ x0 ][ y0 ] is set equal to 2;

- Otherwise, the variable TuCResMode[ x0 ][ y0 ] is set equal to 3.

- 7.3.9.10 Transform unit syntax

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| … |  |
| if( tu_cbf_cb[ xC ][ yC ] && treeType != DUAL_TREE_LUMA ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 1 ] && wC <= MaxTsSize && hC <= MaxTsSize && !cu_sbt_flag && TuCResMode[x0][y0]==0 ) |  |
| transform_skip_flag[ xC ][ yC ][ 1 ] | ae(v) |
| if( !transform_skip_flag[ xC ][ yC ][ 1 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| else |  |
| residual_ts_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| } |  |
| if( tu_cbf_cr[ xC ][ yC ] && treeType != DUAL_TREE_LUMA && !( tu_cbf_cb[ xC ][ yC ] && tu_joint_cbcr_residual_flag[ xC ][ yC ] ) ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 2 ] && wC <= MaxTsSize && hC <= MaxTsSize && !cu_sbt_flag ) |  |
| transform_skip_flag[ xC ][ yC ][ 2 ] | ae(v) |
| if( !transform_skip_flag[ xC ][ yC ][ 2 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| else |  |
| residual_ts_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| } |  |

transform_skip_flag[ x0 ][ y0 ][ cIdx ] specifies whether a transform is applied to the associated transform block or not. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index cIdx specifies an indicator for the colour component; it is equal to 0 for Y, 1 for Cb, and 2 for Cr. transform_skip_flag[ x0 ][ y0 ][ cIdx ] equal to 1 specifies that no transform is applied to the associated transform block. transform_skip_flag[ x0 ][ y0 ][ cIdx ] equal to 0 specifies that the decision whether transform is applied to the associated transform block or not depends on other syntax elements.

When transform_skip_flag[ x0 ][ y0 ][ cIdx ] is not present, it is inferred as follows:

- If cIdx is not equal to 0 and TuCResMode[x0][y0] is not equal to 0, transform_skip_flag[ x0 ][ y0 ][ cIdx ] is inferred to be equal to 0.

- Otherwise If BdpcmFlag[ x0 ][ y0 ][ cIdx ] is equal to 1, transform_skip_flag[ x0 ][ y0 ][ cIdx ] is inferred to be equal to 1.

- Otherwise ( if BdpcmFlag[ x0 ][ y0 ][ cIdx ] is equal to 0), transform_skip_flag[ x0 ][ y0 ][ cIdx ] is inferred to be equal to 0.