| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0124-v1 |
| --- | --- |

| Title: | CE5-related: JC-CCALF with Power of 2 Weight Values |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Che-Wei Kuo Jingya Li Chong Soon Lim | Tel: Email: | +65-6550-5423 chewei.kuo@sg.panasonic.com jingya.li@sg.panasonic.com chongsoon.lim@sg.panasoni.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to use power of 2 values for JC-CCALF weight candidates, to avoid multiplication or division in Cr refinement value derivation. In method 1, the weight candidates are restricted to ±1. In method 2, the weight candidates are extended to 16 weights: ±1/8, ±1/4, ±1/2, ±1, ±2, ±4, ±8, and ±16. Compared with the JC-CCALF in CE5-3.2, negligible coding difference and runtime change are observed.

Simulation results with joint YUV performance (AHG13 8:1:1 metric) are summarized as follows.

|  | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV | Y | U | V | YUV |
| CCALF (CE5 anchor) | 0.17% | -8.31% | -7.06% | -1.40% | 0.14% | -10.14% | -9.82% | -1.88% | 0.12% | -13.00% | -9.48% | -2.15% |
| JC-CCALF (CE5-3.2) | 0.10% | -5.84% | -4.87% | -0.99% | 0.16% | -8.63% | -8.27% | -1.56% | 0.12% | -12.71% | -9.47% | -2.12% |
| weights = ±1 | 0.10% | -6.15% | -5.02% | -1.04% | 0.15% | -9.05% | -8.73% | -1.66% | 0.12% | -12.86% | -9.89% | -2.18% |
| weights = ±1/8 to ±16 | 0.10% | -6.06% | -5.09% | -1.04% | 0.14% | -8.91% | -8.56% | -1.64% | 0.12% | -12.88% | -9.83% | -2.17% |

# Introduction

JC-CCALF is proposed in JVET-P0372 and JVET-Q0229. It halves the CCALF filtering operation by reusing the Cb refinement value for Cr, with an additional weight multiplication in Cr refinement value derivation. The selected weight is signaled in the slice header with a sign flag and an index specifying the absolute value. Table 1 and Table 2 list the weight candidates of the proposed JC-CCALF. It is noted that applying these weights require multiplication or division, and they can be replaced with power of 2 values to further simplify the filtering process without harming coding efficiency.

Table 1: JC-CCALF weight candidates in P0372

| Sign | Index | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ± | ¦ w ¦ | 1/4 | 1/2 | 3/4 | 1 | 4 | 2 | 4/3 |

Table 2: JC-CCALF weight candidates in Q0229

| Sign | Index | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ± | ¦ w ¦ | 1/4 | 1/2 | 3/4 | 1 | 4 | 2 | 3/2 |

# Proposal

In this contribution, it is proposed to use power of 2 values for JC-CCALF weight candidates, to avoid multiplication or division in Cr refinement value derivation. In method 1, the weight candidates are restricted to ±1. In method 2, the weight candidates are extended to ±1/8, ±1/4, ±1/2, ±1, ±2, ±4, ±8, and ±16. A sign flag and a shift value ranges from -4 to 3 indicating the right shift bits (for method 2) are signaled in the slice header. Compared with the original JC-CCALF with weight multiplication, method 1 reduces the number of candidates from 14 to 2, and only sign change is needed.

In implementation, similar with the JC-CCALF encoder, a priority-based fast weight test is applied for speedup. Also, the encoder and decoder are simplified by the shift characteristic.

Table 3: Proposed JC-CCALF weight candidates in Method 1

| Sign | rShift | 0 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ± | ¦ w ¦ | 1 |  |  |  |  |  |  |  |

Table 4: Proposed JC-CCALF weight candidates in Method 2

| Sign | rShift | -3 | -2 | -1 | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ± | ¦ w ¦ | 1/8 | 1/4 | 1/2 | 1 | 2 | 4 | 8 | 16 |

# Simulation results

Simulation results are reported as below. The JVET common test conditions defined in [1] are followed.

## Method 1

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.12% | -3.87% | -3.37% | -0.63% | 101% | 101% |
| Class A2 | 0.08% | -4.89% | -9.46% | -1.37% | 101% | 102% |
| Class B | 0.08% | -10.03% | -6.87% | -1.62% | 101% | 101% |
| Class C | 0.08% | -2.97% | -2.79% | -0.51% | 101% | 101% |
| Class E | 0.12% | -7.50% | -2.14% | -0.87% | 101% | 101% |
| Overall | 0.10% | -6.15% | -5.02% | -1.04% | 101% | 101% |
| Class D | 0.07% | -3.02% | -1.86% | -0.43% | 101% | 99% |
| Class F | 0.07% | -4.63% | -4.99% | -0.91% | 101% | 101% |
| TGM | 0.04% | -3.97% | -3.78% | -0.75% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.14% | -5.11% | -4.66% | -0.87% | 101% | 101% |
| Class A2 | 0.14% | -9.31% | -13.97% | -2.21% | 102% | 102% |
| Class B | 0.21% | -15.34% | -12.37% | -2.61% | 101% | 101% |
| Class C | 0.09% | -3.93% | -3.28% | -0.65% | 100% | 101% |
| Class E |  |  |  |  |  |  |
| Overall | 0.15% | -9.05% | -8.73% | -1.66% | 101% | 101% |
| Class D | 0.08% | -4.07% | -3.30% | -0.68% | 100% | 101% |
| Class F | 0.10% | -3.52% | -3.60% | -0.63% | 100% | 101% |
| TGM | 0.26% | -4.55% | -4.74% | -0.72% | 101% | 101% |
|  | Low delay B Main10 |  |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 0.14% | -18.70% | -16.89% | -3.45% | 100% | 101% |
| Class C | 0.15% | -5.41% | -4.43% | -0.86% | 100% | 100% |
| Class E | 0.07% | -13.07% | -5.50% | -1.80% | 100% | 100% |
| Overall | 0.12% | -12.86% | -9.89% | -2.18% | 100% | 101% |
| Class D | 0.02% | -5.16% | -4.20% | -0.92% | 100% | 100% |
| Class F | 0.00% | -7.93% | -6.58% | -1.45% | 100% | 100% |
| TGM | -0.14% | -6.33% | -5.86% | -1.33% | 100% | 101% |

## Method 2

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.12% | -3.48% | -3.79% | -0.63% | 101% | 102% |
| Class A2 | 0.08% | -4.87% | -9.75% | -1.40% | 101% | 102% |
| Class B | 0.08% | -9.47% | -6.68% | -1.55% | 101% | 102% |
| Class C | 0.09% | -3.01% | -2.89% | -0.52% | 101% | 102% |
| Class E | 0.13% | -8.24% | -2.01% | -0.92% | 101% | 101% |
| Overall | 0.10% | -6.06% | -5.09% | -1.04% | 101% | 102% |
| Class D | 0.07% | -2.54% | -1.86% | -0.39% | 100% | 99% |
| Class F | 0.06% | -4.53% | -4.70% | -0.87% | 101% | 101% |
| TGM | 0.03% | -4.14% | -3.69% | -0.76% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.13% | -5.00% | -4.73% | -0.87% | 101% | 101% |
| Class A2 | 0.14% | -9.02% | -13.96% | -2.19% | 100% | 101% |
| Class B | 0.18% | -15.13% | -11.84% | -2.55% | 100% | 101% |
| Class C | 0.08% | -3.96% | -3.27% | -0.66% | 100% | 101% |
| Class E |  |  |  |  |  |  |
| Overall | 0.14% | -8.91% | -8.56% | -1.64% | 100% | 101% |
| Class D | 0.05% | -3.99% | -2.95% | -0.65% | 100% | 100% |
| Class F | 0.10% | -3.10% | -3.06% | -0.53% | 100% | 101% |
| TGM | 0.19% | -4.04% | -4.44% | -0.70% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 0.16% | -18.70% | -16.56% | -3.40% | 101% | 101% |
| Class C | 0.12% | -5.42% | -4.33% | -0.87% | 100% | 100% |
| Class E | 0.07% | -13.12% | -5.94% | -1.85% | 101% | 100% |
| Overall | 0.12% | -12.88% | -9.83% | -2.17% | 101% | 101% |
| Class D | 0.11% | -5.90% | -4.38% | -0.94% | 100% | 100% |
| Class F | 0.03% | -7.67% | -5.76% | -1.32% | 100% | 100% |
| TGM | -0.14% | -6.20% | -5.70% | -1.30% | 101% | 101% |

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, March 2019.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).