| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0127_r1 |
| --- | --- |

| Title: | Non-CE4: On merge list generation for geometric partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Kei Kawamura Kyohei Unno Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-80-6887-2592 yo-kidani@kddi.com ki-kawamura@kddi.com ky-unno@kddi-research.jp |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution proposes to generate a merge list for geometric partitioning. The proposed method is to reorder neighboring spatial and temporal merge processes when the current block is predicted with geometric partitioning. Simulation results show that an overall luma BD-rates are 0.00%/0.03% for RA/LDB compared with common base scheme based on VTM 7.0 without running time increment.

Introduction

Inter prediction with geometric partitioning is discussed for several meeting cycles to improve the objective and subjective performance. A common base scheme was defined in the new established core experiment 4, which replaces the existing inter prediction including triangle partitioning with that including geometric partitioning [1][2]. Since the triangle partitioning consists of two equaled triangles, merge list construction with triangle partitioning can be similar to that with regular partitioning. On the other hand, geometric partitioning consists of two unequaled regions, which causes disconnected region in a merge list generation. To solve this situation, this contribution proposes reorder the merge list generation process.

Proposed solutions

The proposed solution is derived from two motivations. One is a spatial MV similarity and another is a temporal MV similarity.

As the spatial MV similarity, when the block is predicted with geometric partitioning, possibility of disconnected region for merge block may increase due to the unequaled regions. To consider such the situation, diversity of spatial marge candidates can reduce the disconnect region in the merge list. To realize it as merge list generation process, we propose to reorder neighboring spatial merge processes.

In detail, when the geometric partitioning is used, B2 is reorder in front of B0 and A0 without any other conditions. It is noted that the current order of candidates for a marge list are as follows; B1, A1, B0, A0, and B2, following by some special candidates like temporal, history, pairwise, and zero. In the working draft 7, spatial candidates are illustrated as follows.

Figure 1 Location of spatial candidates in the working draft 7.

As the temporal MV similarity, when temporal motion vector prediction is enabled in a current bitstream, disconnected region can borrow motion vectors from a co-located block in a temporally different picture. To realize is as a merge list generation process, we propose to reorder the temporal merge order in a merge list generation.

In detail, the order of processes for merge list is modified as follows; B1, A1, B2, temporal, B0, and A0, following by remaining special merge processes like history, pairwise, and zero. It is noted that pruning process is maintained as regular marge list generation.

Simulation results

The proposed methods are implemented on the top of CE4 common base software. The anchor is common base scheme based on VTM 7.0, which test conditions are following the core experiment 4 [3].

It is observed that there is coding impact of class C and E. Encoding and decoding run time is almost the same.

Table 1 The simulation results of the prposed method vs common base scheme

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over common base scheme based on VTM 7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.09% | 0.05% | 99% | 100% |
| Class A2 | 0.01% | 0.02% | -0.03% | 99% | 100% |
| Class B | 0.01% | -0.01% | 0.05% | 99% | 100% |
| Class C | -0.02% | 0.06% | 0.04% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.03% | 99% | 100% |
| Class D | 0.00% | 0.04% | 0.13% | 100% | 100% |
| Class F | 0.03% | 0.08% | 0.06% | 99% | 100% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over common base scheme based on VTM 7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.24% | -0.02% | 99% | 100% |
| Class C | 0.08% | 0.07% | 0.15% | 99% | 100% |
| Class E | 0.04% | -0.01% | 0.16% | 99% | 100% |
| Overall | 0.03% | -0.08% | 0.08% | 99% | 100% |
| Class D | 0.00% | 0.15% | 0.50% | 99% | 101% |
| Class F | 0.01% | -0.17% | 0.15% | 99% | 100% |

Conclusion

It was proposed to reorder neighboring spatial merge and temporal merge processes in the merge list generation when the block is predicted with geometric partitioning. It is noted that the difference among regular merge and geometric merge is very minor.

This work was conducted under the research project of Ministry of Internal Affairs and Communication, "Next generation video transmission and communication technology for ultra-high definition video application under versatile purpose and environment".

References

- H. Gao, S. Essenlik, E. Alshina, A. M. Kotra, B. Wang, M. Blaser, J. Sauer, “Simplified GEO without multiplication and minimum blending mask storage (harmonization of JVET-P0107, JVET-P0264 and JVET-P0304)”, JVET-P0884, Geneva, CH, 1–11 October 2019.

- K. Reuzé, C.-C Chen, H. Huang, W.-J Chien, V. Seregin, M. Karczewicz, R.-L Liao, J. Chen, Y. Ye, J. Luo, M. Bläser, J. Sauer, “Simplified GEO without multiplication and minimum blending mask storage (harmonization of JVET-P0107, JVET-P0264 and JVET-P0304)”, JVET-P0885, Geneva, CH, 1–11 October 2019.

- C.-C. Chen, R.-L. Liao, X. Xiu, H. Yang, “Description of Core Experiment 4 (CE4): Inter prediction with geometric partitioning”, JVET-P2024, Geneva, CH, 1–11 October 2019.

- https://vcgit.hhi.fraunhofer.de/jvet-p-ce4/VVCSoftware_VTM/blob/common_base/CE4_Common_Base_WD(on_top_of_JVET-P2001-vE).docx

Patent rights declaration

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Spec text

The following shows the difference of proposed method from WD of CE4 common base scheme on top of JVET-P2001-vE.docx [4] as highlighted by yellow.

- 8.5.2.2 Derivation process for luma motion vectors for merge mode

This process is only invoked when general_merge_flag[ xCb ][ yCb ] is equal to 1, where ( xCb, yCb ) specify the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture.

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Outputs of this process are:

- the luma motion vectors in 1/16 fractional-sample accuracy mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ],

- the reference indices refIdxL0 and refIdxL1,

- the prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ],

- the half sample interpolation filter index hpelIfIdx,

- the bi-prediction weight index bcwIdx.

- the merging candidate list mergeCandList.

The bi-prediction weight index bcwIdx is set equal to 0.

The motion vectors mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ], the reference indices refIdxL0 and refIdxL1 and the prediction utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ] are derived by the following ordered steps:

- The derivation process for spatial merging candidates from neighbouring coding units as specified in clause 8.5.2.3 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth, and the luma coding block height cbHeight as inputs, and the output being the availability flags availableFlagA0, availableFlagA1, availableFlagB0, availableFlagB1 and availableFlagB2, the reference indices refIdxLXA0, refIdxLXA1, refIdxLXB0, refIdxLXB1 and refIdxLXB2, the prediction list utilization flags predFlagLXA0, predFlagLXA1, predFlagLXB0, predFlagLXB1 and predFlagLXB2, and the motion vectors mvLXA0, mvLXA1, mvLXB0, mvLXB1 and mvLXB2, with X being 0 or 1, the half sample interpolation filter indices hpelIfIdxA0, hpelIfIdxA1, hpelIfIdxB0, hpelIfIdxB1, hpelIfIdxB2, and the bi-prediction weight indices bcwIdxA0, bcwIdxA1, bcwIdxB0, bcwIdxB1, bcwIdxB2.

- The reference indices, refIdxLXCol, with X being 0 or 1, and the bi-prediction weight index bcwIdxCol for the temporal merging candidate Col are set equal to 0 and hpelIfIdxCol is set equal to 0.

- The derivation process for temporal luma motion vector prediction as specified in in clause 8.5.2.11 is invoked with the luma location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight and the variable refIdxL0Col as inputs, and the output being the availability flag availableFlagL0Col and the temporal motion vector mvL0Col. The variables availableFlagCol, predFlagL0Col and predFlagL1Col are derived as follows:

availableFlagCol = availableFlagL0Col		(485)

predFlagL0Col = availableFlagL0Col		(486)

predFlagL1Col = 0		(487)

- When slice_type is equal to B, the derivation process for temporal luma motion vector prediction as specified in clause 8.5.2.11 is invoked with the luma location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight and the variable refIdxL1Col as inputs, and the output being the availability flag availableFlagL1Col and the temporal motion vector mvL1Col. The variables availableFlagCol and predFlagL1Col are derived as follows:

availableFlagCol = availableFlagL0Col  | |  availableFlagL1Col	(488)

predFlagL1Col = availableFlagL1Col		(489)

- The merging candidate list, mergeCandList, is constructed as follows:

i = 0
if( availableFlagB1 )
	mergeCandList[ i++ ] = B1
if( availableFlagA1 )
	mergeCandList[ i++ ] = A1
if( availableFlagB0 && !MergeGeoFlag)
	mergeCandList[ i++ ] = B0		(490)
if( availableFlagA0 && !MergeGeoFlag)
	mergeCandList[ i++ ] = A0
if( availableFlagB2 )
	mergeCandList[ i++ ] = B2
if( availableFlagCol )
	mergeCandList[ i++ ] = Col
if( availableFlagB0 && MergeGeoFlag)
	mergeCandList[ i++ ] = B0
if( availableFlagA0 && MergeGeoFlag)
	mergeCandList[ i++ ] = A0

- The variable numCurrMergeCand and numOrigMergeCand are set equal to the number of merging candidates in the mergeCandList.

- When numCurrMergeCand is less than (MaxNumMergeCand − 1) and NumHmvpCand is greater than 0, the following applies:

- The derivation process of history-based merging candidates as specified in 8.5.2.6 is invoked with mergeCandList and numCurrMergeCand as inputs, and modified mergeCandList and numCurrMergeCand as outputs.

- numOrigMergeCand is set equal to numCurrMergeCand.

- When numCurrMergeCand is less than MaxNumMergeCand and greater than 1, the following applies:

- The derivation process for pairwise average merging candidate specified in clause 8.5.2.4 is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N, and the half sample interpolation filter index hpelIfIdxN of every candidate N in mergeCandList, and numCurrMergeCand as inputs, and the output is assigned to mergeCandList, numCurrMergeCand, the reference indices refIdxL0avgCand and refIdxL1avgCand, the prediction list utilization flags predFlagL0avgCand and predFlagL1avgCand, the motion vectors mvL0avgCand and mvL1avgCand, and the half-sample interpolation filter index hpelIfIdxavgCand of candidate avgCand being added into mergeCandList. The bi-prediction weight index bcwIdx of candidate avgCand being added into mergeCandList is set equal to 0.

- numOrigMergeCand is set equal to numCurrMergeCand.

- The derivation process for zero motion vector merging candidates specified in clause 8.5.2.5 is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList and numCurrMergeCand as inputs, and the output is assigned to mergeCandList, numCurrMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm and the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added into mergeCandList. The half sample interploation filter index hpelIfIdx of every new candidate zeroCandm being added into mergeCandList is set equal to 0. The bi-prediction weight index bcwIdx of every new candidate zeroCandm being added into mergeCandList is set equal to 0. The number of candidates being added, numZeroMergeCand, is set equal to ( numCurrMergeCand − numOrigMergeCand ). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand − 1, inclusive.

- The following assignments are made with N being the candidate at position merge_idx[ xCb ][ yCb ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xCb ][ yCb ] ] ) and X being replaced by 0 or 1:

refIdxLX = refIdxLXN		(491)

predFlagLX[ 0 ][ 0 ] = predFlagLXN		(492)

mvLX[ 0 ][ 0 ][ 0 ] = mvLXN[ 0 ]		(493)

mvLX[ 0 ][ 0 ][ 1 ] = mvLXN[ 1 ]		(494)

hpelIfIdx = hpelIfIdxN		(495)

bcwIdx = bcwIdxN		(496)

- When mmvd_merge_flag[ xCb ][ yCb ] is equal to 1, the following applies:

- The derivation process for merge motion vector difference as specified in 8.5.2.7 is invoked with the luma location ( xCb, yCb ), the reference indices refIdxL0, refIdxL1 and the prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ] as inputs, and the motion vector differences mMvdL0 and mMvdL1 as outputs.

- The motion vector difference mMvdLX is added to the merge motion vectors mvLX for X being 0 and 1 as follows:

mvLX[ 0 ][ 0 ][ 0 ] += mMvdLX[ 0 ]		(497)

mvLX[ 0 ][ 0 ][ 1 ] += mMvdLX[ 1 ]		(498)

mvLX[ 0 ][ 0 ][ 0 ] = Clip3( −217, 217 − 1, mvLX[ 0 ][ 0 ][ 0 ] )	(499)

mvLX[ 0 ][ 0 ] [ 1 ] = Clip3( −217, 217 − 1, mvLX[ 0 ][ 0 ][ 1 ] ) 	(500)