| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0134-v2 |
| --- | --- |

| Title: | AHG8: Disabling reference wraparound for reference picture resampling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Brian Heng, Peisong Chen, Tim Hellman, Wade Wan, Minhua Zhou 1320 Ridder Park Drive San Jose, California 95131 | Email: | firstname.lastname@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

In the VVC Draft 7, there is no restriction on enabling RPR and reference picture wraparound at the same time.  This contribution contends that reference wraparound should be disabled when RPR is enabled, in that the reference picture wraparound does not work when a current picture and its reference have different resolutions.

# Introduction

RPR was adopted to VVC in the15th meeting at Gothenburg on July 2019 [1]. It supports adaptive resolution change within a coded video sequence. Specifically, the current picture’s resolution can be different from those of its reference pictures.

Reference picture wraparound is primarily to support certain 360o video applications, where the left and right edges of the video or a part of the video belongs to one continuous scene.

In the current VVC Specification [2], it is not prohibited to have both RPR and reference picture wraparound enabled at the same time, even though these two coding tools do not work jointly in VTM 7.0 reference software.

According to the current VVC Specification [2],

“sps_ref_wraparound_offset_minus1 plus 1 specifies the offset used for computing the horizontal wrap-around position in units of MinCbSizeY luma samples. The value of ref_wraparound_offset_minus1 shall be in the range of ( CtbSizeY / MinCbSizeY ) + 1 to ( pic_width_in_luma_samples / MinCbSizeY ) − 1, inclusive, where pic_width_in_luma_samples is the value of pic_width_in_luma_samples in any PPS that refers to the SPS.”

sps_ref_wraparound_offset_minus1 is a fixed number for all pictures referring to the sequence parameter set and is defined at the granularity of MinCbSizeY. Under the scenario of reference pictures with different resolutions from the current picture, a fixed wrap-around offset does not make sense. Furthermore,  the unit of MinCbSizeY as required in the specification does not match flexible scaling ratios used in RPR. According to the specification, the reference picture scaling ratios are defined as

RefPicScale[ i ][ j ][ 0 ] = ( ( fRefWidth  <<  14 ) + ( PicOutputWidthL  >>  1 ) ) / PicOutputWidthL
		RefPicScale[ i ][ j ][ 1 ] = ( ( fRefHeight  <<  14 ) + ( PicOutputHeightL  >>  1 ) ) / PicOutputHeightL

To resolve these issues, it is proposed that reference wraparound should be disabled when RPR is on.

# Specification draft changes

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …… |  |
| ref_pic_resampling_enabled_flag | u(1) |
| …… |  |
| if( !ref_pic_resampling_enabled_flag ) |  |
| sps_ref_wraparound_enabled_flag | u(1) |
| …… |  |
| } |  |

sps_ref_wraparound_enabled_flag equal to 1 specifies that horizontal wrap-around motion compensation is applied in inter prediction. sps_ref_wraparound_enabled_flag equal to 0 specifies that horizontal wrap-around motion compensation is not applied. When the value of ( CtbSizeY / MinCbSizeY + 1) is less than or equal to ( pic_width_in_luma_samples / MinCbSizeY − 1 ), where pic_width_in_luma_samples is the value of pic_width_in_luma_samples in any PPS that refers to the SPS, the value of sps_ref_wraparound_enabled_flag shall be equal to 0.

When not present, the value of sps_ref_wraparound_enabled_flag is inferred to be equal to 0.

# References

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, Gothenburg, SE, July 2019.

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 7)”, JVET-P2001, Geneva, CH, October 2019.

# Patent rights declaration(s)

Broadcom Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).