| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0136-v1 |
| --- | --- |

| Title: | Alignment of MTS index signalling condition with MTS zero-out |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Moonmo Koo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Mehdi Salehifar Seung-Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +1-858-635-5226 moonmo.koo@lge.com jaehyun.lim@lge.com mehdi.salehifar@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In the recent VVC working draft, MTS index is signalled only if last non-zero coefficient is not located outside of top-left 16x16 region in a Luma TB, which is motivated by MTS zero-out. However, this scheme cannot detect the case that last non-zero coefficient lies within the top-left 16x16 region outside of which at least one non-zero coefficient exists, as the case that MTS index does not need to be signalled. In this case, the associated MTS index could have been inferred to be zero, i.e. DCT-2 pair, which was reported via JVET bug tracker ticket #678. In the VVC WD, a constraint that MTS index must be signalled to be zero when that case occurs is described for bitstream conformance.

In this contribution, it is proposed that a variable, MtsZeroOutSigCoeffFlag should be determined by checking whether coded_sub_block_flag or sig_coeff_flag outside of the top-left 16x16 region is zero or not, in order to indicate the existence of non-zero coefficients outside of the top-left 16x16 region. The Y/U/V BD-rate changes are 0.00%/0.00%/-0.01% (AI), 0.00%/-0.01%/0.02% (RA), and 0.00%/-0.10%/-0.03% (LD) under CTC. In overall, performance and complexity of the proposed method are almost the same as those of VTM 7.0 and design of MTS index signalling becomes more consistent with the existing MTS zeroing-out. Furthermore, the bitstream conformance constraint can be removed, which may reduce conformance testing burden.

Problem Statement

In the current MTS index signalling, the cases that MTS index does not have to be signalled, which are certain that DCT-2 pair was applied, are filtered out with a variable, MtsZeroOutSigCoeffFlag, as the following two syntax tables:

Table 1  The MTS index signaling in the current VVC WD (JVET-P2001-vE.docx)

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { |  |
| --- | --- |
| …… |  |
| if( treeType != DUAL_TREE_CHROMA && lfnst_idx = = 0 && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && Max( cbWidth, cbHeight ) <= 32 && IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT && cu_sbt_flag = = 0 && MtsZeroOutSigCoeffFlag = = 1 && tu_cbf_luma[ x0 ][ y0 ] ) { |  |
| if( ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag ) ) ) |  |
| mts_idx | ae(v) |
| } |  |
| …… |  |

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| …… |  |  |
| numSbCoeff = 1 << ( log2SbW + log2SbH ) |  |
| lastScanPos = numSbCoeff |  |
| lastSubBlock = ( 1 << ( log2TbWidth + log2TbHeight − ( log2SbW + log2SbH ) ) ) − 1 |  |
| do { |  |
| if( lastScanPos = = 0 ) { |  |
| lastScanPos = numSbCoeff |  |
| lastSubBlock− − |  |
| } |  |
| lastScanPos− − |  |
| xS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ lastSubBlock ][ 0 ] |  |
| yS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ lastSubBlock ][ 1 ] |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ lastScanPos ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ lastScanPos ][ 1 ] |  |
| } while( ( xC != LastSignificantCoeffX ) ¦ ¦ ( yC != LastSignificantCoeffY ) ) |  |
| …… |  |
| if( ( LastSignificantCoeffX > 15 ¦ ¦ LastSignificantCoeffY > 15 ) && cIdx = = 0 ) |  |
| MtsZeroOutSigCoeffFlag = 0 |  |
| …… |  |

As shown in Table 1, one of the conditions for filtering out is to check whether or not last non-zero coefficient lies inside of top-left 16x16 region in a Luma TB that corresponds to where transform coefficients generated from DST-7 or DCT-8 can exist. However, the filtering-out is not perfect, because there can happen the case that non-zero coefficients exist although last non-zero coefficient is placed within the top-left 16x16 region. This case can occur only when DCT-2 pair is applied, but it cannot be detected from the current condition checking in Table 1 so that the associated MTS index must be signalled to be 0 unnecessarily.  Therefore, it seems to be consistent with the current MTS zeroing-out design to separate out DCT-2 cases completely through explicitly checking the existence of non-zero coefficients outside of the top-left 16x16 region. The suggested spec text change to realize this is presented as the following two syntax tables:

Table 2  The suggested spec text change using coded_sub_block_flag for deciding MtsZeroOutSigCoeffFlag that indicates the existence of non-zero coefficients outside of top-left 16x16 region in a Luma TB

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| …… |  |  |
| if( ( LastSignificantCoeffX > 15 ¦ ¦ LastSignificantCoeffY > 15 ) && cIdx = = 0 ) |  |  |
| MtsZeroOutSigCoeffFlag = 0 |  |  |
| …… |  |  |
| for( i = lastSubBlock; i >= 0; i− − ) { |  |
| startQStateSb = QState |  |
| xS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ i ][ 0 ] |  |
| yS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ i ][ 1 ] |  |
| inferSbDcSigCoeffFlag = 0 |  |
| if( ( i < lastSubBlock ) && ( i > 0 ) ) { |  |
| coded_sub_block_flag[ xS ][ yS ] | ae(v) |
| inferSbDcSigCoeffFlag = 1 |  |
| } |  |
| if( coded_sub_block_flag[ xS ][ yS ] && ( ( xS << log2SbW ) > 15 ¦ ¦ ( yS << log2SbH ) > 15 ) && cIdx == 0 ) |  |
| MtsZeroOutSigCoeffFlag = 0 |  |
| …… |  |
| } |  |
| } |  |  |

Table 3  The suggested spec text change using sig_coeff_flag for deciding MtsZeroOutSigCoeffFlag that indicates the existence of non-zero coefficients outside of top-left 16x16 region in a Luma TB

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| …… |  |
| if( ( LastSignificantCoeffX > 15 ¦ ¦ LastSignificantCoeffY > 15 ) && cIdx = = 0 ) |  |
| MtsZeroOutSigCoeffFlag = 0 |  |
| …… |  |
| for( n = firstPosMode0; n >= 0 && remBinsPass1 >= 4; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( coded_sub_block_flag[ xS ][ yS ] && ( n > 0 ¦ ¦ !inferSbDcSigCoeffFlag ) && ( xC != LastSignificantCoeffX ¦ ¦ yC != Last SignificantCoeffY ) ) { |  |
| sig_coeff_flag[ xC ][ yC ] | ae(v) |
| remBinsPass1− − |  |
| if( sig_coeff_flag[ xC ][ yC ] ) |  |
| inferSbDcSigCoeffFlag = 0 |  |
| } |  |
| if( sig_coeff_flag[ xC ][ yC ] ) { |  |  |
| if( ( xC > 15 ¦ ¦ yC > 15 ) && cIdx = = 0 ) |  |  |
| MtsZeroOutSigCoeffFlag = 0 |  |  |
| …… |  |
| } |  |
| …… |  |
| } |  |
| …… |  |
| } |  |
| } |  |  |

The above Table 2 and Table 3 make use of coded_sub_block_flag and sig_coeff_flag respectively, in order to figure out whether non-zero coefficients exist in unallowable area if DST-7 or DCT-8 was applied in both horizontal and vertical directions. It should be noted that coded_sub_block_flag and sig_coeff_flag are inferred as 1 if it is not present, so that any extra conditions for taking care of the cases that coded_sub_block_flag or sig_coeff_flag is not signalled are not necessary.

In the current spec text, a constraint to enforce MTS index to be signalled to be zero, i.e. DCT-2 pair, when last non-zero coefficient resides in top-left 16x16 region outside of which non-zero coefficients exist, is described for bitstream conformance in section 7.4.10.5 (Coding unit semantics). Employing the proposed Table 2 or Table 3, the constraint can be removed as in Fig. 1, because the cases that DCT-2 pair was certainly applied can be clearly separated from others.

7.4.10.5 Coding unit semantics …… mts_idx specifies which transform kernels are applied along the horizontal and vertical direction of the associated luma transform blocks in the current coding unit. When mts_idx is not present, it is inferred to be equal to 0. It is a requirement of bitstream conformance that mts_idx shall be equal to 0 if in the current coding unit at least one coded_sub_block_flag[ xS ][ yS ] in the residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) syntax structure is not equal to 0 for cIdx equal to 0 and xS or yS greater than 3. ……

Figure 1  The removal of the bitstream conformance constraint if the proposed method in Table 2 or Table 3 is employed for deciding MtsZeroOutSigCoeffFlag

Therefore, with the spec text changes as shown in Table 2 or Table 3, the efforts for conformance testing can be reduced, which appears to be one of important benefits considering the current VVC development stage.

Experimental results

The proposed method has been implemented based on VTM 7.0 reference software according to common test conditions defined in JVET-N1010 [1].

CTC results

Conclusion

The current conditions for MTS index signaling cannot perfectly detect the cases that DCT-2 pair was certainly applied, which could cause design inconsistency with the current MTS zeroing-out scheme. With the proposed changes to fully detect the cases, design of MTS index signaling becomes consistent with the MTS zeroing-out, and a bitstream conformance constraint can be removed, which enforces MTS index to be signalled as zero if non-zero coefficients are found outside of top-left 16x16 region inside of which last non-zero coefficient lies. Coding gain and complexity are not effectively changed. It is recommended that the proposed fix should be adopted in next VTM and VVC WD.

References

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, 14th meeting, Geneva, CH, 19–27 March 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).