| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0138-v1 |
| --- | --- |

| Title: | Separate transform skip checking of Luma and Chroma for LFNST index signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Moonmo Koo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Mehdi Salehifar Seung-Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +1-858-635-5226 moonmo.koo@lge.com jaehyun.lim@lge.com mehdi.salehifar@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In the current VVC working draft, LFNST index signaling for both Luma and Chroma only depends on Luma transform skip flag. However, in the 16th JVET meeting, transform skip flag signaling of each color component was adopted so that the current dependency on Luma transform skip flag for Chroma components does not seem to be desirable considering separate tree structure. Furthermore, in VTM 7.0, it is controlled by each transform skip flag whether LFNST computation for each color component is performed or not. Therefore, LFNST index signalling condition should be modified according to separate transform skip flag for each color component, and the associated SW implementation should be aligned accordingly. In this contribution, two methods where Luma and Chroma depend only on their own transform skip flags in separate tree case are proposed. In the first method (Method 1), transform skip flag checking is absorbed in a variable, LfnstDcOnly, where,

- A Luma LFNST index can be signalled when Luma transform skip flag is equal to zero in Luma separate tree.

- A Chroma LFNST index can be signalled when any of Cb/Cr transform skip flags is equal to zero in Chroma separate tree.

- One LFNST index can be signalled when any of Y/Cb/Cr transform skip flags is equal to zero in single tree.

The second method (Method 2) employs different signaling for single tree in a way that LFNST index can be signalled when Luma transform skip flag is equal to zero, which is the same as in VTM 7.0, of which LFNST index signaling for separate tree is the same as that of Method 1.

The BD-rate/EncT/DecT changes for Method 1 are reportedly observed, which are 0.00%/100%/100% (AI), and 0.00%/100%/99% (RA), respectively.

The BD-rate/EncT/DecT changes for Method 2 are reportedly observed, which are 0.00%/101%/102% (AI), and 0.00%/100%/100% (RA), respectively.

Problem Statement

In the current spec text, LFNST index signaling is not performed when Luma is coded by transform skip as the following syntax table:

Table 1   The dependency on Luma transform skip flag for LFNST index signaling and the update of LfnstDcOnly variable

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { |  |
| --- | --- |
| …… |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && ( treeType != DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |
| …… |  |

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| …… |  |
| numSbCoeff = 1 << ( log2SbW + log2SbH ) |  |
| lastScanPos = numSbCoeff |  |
| lastSubBlock = ( 1 << ( log2TbWidth + log2TbHeight − ( log2SbW + log2SbH ) ) ) − 1 |  |
| do { |  |
| if( lastScanPos = = 0 ) { |  |
| lastScanPos = numSbCoeff |  |
| lastSubBlock− − |  |
| } |  |
| lastScanPos− − |  |
| xS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ lastSubBlock ][ 0 ] |  |
| yS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ lastSubBlock ][ 1 ] |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ lastScanPos ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ lastScanPos ][ 1 ] |  |
| } while( ( xC != LastSignificantCoeffX ) ¦ ¦ ( yC != LastSignificantCoeffY ) ) |  |
| if( lastSubBlock = = 0 && log2TbWidth >= 2 && log2TbHeight >= 2 && !transform_skip_flag[ x0 ][ y0 ][ cIdx ] && lastScanPos > 0 ) |  |
| LfnstDcOnly = 0 |  |
| …… |  |

The checking of Luma transform skip flag in Table 1 is problematic when Chroma LFNST index is signalled in separate tree, because Chroma CU is not aligned with Luma one and it is inconsistent with separate tree concept, i.e. parsing of Chroma LFNST index depends on that of Luma counterpart. Furthermore, the execution of LFNST computation in VTM 7.0 is controlled by the transform skip flag for each color component as the following:

void TrQuant::xInvLfnst( const TransformUnit &tu, const ComponentID compID ) { const CompArea& area = tu.blocks[ compID ]; const uint32_t width = area.width; const uint32_t height = area.height; const uint32_t lfnstIdx = tu.cu->lfnstIdx; #if JVET_P0058_CHROMA_TS if (lfnstIdx && tu.mtsIdx[compID] != MTS_SKIP && width >= 4 && height >= 4) #else if( lfnstIdx && tu.mtsIdx != MTS_SKIP && width >= 4 && height >= 4 ) #endif { …… } }

Figure 1  The checking of the transform skip flag for each color component in LFNST computation in VTM 7.0

According to Fig. 1, it seems to be reasonable that Chroma LFNST index signaling should rely on only Chroma transform skip flags, and the section 8.7.4.1 in VVC WD can be modified accordingly as in Fig. 2.

8.7.4. Transformation process for scaled transform coefficients 8.7.4.1 General Inputs to this process are: - a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture, - a variable nTbW specifying the width of the current transform block, - a variable nTbH specifying the height of the current transform block, - a variable cIdx specifying the colour component of the current block, - an (nTbW)x(nTbH) array d[ x ][ y ] of scaled transform coefficients with x = 0..nTbW − 1, y = 0..nTbH − 1. Output of this process is the (nTbW)x(nTbH) array res[ x ][ y ] of residual samples with x = 0..nTbW − 1, y = 0..nTbH − 1. When lfnst_idx is not equal to 0 and transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 0 and both nTbW and nTbH are greater than or equal to 4, the following applies: ……

Figure 2   The suggested spec text change checking the transform skip flag for each color component in the section regarding LFNST computation

As shown in Table 1, a variable, LfnstDcOnly, also takes care of transform skip flags properly, which means that only Luma (Chroma) transform skip flag(s) is (are) referred for Luma (Chroma) separate tree and both Luma and Chroma transform skip flags are referred for single tree. In fact, even without the explicit checking of transform_skip_flag[ x0 ][ y0 ][ cIdx ] in updating LfnstDcOnly, LfnstDcOnly can only be updated when the associated TBs are not coded by transform skip, because residual_ts_coding is invoked instead of residual_coding when each transform skip flag is equal to 1 (Table 4).  In summary, it is possible to fix the aforementioned problem by using LfnstDcOnly with removing or modifying the current Luma transform skip flag checking in coding_unit syntax table.

In this contribution, the following two methods are proposed to prevent Luma transform skip flag from being checked in Chroma separate tree.

Method 1

The first method is only removing the explicit Luma transform skip checking in Table 1 as shown in Table 2, which appears to be the simplest solution.

Table 2   The suggested Method 1 spec text change removing explicit Luma transform skip condition checking

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| …… |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && ( treeType = = DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |
| …… |  |

Absorbing transform skip conditions into LfnstDcOnly, a Luma (Chroma) LFNST index can be signalled when (any of) Luma (Chroma) transform skip flag(s) is equal to zero in Luma (Chroma) separate tree, and one LFNST index can be signalled when any of Y/Cb/Cr transform skip flags is equal to zero in single tree.

It should be noted that Luma transform skip flag must be zero (actually, inferred as 0) when ISP is applied, so that Luma LFNST index can be signalled with the relevant transform skip flag considered effectively, irrespective of LfnstDcOnly. Of course, Chroma transform skip flags are not associated with ISP.

Method 2

The second method is keeping LFNST index signaling for single tree the same as in VTM 7.0, because the current checking of only Luma transform skip is not problematic even if the ignorance of Chroma transform skip flags could be seen exceptional. The suggested spec text change for Method 2 is shown in Table 3.

Table 3 The suggested Method 2 spec text change keeping the same checking of transform skip flag for single tree case as in VTM 7.0

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| …… |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && ( treeType != SINGLE_TREE ¦ ¦ transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 ) && ( treeType = = DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |
| …… |  |
| } |  |

When Method 2 is applied in separate tree, checking of transform skip flags of Luma or Chroma exclusively is fulfilled via LfnstDcOnly as in the Method 1.

As a matter of fact, for both Method 1 and Method 2, the condition checking of transform_skip_flag of each component is not necessary in residual_coding, because residual_ts_coding where LfnstDcOnly is not updated is invoked instead of residual_coding when the transform_skip_flag is equal to 1. Therefore, as in Table 4, the condition, !transform_skip_flag[ x0 ][ y0 ][ cIdx ], for updating LfnstDcOnly, can be removed in residual_coding.

Table 4  The removal of transform_skip_flag checking in LfnstDcOnly update in residual_coding

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| …… |  |
| if( lastSubBlock = = 0 && log2TbWidth >= 2 && log2TbHeight >= 2 && !transform_skip_flag[ x0 ][ y0 ][ cIdx ] && lastScanPos > 0 ) |  |
| LfnstDcOnly = 0 |  |
| …… |  |

Experimental results

The proposed method has been implemented based on VTM 7.0 reference software according to common test conditions defined in JVET-N1010 [1].

Method 1

Method 2

Conclusion

As Chroma transform skip mode was adopted in the last meeting, LFNST index signaling should be adjusted accordingly in such a way that depends on only each transform skip flags for Luma or Chroma separate tree respectively. In this contribution, two methods using the existing variable, LfnstDcOnly, are proposed for remedying the reported problem, and the BD-rates for the two are effectively not changed relative to VTM 7.0. It is recommended that one of the proposed two methods should be adopted in the next VTM and VVC WD.

References

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, 14th meeting, Geneva, CH, 19–27 March 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).