| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0139-v2 |
| --- | --- |

| Title: | CE3-related: Lookup Table Free Rice Parameter Derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba Masaru Ikeda Yoichi Yagasaki Teruhiko Suzuki 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-50-3750-2779 Takeshi.Tsukuba@sony.com Masaru.Ikeda@sony.com Yoichi.Yagasaki@sony.com Teruhiko.S@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution proposes a lookup table free rice parameter derivation for regular residual coding and transform skip residual coding, where rice parameter is derived simple bit-shift operation.

It is reported that, at common QP condition, proposed method provides average BD-rate differences of (0.00 %, 0.01 %, -0.02 %) for AI, (0.00%, -0.01 %, 0.01%) for RA, and (-0.04%, -0.22 %, -0.15%) for LB; For ClassF, (-0.14%, -0.25%, -0.04%) for AI, (-0.10%, 0.04%, -0.07%) for RA, and (-0.10%, -0.50%, 0.57%) for LB; For ClassTGM, (-0.35%, -0.27%, -0.28%) for AI, (-0.17%, -0.20%, -0.15%) for RA, and (-0.17%, -0.20%, -0.26%) for LB.

It is reported that, at low QP condition, proposed method provides average BD-rate differences of (-0.04%, -0.05%, -0.08%) for AI, (0.xx%, 0.yy%, 0.zz%) for RA, and (0.02%, 0.05%, 0.06%) for LB; For ClassF, (-1.61%, -0.99%, -0.98%) for AI, (-2.02%, -0.89%, -0.96%) for RA, and (-0.62%, -0.58%, -0.73%) for LB; For ClassTGM, (-1.98%, -1.74%, -1.77%) for AI, (-2.47%, -1.54%, -1.53%) for RA, and (-1.51%, -1.27%, -1.26%) for LB.

It is reported that, at lossless condition, average bitrate differences of -5.20 % for AI, -3.57% for RA, and -2.54% for LB; For ClassF, -4.30% for AI, -3.25% for RA, and -3.02% for LB; For ClassTGM, -5.35% for AI, -6.90% for RA, -7.09% for LB.

Introduction

In WD [1] and VTM-7.0, rice parameter (cRiceParam) for abs_remainder and dec_abs_level in regular residual coding (RRC) is derived like eq. (A1) while rice parameter in transform skip residual coding (TSRC) is always set to 1.

cRiceParam = RiceParTable [ Clip3( 0, 31, locSumAbs − baseLevel * 5 )]      - (A1)

, where RiceParTable is defined as below:

RiceParTable = { 0,0,0,0,0,0,0,1,1,1,1,1,1,1,2,2,2,2,2,2,2,2,2,2,2,2,2,2,3,3,3,3 };

In CE3 on Lossless Coding, there are four proposals to improve transform skip residual coding as below:

| CE3-1.1 | JVET-P0559: Modified RRC and TSRC for lossless 3. modification to TSRC as discussed in JVET-P0559-m2 4. similar modification to TSRC as 3 without normalization |
| --- | --- |
| CE3-1.2 | JVET-P1028: Modified TSRC for lossless |
| CE3-1.5 | JVET-P0463/JVET-P0072: Rice parameter derivation of TSRC for lossless |

In CE3-1.1&CE3-1.2, following rice parameter lookup table for TSRC is newly introduced, which is different form RRC’s:

// CE3-1.1c/d

RiceParTable = { 0,0,0,0,1,1,1,1,1,1,1,2,2,2,2,2,2,2,2,2,2,2,3,3,3,3,3,3,3,3,3,3,4,5 };

// CE3-1.2

RiceParTable = { 0,0,0,0,1,1,1,1,1,1,1,2,2,2,2,2,2,2,2,2,2,2,3,3,3,3,3,3,3,3,3,3 };

In CE3-1.5, rice parameter lookup table for RRC is reused for TSRC.

// CE3-1.5

RiceParTable = { 0,0,0,0,0,0,0,1,1,1,1,1,1,1,2,2,2,2,2,2,2,2,2,2,2,2,2,2,3,3,3,3};

Figure 1-1 plots rice parameters proposed in each CE proposal.

Figure 1-1: rice parameters proposed in CE3-1.1c/d, CE3-1.2, and CE3-1.5

It is desirable to unify rice parameter lookup table between RRC and TSRC. Furthermore, lookup table free is more attractive.

Proposal

This contribution proposes a lookup table free rice parameter derivation for RRC and TSRC.

Basically, cRiceParam is derived using eq. (A2) after locSumAbs is obtained as below:

cRiceParam = min( max(0, locSumAbs + offset)>>3, 4 )                   - (A2)

, where offset is set to 1 for RRC, and 5 for TSRC.

Figure 2-1 plots rice parameters derived from eq. (A2). A lookup table for rice parameter can be completely removed and rice parameter range is extended to up to 4 comparing to VTM-7.0.

| - Rice parameters for RRC | (b) rice parameters for TSRC |
| --- | --- |

Figure 2-1: Proposed rice parameters

Proposed rice parameters in eq.(A2) can be also derived from lookup tables like using eq. (A3):

cRiceParam = RiceParTable [ Clip3( 0, 32, locSumAbs ) ]	- (A3)

// RRC

RiceParTable = { 0,0,0,0,0,0,0,1,1,1,1,1,1,1,1,2,2,2,2,2,2,2,2,3,3,3,3,3,3,3,3,4 }:

// TSRC

RiceParTable = { 0,0,0,1,1,1,1,1,1,1,1,2,2,2,2,2,2,2,2,3,3,3,3,3,3,3,3,4,4,4,4,4 };

Simulation Results

The proposed method was implemented on top of CE3-1.1c (CE3-1.1-subset3) and following simulations were conducted under CE test condition [3]:

- Test1:	VTM-7.0 vs Tested at common QP condition

- Test2: VTM-7.0 vs Tested at low QP condition

- Test3: VTM-7.0 vs Tested at lossless condition

Table 3-1 to 3-3 shows the results of each Test1, Test2 and Test3, respectively.

Table 3-1 Results of Test1 (Common QP condition)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.09% | 0.07% | 100% | 102% |
| Class A2 | 0.01% | 0.06% | 0.00% | 103% | 102% |
| Class B | 0.01% | 0.05% | -0.13% | 101% | 102% |
| Class C | -0.01% | 0.05% | 0.06% | 101% | 102% |
| Class E | 0.01% | -0.06% | -0.04% | 99% | 101% |
| Overall | 0.00% | 0.01% | -0.02% | 101% | 102% |
| Class D | -0.03% | 0.06% | -0.02% | 100% | 102% |
| Class F | -0.14% | -0.25% | -0.04% | 100% | 100% |
| Class TGM | -0.35% | -0.27% | -0.28% | 99% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.07% | 0.01% | 103% | 103% |
| Class A2 | -0.01% | 0.00% | -0.04% | 101% | 100% |
| Class B | 0.01% | 0.04% | 0.13% | 100% | 102% |
| Class C | 0.00% | -0.05% | -0.09% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | 0.01% | 101% | 102% |
| Class D | -0.02% | -0.19% | -0.06% | 102% | 106% |
| Class F | -0.10% | 0.04% | -0.07% | 101% | 102% |
| Class TGM | -0.17% | -0.20% | -0.15% | 101% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.42% | -0.03% | 100% | 100% |
| Class C | 0.01% | -0.38% | -0.29% | 102% | 103% |
| Class E | -0.17% | 0.33% | -0.15% | 99% | 99% |
| Overall | -0.04% | -0.22% | -0.15% | 100% | 101% |
| Class D | -0.04% | 0.03% | 0.03% | 100% | 103% |
| Class F | -0.10% | -0.50% | 0.57% | 101% | 101% |
| Class TGM | -0.17% | -0.20% | -0.26% | 98% | 101% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.03% | 0.03% | 100% | 100% |
| Class A2 | 0.10% | 0.09% | 0.09% | 100% | 101% |
| Class B | -0.04% | -0.03% | -0.07% | 102% | 103% |
| Class C | -0.02% | 0.01% | 0.02% | 102% | 100% |
| Class E | -0.26% | -0.41% | -0.49% | 104% | 106% |
| Overall | -0.04% | -0.05% | -0.08% | 102% | 102% |
| Class D | -0.15% | -0.18% | -0.16% | 101% | 102% |
| Class F | -1.61% | -0.99% | -0.98% | 102% | 107% |
| Class SCC | -1.98% | -1.74% | -1.77% | 104% | 103% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 | 0.06% | 0.08% | 0.08% | 100% | 100% |
| Class B | 0.01% | 0.02% | 0.01% | 101% | 103% |
| Class C | 0.02% | 0.06% | 0.04% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.04% | 0.03% | -0.07% | 101% | 103% |
| Class F | -2.02% | -0.89% | -0.96% | 101% | 102% |
| Class SCC | -2.47% | -1.54% | -1.53% | 102% | 104% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.04% | 0.05% | 103% | 103% |
| Class C | 0.05% | 0.09% | 0.12% | 102% | 101% |
| Class E | -0.03% | -0.01% | -0.02% | 102% | 102% |
| Overall | 0.02% | 0.05% | 0.06% | 102% | 102% |
| Class D | -0.02% | 0.11% | -0.03% | 101% | 97% |
| Class F | -0.62% | -0.58% | -0.73% | 102% | 100% |
| Class SCC | -1.51% | -1.27% | -1.26% | 100% | 100% |

Table 3-2 Results of Test3 (Lossless condition)

Conclusion

It is recommended to adopt proposed method into the next VTM/VVC WD.

Reference

[1] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 7), ” JVET-P2001, Oct. 2019

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Apr. 2019

[3] T. Nguyen, T. C. Ma, A. Nalci, “Description of Core Experiment 3 (CE3): Lossless Coding,” JVET-P2023, Oct. 2019

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Spec Changes

Proposed changes relative to JVET-P2001v14 is highlighted in yellow.

- 9.3.3.2 Rice parameter derivation process for abs_remainder[ ] and dec_abs_level[ ]

Inputs to this process are the base level baseLevel, the colour component index cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight.

Output of this process is the Rice parameter cRiceParam.

Given the array AbsLevel[ x ][ y ] for the transform block with component index cIdx and the top-left luma location ( x0, y0 ), the variable locSumAbs is derived as specified by the following pseudo code:

- If transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 1, the following applies:

locSumAbs = 0
noPos = 0
if( xC > 0 ){
	locSumAbs += AbsLevel[ xC − 1 ][ yC ]
  noPos += 1
if( yC > 0 ){			(149X)
	locSumAbs += AbsLevel[ xC ][ yC − 1 ]
  noPos +=1
}
locSumAbs = (noPos = = 1 ) ? 2*locSumAbs : locSumAbs

locSumAbs += (baseLevel==10) ? -8 ((baseLevel==0) ? 0 : -4)

- Otherwise (transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 0), the following applies:

locSumAbs = 0
if( xC < (1 << log2TbWidth) − 1 ) {
	locSumAbs += AbsLevel[ xC + 1 ][ yC ]
	if( xC < (1 << log2TbWidth) − 2 )
		locSumAbs += AbsLevel[ xC + 2 ][ yC ]
	if( yC < (1 << log2TbHeight) − 1 )
		locSumAbs += AbsLevel[ xC + 1 ][ yC + 1 ]	(1494)
}
if( yC < (1 << log2TbHeight) − 1 ) {
	locSumAbs += AbsLevel[ xC ][ yC + 1 ]
	if( yC < (1 << log2TbHeight) − 2 )
		locSumAbs += AbsLevel[ xC ][ yC + 2 ]
}

locSumAbs -= baseLevel * 5

locSumAbs = Clip3( 0, 31, locSumAbs − baseLevel * 5 )

Given the variable locSumAbs, the Rice parameter cRiceParam is derived as below:specified in Table 124.

cRiceParam = min( max(0, locSumAbs + offset)>>3, 4)

offset = transform_skip_flag[x0][y0][cIdx] ? 5 : 1	 	(149Y)

When baseLevel is equal to 0, the variable ZeroPos[ n ] is derived as follows:

ZeroPos[ n ] = ( QState < 2 ? 1 : 2 )  <<  cRiceParam	(1495)

Table 7-1 – Specification of cRiceParam based on locSumAbs

| locSumAbs | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cRiceParam | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 |
| locSumAbs | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| cRiceParam | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 |

- 9.3.3.11 Binarization process for abs_remainder[ ]

Input to this process is a request for a binarization for the syntax element abs_remainder[ n ], the colour component cIdx, the current sub-block index i, and the luma location ( x0, y0 ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight.

Output of this process is the binarization of the syntax element.

The variables lastAbsRemainder and lastRiceParam are derived as follows:

- If this process is invoked for the first time for the current sub-block index i, lastAbsRemainder and lastRiceParam are both set equal to 0.

- Otherwise (this process is not invoked for the first time for the current sub-block index i), lastAbsRemainder and lastRiceParam are set equal to the values of abs_remainder[ n ] and cRiceParam, respectively, that have been derived during the last invocation of the binarization process for the syntax element abs_remainder[ n ] as specified in this clause.

The rice parameter cRiceParam is derived as follows:

- If transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 1, the Rice parameter cRiceParam is set equal to 1.

- Otherwise, the rice parameter cRiceParam is derived by invoking the rice parameter derivation process for abs_remainder[] as specified in clause 9.3.3.2 with the variable baseLevel set equal to 4, the colour component index cIdx, the luma location ( x0, y0 ), the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight as inputs.

The variable cMax is derived from cRiceParam as:

cMax = 6  <<  cRiceParam		(1503)

The binarization of the syntax element abs_remainder[ n ] is a concatenation of a prefix bin string and (when present) a suffix bin string.

For the derivation of the prefix bin string, the following applies:

- The prefix value of abs_remainder[ n ], prefixVal, is derived as follows:

prefixVal = Min( cMax, abs_remainder[ n ] )		(1504)

- The prefix bin string is specified by invoking the TR binarization process as specified in clause 9.3.3.3 for prefixVal with the variables cMax and cRiceParam as inputs.

When the prefix bin string is equal to the bit string of length 6 with all bits equal to 1, the suffix bin string is present and it is derived as follows:

- The suffix value of abs_remainder[ n ], suffixVal, is derived as follows:

suffixVal = abs_remainder[ n ] − cMax		(1505)

- The suffix bin string is specified by invoking the limited k-th order EGk binarization process as specified in clause 9.3.3.6 for the binarization of suffixVal with the Exp-Golomb order k set equal to cRiceParam + 1, variable cRiceParam, variable log2TransformRange set equal to 15 and variable maxPreExtLen set equal to 11 as input.

- 9.3.3.12 Binarization process for dec_abs_level[ ]

Input to this process is a request for a binarization of the syntax element dec_abs_level[ n ], the colour component cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left luma sample of the picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight.

Output of this process is the binarization of the syntax element.

The rice parameter cRiceParam is derived by invoking the rice parameter derivation process for dec_abs_level[] as specified in clause 9.3.3.2 with the variable baseLevel set equal to 0, the colour component index cIdx, the luma location ( x0, y0 ), the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight as inputs.

The variable cMax is derived from cRiceParam as:

cMax = 6  <<  cRiceParam		(1506)

The binarization of dec_abs_level[ n ] is a concatenation of a prefix bin string and (when present) a suffix bin string.

For the derivation of the prefix bin string, the following applies:

- The prefix value of dec_abs_level[ n ], prefixVal, is derived as follows:

prefixVal = Min( cMax, dec_abs_level[ n ] )		(1507)

- The prefix bin string is specified by invoking the TR binarization process as specified in clause 9.3.3.3 for prefixVal with the variables cMax and cRiceParam as inputs.

When the prefix bin string is equal to the bit string of length 6 with all bits equal to 1, the suffix bin string is present and it is derived as follows:

- The suffix value of dec_abs_level[ n ], suffixVal, is derived as follows:

suffixVal = dec_abs_level[ n ] − cMax		(1508)

- The suffix bin string is specified by invoking the limited k-th order EGk binarization process as specified in clause 9.3.3.6 for the binarization of suffixVal with the Exp-Golomb order k set equal to cRiceParam + 1, variable cRiceParam, variable log2TransformRange set equal to 15 and variable maxPreExtLen set equal to 11  as input.