| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0141 |
| --- | --- |

| Title: | Non-CE1/Non-CE3: Deblocking filter control for lossless blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kyohei Unno Kei Kawamura Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-70-4560-2752 ky-unno@kddi-research.jp ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

In this contribution, it is proposed that disabling deblocking filter for inside a TU when transform skip is applied to the TU and Qp′ of the TU is less than or equal to 4. In terms of deblocking filter control, it can be guaranteed that the TU is completely lossless by the proposed method. This functionality is supported by HEVC RExt.

Introduction

In the 16th meeting at Geneva, it was discussed the way to realize lossless coding in VVC. Consequently, lossy tools are controlled by not some kind of “lossless flag” but flags of each tool in VVC D7 [1]. However, deblocking filter doesn’t have any disabling flags at CU or TU level. Therefore, if in case only some regions of a picture are lossless coded, boundaries inside a lossless block can be deblocked depending on the QP value of a neighboring block. That means, it is not guaranteed completely lossless in a lossless block. On the other hand, HEVC RExt has cu_transquant_bypass_flag that realizes lossless coding at CU level. Deblocking filter is not applied inside a CU in the case of cu_transquant_bypass_flag=1. In other words, VVC D7 doesn't have functionality that realizing lossless for block-level in contrast with HEVC RExt has the functionality.

Proposed Method

It is proposed that if a TU is “lossless block”, deblocking filter is not applied inside the TU. More precisely, when transform skip is applied to a TU and Qp′ of the TU is less than or equal to 4, deblocking filter is not applied to pixels in the TU.

An example of the proposed text changes is shown below. It is the same way for palette mode in VVC D7 and deblocking in HEVC RExt.

##### 8.8.3.6.6 Filtering process for a luma sample using short filters

Inputs to this process are:

- the variables maxFilterLengthP and maxFilterLengthQ,

- the sample values pi and qj with i = 0..maxFilterLengthP and j = 0..maxFilterLengthQ,

- a variable dE,

- the variables dEp and dEq containing decisions to filter samples p1 and q1, respectively,

- a variable tC.

Outputs of this process are:

- the number of filtered samples nDp and nDq,

- the filtered sample values pi′ and qj′ with i = 0..nDp − 1, j = 0..nDq − 1.

Depending on the value of dE, the following applies:

- If the variable dE is equal to 2, nDp and nDq are both set equal to 3 and the following strong filtering applies:

p0′ = Clip3( p0 − 3 * tC, p0 + 3 * tC, ( p2 + 2 * p1 + 2 * p0 + 2 * q0 + q1 + 4 )  >>  3 )	(1350)

p1′ = Clip3( p1 − 2 * tC, p1 + 2 * tC, ( p2 + p1 + p0 + q0 + 2 )  >>  2 )	(1351)

p2′ = Clip3( p2 − 1 * tC, p2 + 1*tC, ( 2 * p3 + 3 * p2 + p1 + p0 + q0 + 4 )  >>  3 )	(1352)

q0′ = Clip3( q0 − 3 * tC, q0 + 3 * tC, ( p1 + 2 * p0 + 2 * q0 + 2 * q1 + q2 + 4 )  >>  3 )	(1353)

q1′ = Clip3( q1 − 2 * tC, q1 + 2 * tC, ( p0 + q0 + q1 + q2 + 2 )  >>  2 )	(1354)

q2′= Clip3( q2 − 1 * tC, q2 + 1 * tC, ( p0 + q0 + q1 + 3 * q2 + 2 * q3 + 4 )  >>  3 )	(1355)

- Otherwise, nDp and nDq are set both equal to 0 and the following weak filtering applies:

  - The following applies:

= ( 9 * ( q0 −  p0 ) − 3 * ( q1 − p1 ) + 8 )  >>  4	(1356)

  - When Abs() is less than tC * 10, the following ordered steps apply:

    - The filtered sample values p0′ and q0′ are specified as follows:

= Clip3( −tC, tC,  )		(1357)

p0′ = Clip1( p0 +  )		(1358)

q0′ = Clip1( q0 −  )		(1359)

    - When dEp is equal to 1, the filtered sample value p1′ is specified as follows:

p = Clip3( −( tC  >>  1 ), tC  >>  1, ( ( ( p2 + p0 + 1 )  >>  1 ) − p1 +  )  >>  1 )	(1360)

p1′ = Clip1( p1 + p )		(1361)

    - When dEq is equal to 1, the filtered sample value q1′ is specified as follows:

q = Clip3( −( tC  >>  1 ), tC  >>  1, ( ( ( q2 + q0 + 1 )  >>  1 ) − q1 −  )  >>  1 )	(1362)

q1′ = Clip1( q1 + q )		(1363)

    - nDp is set equal to dEp + 1 and nDq is set equal to dEq + 1.

When nDp is greater than 0 and pred_mode_plt_flag of the coding unit that includes the coding block containing the sample p0 is equal to 1, nDp is set equal to 0.

When nDq is greater than 0 and pred_mode_plt_flag of the coding unit that includes the coding block containing the sample q0 is equal to 1, nDq is set equal to 0.

When all of the following conditions are ture, nDp is set equal to 0:

- nDp is greater than 0,

- transform_skip_flag of the transform unit that includes the transform block containing the sample p0 is equal to 1,

- Qp′Y that includes the transform block containing the sample p0 is less than or equal to 4.

When all of the following conditions are ture, nDq is set equal to 0:

- nDq is greater than 0,

- transform_skip_flag of the transform unit that includes the transform block containing the sample q0 is equal to 1,

- Qp′Y that includes the transform block containing the sample q0 is less than or equal to 4.

Conclusion

In this contribution, it was proposed that disabling deblocking filter for inside a TU when transform skip was applied the TU, and Qp′ of the TU was less than or equal to 4. In terms of deblocking filter control, it was guaranteed that the TU is completely lossless. It is recommended that the proposed method is adopted to next VVC Draft and VTM.

This work was conducted under the research project of Ministry of Internal Affairs and Communication.

References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 7)”, JVET-P2001, Geneva, CH, Oct. 2019.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).