| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, 6–17 January 2020 | Document: JVET-Q0166 |
| --- | --- |

| Title: | On the adaptive color transform |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Adarsh Krishnan Ramasubramonian, Chun-Chi Chen, Vadim Seregin, Ted Hsieh, Yung-Hsuan Chao, Wei-Jung Chien, Geert Van der Auwera, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA 92121 | Email: | hanhuang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In the 16th JVET meeting, the adaptive colour transform (ACT) for 444 video coding was adopted in VVC. It is asserted that the following issues are present in the current ACT design:

- The quantization parameter (QP) after adding ACT offset may be out of QP range.

- The chroma ACT QP offset may be added together with regular chroma QP offset.

- The chroma QP used in deblocking may be different from quantization stage due to the ACT offset, which violates the adoption of JVET-P1001.

- The inverse ACT increases the dynamic range of reconstructed residual that may exceed 16 bits.

- The ACT QP offsets are fixed instead of signalled in high level syntax as in HEVC.

- The Cb or Cr ACT QP offset may be applied in JointCbCr modes.

It’s proposed to address the above issues by the following:1) add ACT QP offset before clipping of QP; 2) CU-level switching of QP offset values for reconstructing residuals according to the ACT CU-level flag as in HEVC; 3) align the chroma QP at the deblocking stage to that at the dequantization stage as in adopted JVET-P1001; 4) clip the output of inverse ACT to 16 bits dynamic range. 5) signal ACT QP offsets as in HEVC; 6) use a separate chroma ACT QP offset for JointCbCr mode 2 to align with chroma QP offset in non-ACT mode.

# Introduction

In JVET-P0517 [1], the adaptive color transform (ACT) tool in HEVC was implemented into the VVC framework and it was adopted in the 16th JVET meeting. The color space conversion in JVET-P0517 is carried out in the residual domain. Two additional changes were proposed for implementation in VVC: 1) move the ACT enabling/disabling flag from TU-level to CU-level; 2) disable the ACT when the separate-tree partition or intra sub-partition prediction (ISP) mode is applied.

The forward and inverse ACT are described as follows:

| C0'C1'C2'=2112-1-10-22C0C1C2/4YCoCg=12120-2-12-1RGBYCoCg=12120-2-12-1RGB C0C1C2=1101-1-11-11C0'C1'C2' | (1) |
| --- | --- |

Fixed QP adjustments of (-5, -5, -3) are applied to the transform residuals if ACT is applied.

# Problem statement and proposed fixes

The following issues are identified in the ACT of current VVC:

- The quantization parameter (QP) after adding ACT offset may be out of QP range.

- The chroma ACT QP offset may be added together with regular chroma QP offset.

- The chroma QP used in deblocking may be different from quantization stage due to the ACT offset, which violates the adoption of JVET-P1001.

- The inverse ACT increases the dynamic range of reconstructed residual that may exceed 16 bits.

- The ACT QP offsets are fixed instead of signalled in high level syntax as in HEVC.

- The Cb or Cr ACT QP offset may be applied to JointCbCr mode if ACT is applied.

Item 1) to 3) are due to the fact that the ACT offset is added after the QP derivation in “QpParam” in the current implementation. But the ACT offset is not added in the deblocking process.

In this contribution, it’s proposed to

- Add ACT QP offset before clipping of QP, i.e., move the ACT offset inside “QpParam”;

- CU-level switching of QP offset values for reconstructing residuals according to the ACT CU-level flag as in HEVC; (In the JCT-VC T meeting, “it is pointed out that the effect of QP offset is different when YCoCg is used”. Therefore, different QP offset is used depending on ACT setting in HEVC.)

- Align the chroma QP at the deblocking stage to that at the dequantization stage as in as in adopted JVET-P1001;

- Clip the output of inverse ACT to 16 bits dynamic range.

- Signal ACT QP offset as in HEVC;

- Use a separate chroma ACT QP offset for JointCbCr mode 2 as in regular chroma QP offset.

# Test Results

Two tests were performed on top of VTM-7.1. Test 1 is conducted by applying the proposed items 1) to item 5). Test 2 is conducted by applying all the proposed items.  In Test 1, the default ACT offsets are the same as in VTM-7.1. In Test 2, the default ACT offset for the JointCbCr mode 2 is -5. The BD-rate performances for dual-tree off condition are shown in the following tables.

Table 1 RGB results of Test 1

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | -0.3% | -0.2% |
| RGB, mixed content, 1440p & 1080p | 0.0% | -0.7% | -0.5% |
| RGB, Animation, 720p | 0.0% | -1.5% | -1.0% |
| RGB, camera captured, 1080p | -0.1% | -0.5% | -0.4% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 104% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | -0.4% | -0.3% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -1.0% | -0.6% |
| RGB, Animation, 720p | -0.1% | -1.3% | -0.7% |
| RGB, camera captured, 1080p | 0.1% | -0.3% | -0.2% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | -0.3% | -0.4% |
| RGB, mixed content, 1440p & 1080p | -0.2% | -0.9% | -0.7% |
| RGB, Animation, 720p | -0.1% | -0.6% | -0.5% |
| RGB, camera captured, 1080p | -0.1% | 0.0% | -0.4% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 102% |  |  |

Table 2 YUV444 results of Test 1

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | G/Y |  | B/U |  | R/V |
| YUV444, text & graphics with motion, 1080p & 720p | 0.0% |  | 0.0% |  | 0.0% |
| YUV444, mixed content, 1440p & 1080p | 0.0% |  | 0.0% |  | 0.0% |
| YUV444, Animation, 720p | 0.0% |  | 0.0% |  | 0.0% |
| YUV444, camera captured, 1080p | 0.0% |  | 0.0% |  | 0.0% |
| Enc Time[%] | 95% |  |  |  |  |
| Dec Time[%] | 101% |  |  |  |  |
|  | Random Access |  |  |  |  |
|  | G/Y |  | B/U |  | R/V |
| YUV444, text & graphics with motion, 1080p & 720p | 0.0% |  | 0.0% |  | 0.0% |
| YUV444, mixed content, 1440p & 1080p | 0.0% |  | 0.0% |  | 0.1% |
| YUV444, Animation, 720p | 0.0% |  | 0.0% |  | 0.0% |
| YUV444, camera captured, 1080p | 0.0% |  | 0.0% |  | 0.0% |
| Enc Time[%] | 99% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |
|  | Low delay B |  |  |  |  |
|  | G/Y |  | B/U |  | R/V |
| YUV444, text & graphics with motion, 1080p & 720p | 0.0% |  | -0.2% |  | -0.3% |
| YUV444, mixed content, 1440p & 1080p | 0.0% |  | -0.1% |  | -0.1% |
| YUV444, Animation, 720p | 0.1% |  | 0.1% |  | -0.1% |
| YUV444, camera captured, 1080p | 0.1% |  | 0.0% |  | 0.1% |
| Enc Time[%] | 100% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |
|  | All Intra |  |  |  |  |
|  | G/Y | B/U |  | R/V |  |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | -0.3% |  | -0.2% |  |
| RGB, mixed content, 1440p & 1080p | 0.0% | -0.7% |  | -0.5% |  |
| RGB, Animation, 720p | 0.0% | -1.5% |  | -1.0% |  |
| RGB, camera captured, 1080p | -0.1% | -0.5% |  | -0.4% |  |
| Enc Time[%] | 104% |  |  |  |  |
| Dec Time[%] | 94% |  |  |  |  |

Table 3 RGB results of Test 2

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | -0.3% | -0.2% |
| RGB, mixed content, 1440p & 1080p | 0.0% | -0.7% | -0.5% |
| RGB, Animation, 720p | 0.0% | -1.5% | -1.0% |
| RGB, camera captured, 1080p | -0.1% | -0.5% | -0.4% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 94% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | -0.4% | -0.3% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -1.0% | -0.6% |
| RGB, Animation, 720p | -0.1% | -1.4% | -0.7% |
| RGB, camera captured, 1080p | 0.0% | -0.3% | -0.2% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 91% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p |  |  |  |
| RGB, mixed content, 1440p & 1080p | -0.2% | -0.9% | -0.7% |
| RGB, Animation, 720p | -0.1% | -0.7% | -0.5% |
| RGB, camera captured, 1080p |  |  |  |
| Enc Time[%] |  |  |  |
| Dec Time[%] |  |  |  |

Table 4 YUV444 results of Test 2

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| YUV444, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV444, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV444, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV444, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 93% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| YUV444, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV444, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.1% |
| YUV444, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV444, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 91% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| YUV444, text & graphics with motion, 1080p & 720p |  |  |  |
| YUV444, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% |
| YUV444, Animation, 720p | 0.1% | 0.0% | 0.0% |
| YUV444, camera captured, 1080p |  |  |  |
| Enc Time[%] |  |  |  |
| Dec Time[%] |  |  |  |

# Conclusion

In this contribution, several fixes for the current ACT in VVC are presented. It’s proposed to adopt the fixes to the next version of VVC draft.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang, “Support of adaptive color transform for 444 video coding in VVC”, JVET-P0517, 16th Meeting: Geneva, CH, 1–11 October 2019