| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0183-v1 |
| --- | --- |

| Title: | AHG9: High-level syntax related to transform skip mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {shih-ta.hsiang, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes three high-level syntax modifications related to transform skip mode, summarized as follows:

- Skip coding min_qp_prime_ts_minus4 when sps_transform_skip_enabled_flag is equal to 0;

- Add a separate SPS syntax flag sps_transform_skip_chroma_enabled_flag to control whether to enable or disable transform skip mode for chroma;

- Move log2_transform_skip_max_size_minus2 from the PPS to the SPS.

# Introduction

The related parts (highlighted in blue) of the syntax tables to transform skip mode in VVC Draft 7 [1] are provided below:

#### 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| … |  |
| bit_depth_minus8 | ue(v) |
| min_qp_prime_ts_minus4 | ue(v) |
| … |  |
| sps_alf_enabled_flag | u(1) |
| sps_transform_skip_enabled_flag | u(1) |
| if( sps_transform_skip_enabled_flag ) |  |
| sps_bdpcm_enabled_flag | u(1) |
| if( sps_bdpcm_enabled_flag && chroma_format_idc = = 3 ) |  |
| sps_bdpcm_chroma_enabled_flag | u(1) |
| … |  |
| } |  |

#### 7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | u(4) |
| … |  |
| log2_transform_skip_max_size_minus2 | ue(v) |
| cu_qp_delta_enabled_flag | u(1) |
| … |  |
| } |  |

#### 7.3.9.10 Transform unit syntax

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| … |  |
| if( tu_cbf_luma[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 0 ] && tbWidth <= MaxTsSize && tbHeight <= MaxTsSize && ( IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT ) && !cu_sbt_flag ) |  |
| transform_skip_flag[ x0 ][ y0 ][ 0 ] | ae(v) |
| if( !transform_skip_flag[ x0 ][ y0 ][ 0 ] ) |  |
| residual_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| else |  |
| residual_ts_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| } |  |
| if( tu_cbf_cb[ xC ][ yC ] && treeType != DUAL_TREE_LUMA ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 1 ] && wC <= MaxTsSize && hC <= MaxTsSize && !cu_sbt_flag ) |  |
| transform_skip_flag[ xC ][ yC ][ 1 ] | ae(v) |
| if( !transform_skip_flag[ xC ][ yC ][ 1 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| else |  |
| residual_ts_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| } |  |
| if( tu_cbf_cr[ xC ][ yC ] && treeType != DUAL_TREE_LUMA && !( tu_cbf_cb[ xC ][ yC ] && tu_joint_cbcr_residual_flag[ xC ][ yC ] ) ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 2 ] && wC <= MaxTsSize && hC <= MaxTsSize && !cu_sbt_flag ) |  |
| transform_skip_flag[ xC ][ yC ][ 2 ] | ae(v) |
| if( !transform_skip_flag[ xC ][ yC ][ 2 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| else |  |
| residual_ts_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| } |  |
| … |  |
| } |  |

# Proposed modifications

## Modification 1 on min_qp_prime_ts_minus4

In VVC Draft 7, the syntax element min_qp_prime_ts_minus4 specifies the minimum allowed quantization parameter for transform skip mode and is always signalled in the SPS. However, min_qp_prime_ts_minus4 is only applicable to residual blocks coded in the transform skip mode. In the proposed modification, min_qp_prime_ts_minus4 is signalled after the syntax element sps_transform_skip_enabled_flag in the parsing order in the SPS. min_qp_prime_ts_minus4 is skipped when sps_transform_skip_enabled_flag is equal to 0.

## Modification 2 on enabling transform skip mode for chroma

In VVC Draft 7, setting the syntax element sps_transform_skip_enabled_flag equal to 1 in the SPS enables transform skip mode for chorma components. However, under the common test conditions (CTCs) for coding test sequences in 4:2:0, transform skip mode for chorma is not enabled at encoder (with the encoder configuration option ChromaTS=0) and the coded syntax value for transform_skip_flag is always equal to 0 for a chorma residual block.

In the proposed modification, a separate syntax element sps_transform_skip_chroma_enabled_flag is added to indicate whether transform skip mode is enabled for chroma components, corresponding to the optional high-level flag proposed in [2]. In this way, the modified syntax can support to disable the TSM for chroma components while still enabling the TSM for luma components. One context-coded bin can thus be saved for parsing transform_skip_flag associated with a chroma residual block. The modified high-level syntax for the TSM is also consistent with the latest high-level syntax for BDPCM in VVC Draft 7, where the two separate on/off control flags (sps_bdpcm_enabled_flag and sps_bdpcm_chroma_enabled_flag) are coded for luma and chroma, respectively, in the SPS.

## Modification 3 on log2_transform_skip_max_size_minus2

In VVC Draft 7, the syntax element log2_transform_skip_max_size_minus2 specifying the maximum block size used for transform skip is signalled in the PPS. In the proposed modification, log2_transform_skip_max_size_minus2 is moved to the SPS such that the high-level syntax elements related to transform skip mode are all signalled in one high-level syntax set (SPS). When sps_transform_skip_enabled_flag is equal to 0, coding log2_transform_skip_max_size_minus2 can be skipped and further save one bit.

## Modified syntax tables

The modified parts (further highlighted in magenta) of the syntax tables based on VVC Draft 7 [1] are provided below:

#### 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| … |  |
| bit_depth_minus8 | ue(v) |
| min_qp_prime_ts_minus4 | ue(v) |
| … |  |
| sps_alf_enabled_flag | u(1) |
| sps_transform_skip_enabled_flag | u(1) |
| if( sps_transform_skip_enabled_flag ) { |  |
| min_qp_prime_ts_minus4 | ue(v) |
| if( ChromaArrayType != 0 ) |  |
| sps_transform_skip_chroma_enabled_flag | u(1) |
| log2_transform_skip_max_size_minus2 | ue(v) |
| sps_bdpcm_enabled_flag | u(1) |
| if( sps_bdpcm_enabled_flag && chroma_format_idc = = 3 && sps_transform_skip_chroma_enabled_flag ) |  |
| sps_bdpcm_chroma_enabled_flag | u(1) |
| } |  |
| … |  |
| } |  |

#### 7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | u(4) |
| … |  |
| log2_transform_skip_max_size_minus2 | ue(v) |
| cu_qp_delta_enabled_flag | u(1) |
| … |  |
| } |  |

#### 7.3.9.10 Transform unit syntax

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| … |  |
| if( tu_cbf_luma[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 0 ] && tbWidth <= MaxTsSize && tbHeight <= MaxTsSize && ( IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT ) && !cu_sbt_flag ) |  |
| transform_skip_flag[ x0 ][ y0 ][ 0 ] | ae(v) |
| if( !transform_skip_flag[ x0 ][ y0 ][ 0 ] ) |  |
| residual_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| else |  |
| residual_ts_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| } |  |
| if( tu_cbf_cb[ xC ][ yC ] && treeType != DUAL_TREE_LUMA ) { |  |
| if( sps_transform_skip_chroma_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 1 ] && wC <= MaxTsSize && hC <= MaxTsSize && !cu_sbt_flag ) |  |
| transform_skip_flag[ xC ][ yC ][ 1 ] | ae(v) |
| if( !transform_skip_flag[ xC ][ yC ][ 1 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| else |  |
| residual_ts_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| } |  |
| if( tu_cbf_cr[ xC ][ yC ] && treeType != DUAL_TREE_LUMA && !( tu_cbf_cb[ xC ][ yC ] && tu_joint_cbcr_residual_flag[ xC ][ yC ] ) ) { |  |
| if( sps_transform_skip_chroma_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 2 ] && wC <= MaxTsSize && hC <= MaxTsSize && !cu_sbt_flag ) |  |
| transform_skip_flag[ xC ][ yC ][ 2 ] | ae(v) |
| if( !transform_skip_flag[ xC ][ yC ][ 2 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| else |  |
| residual_ts_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| } |  |
| … |  |
| } |  |

## Modified semantics

#### 7.4.3.3 Sequence parameter set RBSP semantics

sps_transform_skip_enabled_flag equa to 1 specifies that transform_skip_flag associated with a luma transform block in the transform unit syntax may be present. sps_transform_skip_enabled_flag equal to 0 specifies that transform_skip_flag is not present in the transform unit syntax.

sps_transform_skip_chroma_enabled_flag equa to 1 specifies that transform_skip_flag associated with a chroma transform block may be present in the transform unit syntax. sps_transform_skip_chroma_enabled_flag equal to 0 specifies that transform_skip_flag is not present in the transform unit syntax.

log2_transform_skip_max_size_minus2 plus 2 specifies the maximum block size used for transform skip, and shall be in the range of 0 to 3, inclusive.

The variable MaxTsSize is set equal to 1  <<  ( log2_transform_skip_max_size_minus2 + 2 ).

#### 7.4.3.4 Picture parameter set RBSP semantics

log2_transform_skip_max_size_minus2 specifies the maximum block size used for transform skip, and shall be in the range of 0 to 3, inclusive.

The variable MaxTsSize is set equal to 1  <<  ( log2_transform_skip_max_size_minus2 + 2 ).

# Conclusion

In this contribution, three modifications are proposed for improving high-level syntax related to transform skip mode. In the 1st proposed modification, when sps_transform_skip_enabled_flag is equal to 0, coding min_qp_prime_ts_minus4 is skipped to avoid signalling an irrelevant syntax value and save some high-level syntax bit cost. In the 2nd proposed modification, a separate control flag is introduced thus to support disabling transform skip mode for chroma while still enabling transform skip mode for luma. It can save context-coded bins for signaling transform_skip_flag associated with chroma residual blocks under the CTCs for coding 4:2:0 test videos. The proposed high-level syntax for transform skip mode is also consistent with the latest HLS syntax for BDPCM using separate on/off control flags for luma and chroma, respectively. The 3rd proposed modification moves log2_transform_skip_max_size_minus2 from the PPS to the SPS to group all TSM-related high-level syntax elements in the SPS.

# References

- B. Bross, J. Chen, and S. Liu “Versatile Video Coding (Draft 7),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-P2001, 16th Meeting: Geneva, CH, 1–11 October 2019.

- T. Tsukuba, M. Ikeda, Y. Yagasaki, and T. Suzuki, “CE8-2.1: Transform skip for chroma with limiting maximum number of context-coded bin in TS residual coding,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-P0058, 16th Meeting: Geneva, CH, 1–11 October 2019.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).