| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0192-v2 |
| --- | --- |

| Title: | Chroma wide angle intra prediction mode mapping for 4:2:2 format |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chia-Ming Tsai, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chia-ming.tsai, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

This contribution proposes a bug fix for the 4:2:2 chroma intra mode mapping of wide angle intra prediction (WAIP) modes. In HEVC-RExt, 4:2:2 chroma intra mode mapping is the final step in deriving chroma intra angular prediction mode. However, in VVC Draft 7, 4:2:2 chroma intra mode mapping is not the final step in deriving chroma intra angular prediction mode. The 4:2:2 chroma intra mode mapping is applied before the WAIP mode mapping process, and therefore not every WAIP mode is mapped to the correct angular mode. In the proposed method, the 4:2:2 chroma intra mode mapping is moved after the WAIP mode mapping process, and the mapping table is also extended for supporting WAIP intra mode mapping.

Compared with VTM7.0, results of 4:2:2 test sequences using All Intra (AI) and Random Access (RA) configurations are reported as follows:

VTM7.0-AI: {Y-BD-rate = 0.00%, EncT = 99%, DecT = 96%}
VTM7.0-RA: {Y-BD-rate = 0.00%, EncT = 99%, DecT = 100%}

It is shown that the bug causes very minor impact on BD-rates. However, it is asserted that fixing the bug is good for VVC.

Introduction

In VVC Draft 7 [1], the final intra angular prediction mode for a chroma coding block (CB) is derived in the following steps:

- Parse the signalled chroma intra prediction mode.

- If the signalled chroma intra prediction mode is a direct mode (DM), the current chroma intra prediction mode is set to the corresponding signalled luma intra prediction mode. Otherwise, the current chroma intra prediction mode is set to the signalled chroma intra prediction mode.

- If the colour format of the current picture is 4:2:2, the current chroma intra prediction mode is converted by a chroma intra mode mapping table as specified in Table 1.

- Apply the WAIP mode mapping process to the current chroma intra prediction mode.

- The final intra angular prediction mode is set to the current chroma intra prediction mode.

However, in the 3rd step, the chroma intra mode mapping process uses the intra mode before the WAIP mode mapping process. After the WAIP mode mapping process, not every WAIP mode is mapped to the correct angular mode as highlighted in Table 2.

Table 1. 4:2:2 chroma intra mapping process from mode X to mode Y in VVC Draft 7

| mode X | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mode Y | 0 | 1 | 61 | 62 | 63 | 64 | 65 | 66 | 2 | 3 | 5 | 6 | 8 | 10 | 12 | 13 | 14 | 16 |
| mode X | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 |
| mode Y | 18 | 20 | 22 | 23 | 24 | 26 | 28 | 30 | 31 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 |
| mode X | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 |
| mode Y | 41 | 42 | 43 | 43 | 44 | 44 | 45 | 45 | 46 | 47 | 48 | 48 | 49 | 49 | 50 | 51 | 51 | 52 |
| mode X | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 | 64 | 65 | 66 |  |  |  |  |  |
| mode Y | 52 | 53 | 54 | 55 | 55 | 56 | 56 | 57 | 57 | 58 | 59 | 59 | 60 |  |  |  |  |  |

Table 2. In VVC Draft 7, highlighted WAIP modes are not mapped to the correct angular modes by the aspect in JVET-P0111 [2].

| WAIP modes | Without subsampling |  | With 4:2:2 format |  | Mapping |  | In VVC Draft 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | intraPredAngle (height) | Angle | intraPredAngle*2 | Angle | Closest Angle | Mapped Mode |  |
| -14 | 512 | 86.4237 | 1024 | 88.2101 | 86.4237 | -14 | -15 |
| -13 | 341 | 84.6390 | 682 | 87.3136 | 86.4237 | -14 | -15 |
| -12 | 256 | 82.8750 | 512 | 86.4237 | 86.4237 | -14 |  |
| -11 | 171 | 79.4006 | 342 | 84.6546 | 84.6390 | -13 |  |
| -10 | 128 | 75.9638 | 256 | 82.8750 | 82.8750 | -12 |  |
| -9 | 102 | 72.5820 | 204 | 81.0851 | 79.4006 | -11 | -12 |
| -8 | 86 | 69.5901 | 172 | 79.4608 | 79.4006 | -11 |  |
| -7 | 73 | 66.3295 | 146 | 77.6375 | 75.9638 | -10 | -11 |
| -6 | 64 | 63.4349 | 128 | 75.9638 | 75.9638 | -10 |  |
| -5 | 57 | 60.6900 | 114 | 74.3205 | 75.9638 | -10 |  |
| -4 | 51 | 57.8937 | 102 | 72.5820 | 72.5820 | -9 |  |
| -3 | 45 | 54.5829 | 90 | 70.4269 | 69.5901 | -8 |  |
| -2 | 39 | 50.6307 | 78 | 67.6938 | 66.3295 | -7 | -8 |
| -1 | 35 | 47.5638 | 70 | 65.4328 | 66.3295 | -7 |  |
|  | intraPredAngle (width) | Angle | intraPredAngle/2 | Angle | Closest Angle | Mapped Mode |  |
| 67 | 35 | 47.5638 | 17.5 | 28.6731 | 29.3578 | 61 |  |
| 68 | 39 | 50.6307 | 19.5 | 31.3571 | 32.0054 | 62 |  |
| 69 | 45 | 54.5829 | 22.5 | 35.1120 | 35.7067 | 63 |  |
| 70 | 51 | 57.8937 | 25.5 | 38.5505 | 39.0939 | 64 |  |
| 71 | 57 | 60.6900 | 28.5 | 41.6891 | 42.1844 | 65 |  |
| 72 | 64 | 63.4349 | 32 | 45.0000 | 45.0000 | 66 |  |
| 73 | 73 | 66.3295 | 36.5 | 48.7586 | 47.5638 | 67 |  |
| 74 | 86 | 69.5901 | 43 | 53.3439 | 54.5829 | 69 | 68 |
| 75 | 102 | 72.5820 | 51 | 57.8937 | 57.8937 | 70 |  |
| 76 | 128 | 75.9638 | 64 | 63.4349 | 63.4349 | 72 | 71 |
| 77 | 171 | 79.4006 | 85.5 | 69.4807 | 69.5901 | 74 | 73 |
| 78 | 256 | 82.8750 | 128 | 75.9638 | 75.9638 | 76 | 75 |
| 79 | 341 | 84.6390 | 170.5 | 79.3702 | 79.4006 | 77 |  |
| 80 | 512 | 86.4237 | 256 | 82.8750 | 82.8750 | 78 |  |

Proposed method

In the proposed method, the 4:2:2 chroma intra mode mapping process is moved after the WAIP mapping process. To support WAIP modes, the chroma intra mode mapping table is also extended based on the aspect proposed in JVET-P0111 [2]. The extended mapping table is shown in Table 3 and the extended part is marked in yellow. Besides, because of the 4:2:2 chroma intra mode mapping process is moved after the WAIP mode mapping process, the current chroma block width in the WAIP mode mapping process is not updated with SubWidthC (i.e., the current chroma block width is not divided by 2).

Table 3. The proposed 4:2:2 chroma intra mode mapping from predModeIntra to mode Y

| predModeIntra | −14 | −13 | −12 | −11 | −10 | −9 | −8 | −7 | −6 | −5 | −4 | −3 | −2 | −1 | 0 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mode Y | -14 | -14 | -14 | -13 | -12 | -11 | -11 | -10 | -10 | -10 | -9 | -8 | -7 | -7 | 0 | 1 |
| predModeIntra | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| mode Y | -6 | -5 | -4 | -3 | -2 | -1 | 2 | 3 | 5 | 6 | 8 | 10 | 12 | 13 | 14 | 16 |
| predModeIntra | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| mode Y | 18 | 20 | 22 | 23 | 24 | 26 | 28 | 30 | 31 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
| predModeIntra | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 |
| mode Y | 40 | 41 | 41 | 42 | 43 | 43 | 44 | 44 | 45 | 45 | 46 | 47 | 48 | 48 | 49 | 49 |
| predModeIntra | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 | 64 | 65 |
| mode Y | 50 | 51 | 51 | 52 | 52 | 53 | 54 | 55 | 55 | 56 | 56 | 57 | 57 | 58 | 59 | 59 |
| predModeIntra | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 | 80 |  |
| mode Y | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 69 | 70 | 72 | 74 | 76 | 77 | 78 |  |

Experimental results

The proposed correction is implemented on top of VTM7.0 [3]. The simulations were conducted with 4:2:2 test sequences. Table 4 and Table 5 show the coding performance of the proposed modifications for all intra and random access configurations, respectively.

Table 4. BD-rate performance over VTM7.0 under all intra configuration

|  | Y | U | V | EncTime | DecTime |
| --- | --- | --- | --- | --- | --- |
| EBUGraphics | 0.01% | 0.00% | 0.02% | 100% | 89% |
| EBUHorse | 0.00% | -0.00% | 0.05% | 99% | 93% |
| EBUKidSoccer | 0.00% | 0.01% | -0.11% | 99% | 100% |
| EBUWaterRocksClose | 0.00% | 0.01% | 0.05% | 99% | 99% |
| Kimono | 0.00% | -0.01% | 0.03% | 100% | 99% |
| Seeking | 0.01% | -0.02% | -0.08% | 99% | 99% |
| Traffic | 0.01% | 0.06% | -0.02% | 99% | 98% |
| All | 0.00% | 0.01% | -0.01% | 99% | 96% |

Table 5. BD-rate performance over VTM7.0 under random access configuration

|  | Y | U | V | EncTime | DecTime |
| --- | --- | --- | --- | --- | --- |
| EBUGraphics | -0.03% | 0.04% | -0.03% | 99% | 100% |
| EBUHorse | 0.02% | 0.13% | -0.02% | 99% | 99% |
| EBUKidSoccer | -0.01% | -0.02% | 0.06% | 99% | 101% |
| EBUWaterRocksClose | 0.01% | -0.07% | -0.03% | 99% | 99% |
| Kimono | 0.06% | 0.13% | 0.24% | 100% | 100% |
| Seeking | 0.00% | 0.01% | 0.02% | 100% | 100% |
| Traffic | -0.02% | -0.14% | 0.09% | 100% | 103% |
| All | 0.00% | 0.01% | 0.05% | 99% | 100% |

Conclusion

In this contribution, a bug fix is proposed for 4:2:2 chroma WAIP mode mapping. It is suggested to adopt the proposed fix into VVC.

References

- B. Bross, J. Chen, S. Liu, and Y.-K. Wang, “Versatile Video Coding (Draft 7),” Document of Joint Video Experts Team, JVET-P2001.

- B. Wang, S. Esenlik, A. M. Kotra, H. Gao, and E. Alshina, “CE3-related: Chroma intra prediction mode mapping for 4:2:2 format,” Document of Joint Video Experts Team, JVET-P0111.

- VTM7.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).