| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0194-v1 |
| --- | --- |

| Title: | Cleanup for checking CTU row boundary location in CCLM |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chia-Ming Tsai, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chia-ming.tsai, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, the condition of checking the coding tree unit (CTU) row boundary location in luma downsampling process of cross-component linear model (CCLM) modes is corrected. In VVC Draft 7, the CTU row boundary location is checked by evaluating if the top position of the current chroma transform block (TB) is a multiple of luma coding tree block (CTB) size divided by 2, which is not valid for colour formats 4:2:2 and 4:4:4. In the proposed method, the condition is modified to check if the corresponding luma top position of the current chroma TB is a multiple of luma CTB size.

# Introduction

In the 12th JVET meeting in Macao, using only one line of luma reconstructed samples from the above CTU for deriving linear model parameters [1][2] was adopted into the VVC Draft 3 [3]. However, since only 4:2:0 colour format was support in VVC Draft 3, the CTU row boundary location was found by checking if the top position of the current chroma TB is a multiple of luma CTB size divided by 2. Considering that 4:2:2 and 4:4:4 colour formats are also supported in VVC Draft 7, the condition of checking the CTU row boundary location should be corrected.

# Proposed method

The proposed modifications are done in section 8.4.5.2.13 of the current VVC spec text [4]. The changes relate to one variable as shown below and are highlighted by yellow and blue.

8.4.5.2.13	Specification of INTRA_LT_CCLM, INTRA_L_CCLM and INTRA_T_CCLM intra prediction mode

Inputs to this process are:

- the intra prediction mode predModeIntra,

- a sample location ( xTbC, yTbC ) of the top-left sample of the current transform block relative to the top-left sample of the current picture,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable cIdx specifying the colour component of the current block,

- chroma neighbouring samples p[ x ][ y ], with x = −1, y = 0..2 * nTbH − 1 and x = 0.. 2 * nTbW − 1, y = − 1.

Output of this process are predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

The current luma location ( xTbY, yTbY ) is derived as follows:

( xTbY, yTbY )  =  ( xTbC << ( SubWidthC − 1 ), yTbC << ( SubHeightC − 1 ) )	(346)

The variables availL, availT and availTL are derived as follows:

- The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xTbC, yTbC ), the neighbouring chroma location ( xTbC − 1, yTbC ), checkPredModeY set equal to FALSE, and cIdx as inputs, and the output is assigned to availL.

- The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xTbC, yTbC ), the neighbouring chroma location ( xTbC, yTbC − 1 ), checkPredModeY set equal to FALSE, and cIdx as inputs, and the output is assigned to availT.

- The variable availTL is derived as follows:

availTL  =  availL  &&  availT		(347)

- The number of available top-right neighbouring chroma samples numTopRight is derived as follows:

- The variable numTopRight is set equal to 0 and availTR is set equal to TRUE.

- When predModeIntra is equal to INTRA_T_CCLM, the following applies for x = nTbW..2 * nTbW − 1 until availTR is equal to FALSE or x is equal to 2 * nTbW − 1:

- The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xTbC , yTbC ) the neighbouring chroma location ( xTbC + x, yTbC − 1 ), checkPredModeY set equal to FALSE, and cIdx as inputs, and the output is assigned to availTR

- When availTR is equal to TRUE, numTopRight is incremented by one.

- The number of available left-below neighbouring chroma samples numLeftBelow is derived as follows:

- The variable numLeftBelow is set equal to 0 and availLB is set equal to TRUE.

- When predModeIntra is equal to INTRA_L_CCLM, the following applies for y = nTbH..2 * nTbH − 1 until availLB is equal to FALSE or y is equal to 2 * nTbH − 1:

- The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xTbC , yTbC ), the neighbouring chroma location ( xTbC − 1, yTbC + y ), checkPredModeY set equal to FALSE, and cIdx as inputs, and the output is assigned to availLB

- When availLB is equal to TRUE, numLeftBelow is incremented by one.

The number of available neighbouring chroma samples on the top and top-right numSampT and the number of available neighbouring chroma samples on the left and left-below numSampL are derived as follows:

- If predModeIntra is equal to INTRA_LT_CCLM, the following applies:

numSampT = availT ? nTbW : 0		(348)

numSampL = availL ? nTbH : 0		(349)

- Otherwise, the following applies:

numSampT = ( availT  &&  predModeIntra  = =  INTRA_T_CCLM ) ?
			( nTbW +Min( numTopRight, nTbH ) ) : 0		(350)

numSampL = ( availL  &&  predModeIntra  = =  INTRA_L_CCLM ) ? ( nTbH +
			Min( numLeftBelow, nTbW ) ) : 0		(351)

The variable bCTUboundary is derived as follows:

bCTUboundary = ( yTbC yTbY & ( 1  <<  ( CtbLog2SizeY − 1 ) − 1 )  = =  0 ) ? TRUE : FALSE.	(352)

…

# Experimental results

The proposed correction is implemented on top of VTM7.0 [5]. The simulations were conducted with 4:2:2 test sequences. Table 1 and Table 2 show the coding performance of the proposed modifications for all intra and random access configurations, respectively.

Table 1. BD-rates over VTM7.0 under all intra configuration

|  | Y | U | V | EncTime | DecTime |
| --- | --- | --- | --- | --- | --- |
| EBUGraphics | 0.00% | 0.00% | 0.00% | 101% | 90% |
| EBUHorse | 0.00% | 0.00% | 0.00% | 99% | 93% |
| EBUKidSoccer | 0.00% | 0.00% | 0.00% | 101% | 100% |
| EBUWaterRocksClose | 0.00% | 0.00% | 0.00% | 107% | 102% |
| Kimono | 0.00% | 0.00% | 0.00% | 92% | 100% |
| Seeking | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Traffic | 0.00% | 0.00% | 0.00% | 100% | 98% |
| All | 0.00% | 0.00% | 0.00% | 101% | 97% |

Table 2 BD-rates over VTM7.0 under random access configuration

|  | Y | U | V | EncTime | DecTime |
| --- | --- | --- | --- | --- | --- |
| EBUGraphics | 0.00% | 0.00% | 0.00% | 100% | 100% |
| EBUHorse | 0.00% | 0.00% | 0.00% | 100% | 108% |
| EBUKidSoccer | 0.00% | 0.00% | 0.00% | 100% | 104% |
| EBUWaterRocksClose | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Kimono | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Seeking | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Traffic | 0.00% | 0.00% | 0.00% | 100% | 101% |
| All | 0.00% | 0.00% | 0.00% | 100% | 102% |

# Conclusion

In this contribution, the condition of checking the CTU row boundary location in CCLM is corrected as 4:2:2 and 4:4:4 colour formats are supported.

# References

- C.-M. Tsai, C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, Y.-W. Huang, and S.-M. Lei, “CE3.5.8: Line buffer reduction for LM chroma,” Document of Joint Video Experts Team, JVET-L0085.

- J. Choi, J. Heo, S. Yoo, L. Li, J. Choi, J. Lim, and S. Kim, “CE3 : CCLM with line buffer restriction (Test 5.2.7),” Document of Joint Video Experts Team, JVET-L0136.

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 3),” Document of Joint Video Experts Team, JVET-L1001.

- B. Bross, J. Chen, S. Liu, and Y.-K. Wang, “Versatile Video Coding (Draft 7),” Document of Joint Video Experts Team, JVET-P2001.

- VTM7.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).