| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0212 |
| --- | --- |

| Title: | [AHG9/AHG12] On subpicture wrap around offset signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Ahmed Hamza, InterDigital Communications, Inc. Byeongdoo Choi, Stephan Wenger Tencent America | Tel: Email: | 1-858-210-4807 Yong.He@InterDigital.com bdchoi@tencent.com |
| Source: | InterDigital Communications, Inc., Tencent |  |  |

_____________________________

# Abstract

This contribution proposes to support sub-picture wrap around motion compensation when a sub-picture is treated as a picture.

- Propose to signal horizontal wraparound offset for each sub-picture when a sub-picture is treated as a picture in the decoding process.

- Propose sub-picture wrap around constraint to simplify the decoding process.

Subpicture wrap around is also proposed in JVET_Q0335, JVET_Q0344 and JVET_Q0403.

# Introduction

The horizontal wrap around motion compensation in the VTM7 is a 360-specific coding tool designed to improve the visual quality of reconstructed 360-degree video in the equi-rectangular (ERP) projection format. The horizontal wrap around motion compensation process is as depicted in Figure 1

Figure 1 Horizontal wrap around motion compensation in VVC

The padded ERP (PERP) format was supposed to effectively reduce the seam artifact and was selected as the format used for the 360 CfP anchors. However, padding and blending may not be sufficient to completely resolve the seam issue, as a seam artifact is still visible in Chairlift. The geometry wrap around padding of the reference pictures can significantly reduce the remaining seam artifact for cases where PERP alone is not sufficient.

Table 1 shows the results for horizontal geometry padding over conventional repetitive padding from JVET-L0231 (CE13 test 3.3). BD-rate savings for horizontal geometry padding over conventional repetitive padding for PERP are around {-0.22%, -0.29%, -0.38%}. The maximum BD-rate savings is observed on sequence ChairLift and reaches {-0.73%, -0.95%, -0.85%}. The complexity of horizontal geometry padding is similar to that of repetitive padding.

Table 1. BD-rate for horizontal geometry padding over conventional repetitive padding for PERP.

|  |  | PERP |  |  |
| --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | -0.53% | -1.11% | -1.18% |
|  | ChairLift | -0.73% | -0.95% | -0.85% |
|  | KiteFlite | -0.02% | 0.03% | 0.03% |
|  | Harbor | -0.03% | -0.05% | -0.10% |
|  | Trolley | -0.12% | -0.13% | -0.18% |
|  | GasLamp | -0.01% | 0.02% | 0.01% |
| Class S2 | Balboa | -0.04% | 0.17% | 0.10% |
|  | Broadway | -0.04% | 0.05% | 0.02% |
|  | Landing2 | -0.14% | -0.26% | -0.81% |
|  | BranCastle2 | -0.52% | -0.70% | -0.81% |
| Class S1 |  | -0.24% | -0.37% | -0.38% |
| Class S2 |  | -0.18% | -0.19% | -0.37% |
| Overall |  | -0.22% | -0.29% | -0.38% |
| Enc Time[%] |  | 101% |  |  |
| Dec Time[%] |  | 98% |  |  |

One of the main use cases for sub-picture is 360-degree viewport dependent streaming. Merging of sub-pictures into a composite bitstream is key for many applications when a single decoder is required. In VVC, a subpicture may be treated as a picture in the decoding process excluding in-loop filtering operations. However, the horizontal wraparound motion compensation can not apply to the sub-picture when it is treated as a picture in current VVC.

In addition, the subpicture level information SEI message specifies the level that subpictures in the bitstream conform to when testing conformance of extracted bitstreams containing the subpictures according to Annex A. It is a requirement of bitstream conformance that, when a subpicture level information SEI message is present for a CLVS, the value of subpic_treated_as_pic_flag[ i ] shall be equal to 1 for each value of i in the range of 0 to sps_num_subpics_minus1, inclusive. As a result, the wrap around motion compensation cannot apply to the extracted bitstream as well.

However, for a subpicture treated as a picture for extraction, it is possible that the spherical neighbors outside of the left subpicture boundaries can be obtained from samples inside  the right sub-picture boundary, and vice versa. Figure 2 shows example of viewport dependent streaming where subpictures are extracted and merged to form a viewport dependent picture. The low-resolution top and bottom regions and high-resolution middle region are packed to achieve 8K effective viewing experience.

Figure 2 Sub-picture extraction and merge for achieving 8K effective ERP resolution

# Proposal

## Sub-picture-based wrap around signaling

For viewport dependent streaming, wrap around motion compensation may apply to both high-resolution and low-resolution representations to reduce the seam artifacts, the merged viewport dependent packing frame cannot be corrected decoded since current VVC does not support sub-picture-based wrap around motion compensation as Figure 3 shows.

Figure 3 sub-picture wrap around viewport dependent packing example

This contribution proposes to signal wrap around per sub-picture in SPS to address the above case. Table 2 is the proposed SPS wrap around parameters.

Table 2 Proposed wrap around syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_ref_wraparound_enabled_flag | u(1) |
| if( sps_ref_wraparound_enabled_flag ) { |  |
| sps_num_ref_wraparound_offset_minus1 | u(8) |
| for( i = 0; i <= sps_num_ref_wraparound_offset_minus1; i++ ) |  |
| sps_ref_wraparound_offset_minus1[ i ] | ue(v) |
| } |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| sps_num_subpics_minus1 | u(8) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) { |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| subpic_ctu_top_left_y[ i ] | u(v) |
| subpic_width_minus1[ i ] | u(v) |
| subpic_height_minus1[ i ] | u(v) |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| if ( subpic_treated_as_pic_flag[ i ] ) { |  |
| sps_subpic_ref_wraparound_enabled_flag[ i ] | u(1) |
| if (sps_subpic_ref_wraparound_enabled_flag[ i ] && sps_num_ref_wraparound_minus1) |  |
| sps_ref_wraparound_offset_idx[ i ] | u(v) |
| } |  |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| … |  |
| } |  |

sps_ref_wraparound_enabled_flag equal to 1 specifies that horizontal wrap-around motion compensation is applied in inter prediction. sps_ref_wraparound_enabled_flag equal to 0 specifies that horizontal wrap-around motion compensation is not applied. When the value of ( CtbSizeY / MinCbSizeY + 1) is less than or equal to ( pic_width_in_luma_samples / MinCbSizeY − 1 ), where pic_width_in_luma_samples is the value of pic_width_in_luma_samples in any PPS that refers to the SPS, the value of sps_ref_wraparound_enabled_flag shall be equal to 0.

sps_num_ref_wraparound_offset_minus1 plus 1 specifies the number of wrap around offsets present in SPS.

sps_ref_wraparound_offset_minus1[ i ] plus 1 specifies the i-th offset used for computing the horizontal wrap-around position in units of MinCbSizeY luma samples. The value of ref_wraparound_offset_minus1 shall be in the range of ( CtbSizeY / MinCbSizeY ) + 1 to ( pic_width_in_luma_samples / MinCbSizeY ) − 1, inclusive, where pic_width_in_luma_samples is the value of pic_width_in_luma_samples in any PPS that refers to the SPS. When no present, the value of sps_ref_wraparound_offset_minus1[ i ] is inferred to be equal to 0. When sps_ref_wraparound_enabled_flag is equal to 1 and subpic_present_flag is equal to 0, sps_num_ref_wraparound_offset_minus1 shall be equal to 0.

sps_subpic_ref_wraparound_enabled_flag[ i ] equal to 1 specifies that horizontal wrap-around motion compensation is applied in inter prediction to i-th sub-picture. sps_ref_wraparound_enabled_flag equal to 0 specifies that horizontal wrap-around motion compensation is not applied to i-th sub-picture.

sps_ref_wraparound_offset_idx[ i ] specifies the index into the list of sps_ref_wraparound_offset_minus1. When not present, the value of sps_ref_wraparound_offset_idx[ i ] is inferred to be equal to sps_ref_wraparound_offset_minus1[ 0 ].

subpic_ref_wraparound_offset[ i ] = sps_ref_wraparound_offset_minus1[sps_ref_wraparound_offset_idx[ i ]] + 1

8.5.3.2.2 Luma sample bilinear interpolation process

…

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..1:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

xInti = Clip3( SubPicLeftBoundaryPos, SubPicRightBoundaryPos, 		sps_subic_ref_wraparound_enabled_flag[ SubPicIdx ]  ?
		ClipH( subpic_ref_wraparound_offset[SubPicIdx] * MinCbSizeY, SubPicWidth, ( xIntL + i ) ) : 
		xIntL + i )

yInti = Clip3( SubPicTopBoundaryPos, SubPicBotBoundaryPos, yIntL + i )

–	Otherwise (subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0), the following applies:

xInti = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ?
		ClipH( ( sps_ref_wraparound_offset_minus1[ 0 ] + 1 ) * MinCbSizeY, picW, ( xIntL + i ) ) : 
		xIntL + i )

yInti = Clip3( 0, picH − 1, yIntL + i )

8.5.6.3.2 Luma sample interpolation filtering process

…

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..7:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

xInti = Clip3( SubPicLeftBoundaryPos, SubPicRightBoundaryPos, 		sps_subpic_ref_wraparound_enabled_flag[ SubPicIdx ]  ?
		ClipH( subpic_ref_wraparound_offset[SubPicIdx]* MinCbSizeY, SubPicWidth, xIntL + i − 3 ) : xIntL + i − 3 )

yInti = Clip3( SubPicTopBoundaryPos, SubPicBotBoundaryPos, yIntL + i − 3 )

–	Otherwise (subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0), the following applies:

xInti = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ?
		ClipH( ( sps_ref_wraparound_offset_minus1[ 0 ] + 1 ) * MinCbSizeY, picW, xIntL + i − 3 ) : 
		xIntL + i − 3 )

yInti = Clip3( 0, picH − 1, yIntL + i − 3 )

8.5.6.3.4 Chroma sample interpolation process

…

The chroma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..3:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

The variable xOffset[ SubPicIdx ] is set equal to ( subpic_ref_wraparound_offset[SubPicIdx] * MinCbSizeY ) / SubWidthC.

xInti = Clip3( SubPicLeftBoundaryPos / SubWidthC, SubPicRightBoundaryPos / SubWidthC, sps_subpic_ref_wraparound_enabled_flag[ SubPicIdx ] ? ClipH( xOffset[ SubPicIdx ], SubPicWidthC, xIntC + i ) : xIntL + i )

yInti = Clip3( SubPicTopBoundaryPos / SubHeightC, SubPicBotBoundaryPos / SubHeightC, yIntL + i )

–	Otherwise (subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0), the following applies:

The variable xOffset is set equal to ( sps_ref_wraparound_offset_minus1[0] + 1 ) * MinCbSizeY ) / SubWidthC.

xInti = Clip3( 0, picWC − 1, sps_ref_wraparound_enabled_flag ? ClipH( xOffset, picWC, xIntC + i − 1 ) :	
		xIntC + i − 1 )

yInti = Clip3( 0, picHC − 1, yIntC + i − 1 )

## Sub-picture based wrap around constraint

To simplify sub-picture based wrap around motion compensation, a constraint is proposed to apply the wrap around motion compensation on the sub-pictures whose width is equal to the picture width. Figure 4 illustrates an example where wrap around motion compensation applies on high-resolution representation but not on low-resolution representation. The merged sub-pictures frame for viewport dependent streaming can be decoded correctly when high-resolution sub-picture wrap around is supported.

Figure 4 sub-picture wrap around viewport dependent packing example

This contribution proposes signal sub-picture wrap around in SPS to address the above case. is the proposed SPS wrap around syntax parameters.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_ref_wraparound_enabled_flag | u(1) |
| if( sps_ref_wraparound_enabled_flag ) |  |
| sps_ref_wraparound_offset_minus1 | ue(v) |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| sps_num_subpics_minus1 | u(8) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) { |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| subpic_ctu_top_left_y[ i ] | u(v) |
| subpic_width_minus1[ i ] | u(v) |
| subpic_height_minus1[ i ] | u(v) |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| if ( subpic_treated_as_pic_flag[ i ] && sps_ref_wraparound_enabled_flag ) |  |
| sps_subpic_ref_wraparound_enabled_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| … |  |
| } | u(1) |

sps_subpic_ref_wraparound_enabled_flag[ i ] equal to 1 specifies that horizontal wrap-around motion compensation is applied in inter prediction to i-th sub-picture. sps_subpic_ref_wraparound_enabled_flag equal to 0 specifies that horizontal wrap-around motion compensation is not applied to i-th sub-picture.

It is a requirement of bitstream conformance that sps_subpic_ref_wraparound_enabled_flag[ i ] shall be equal to 0 when the value of subpic_width_minus1[ i ] is not equal to PicWidthInCtbsY.

8.5.3.2.2 Luma sample bilinear interpolation process

…

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..1:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

xInti = sps_subpic_ref_wraparound_enabled_flag[ SubPicIdx ]  ? Clip3( 0, picW − 1, 
		ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, ( xIntL + i ) ) :		Clip3( SubPicLeftBoundaryPos, SubPicRightBoundaryPos, xIntL + i )

yInti = Clip3( SubPicTopBoundaryPos, SubPicBotBoundaryPos, yIntL + i )

–	Otherwise (subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0), the following applies:

xInti = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ?
		ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, ( xIntL + i ) ) : 
		xIntL + i )

yInti = Clip3( 0, picH − 1, yIntL + i )

8.5.6.3.2 Luma sample interpolation filtering process

…

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..7:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

xInti = sps_subpic_ref_wraparound_enabled_flag[ SubPicIdx ]  ? Clip3( 0, picW - 1, ClipH( subpic_ref_wraparound_offset[SubPicIdx] * MinCbSizeY, picW, xIntL + i − 3 ) : Clip3( SubPicLeftBoundaryPos, SubPicRightBoundaryPos, xIntL + i − 3 )

yInti = Clip3( SubPicTopBoundaryPos, SubPicBotBoundaryPos, yIntL + i − 3 )

–	Otherwise (subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0), the following applies:

xInti = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ?
		ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, xIntL + i − 3 ) : 
		xIntL + i − 3 )

yInti = Clip3( 0, picH − 1, yIntL + i − 3 )

8.5.6.3.4 Chroma sample interpolation process

…

The variable xOffset is set equal to ( sps_ref_wraparound_offset_minus1[0] + 1 ) * MinCbSizeY ) / SubWidthC.

The chroma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..3:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

xInti = sps_subpic_ref_wraparound_enabled_flag[ SubPicIdx ] ?                                                                        Clip3( 0, picWC − 1, ClipH( xOffset, picWC, xIntC + i − 1 ) ) : Clip3( SubPicLeftBoundaryPos / SubWidthC, SubPicRightBoundaryPos / SubWidthC, xIntL + i )

yInti = Clip3( SubPicTopBoundaryPos / SubHeightC, SubPicBotBoundaryPos / SubHeightC, yIntL + i )

–	Otherwise (subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0), the following applies:

xInti = Clip3( 0, picWC − 1, sps_ref_wraparound_enabled_flag ? ClipH( xOffset, picWC, xIntC + i − 1 ) :	
		xIntC + i − 1 )

yInti = Clip3( 0, picHC − 1, yIntC + i − 1 )

Table 3 shows the results for subpicture-based horizontal geometry padding with proposed constraint over conventional repetitive padding.

Table 3. BD-rate for horizontal geometry padding over conventional repetitive padding for PERP.

|  |  | End-to-end WS PSNR |  |  |
| --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | -0.58% | -1.41% | -1.54% |
|  | ChairLift | -0.46% | -0.67% | -0.51% |
|  | KiteFlite | 0.01% | 0.00% | 0.10% |
|  | Harbor | 0.00% | 0.00% | 0.05% |
|  | Trolley | -0.15% | -0.19% | -0.39% |
|  | GasLamp | -0.01% | 0.04% | 0.02% |

# Patent rights declaration(s)

InterDigital Communications, Inc. and Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).