| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0294-v2 |
| --- | --- |

| Title: | CE3-related: CTU level local lossless coding of VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer, Yan Ye, Jiancong Luo, Jie Chen 525 Almanor Ave, Sunnyvale, CA | Tel: Email: | +1 (408) 858-6780 m.sarwer@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

# Abstract

In order to achieve efficient compression in mixed lossy and lossless coding, this contribution proposes to signal a flag at each coding tree unit (CTU) to indicate whether a CTU is coded as either in lossless or in lossy mode. If a CTU is lossless coded, an additional CTU level flag is signaled to specify the residual coding method, either regular residual coding or transform skip residual coding, used for that CTU.

Following results of are reported as compared to VTM-7.0.

- Test 1: All lossless CTUs use transform skip residual coding (TSRC):

          - Overall: -0.01% (AI), -0.01% (RA), -0.01% (LB) bit rate saving

          - TGM:  -0.01% (AI), 0.09% (RA), -0.09% (LB) bit rate saving

- Test 2: All lossless CTUs use TSRC with rice parameter derivation method proposed in JVET-Q0269:

          - Overall: -4.33% (AI), -2.61% (RA), -1.99% (LB) bit rate saving

          - TGM:  -5.20% (AI), -6.61% (RA), -6.80% (LB) bit rate saving

- Test 3: All lossless CTUs of class F & TGM sequences are coded with TSRC with rice parameter derivation method proposed in JVET-Q0269. All lossless CTUs of other sequences are coded with regular residual coding (RRC):

          - Overall: -5.24% (AI), -5.59% (RA), -4.98% (LB) bit rate saving

          - TGM:  -5.20% (AI), -6.61% (RA), -6.80% (LB) bit rate saving

- Test 4: Test3 + luma BDPCM on

          - Overall: -6.77% (AI), -5.97% (RA), -5.55% (LB) bit rate saving

          - TGM:  -7.01% (AI), -7.44% (RA), -7.39% (LB) bit rate saving

# Introduction

In VVC7, lossless coding is achieved at sequence level by selecting the transform-skip mode for all of the coding blocks. In order to achieve the lossless coding at sequence level, the following coding tools are configured as follows[1]:

- Set maximum transform size to 32x32

- Enable chroma transform skip

- Disable dependent quantization

- Disable sub-block transform (SBT)

- Disables luma mapping with chroma scaling (LMCS)

- Disable intra sub-partitions (ISP)

- Disable multiple-transform selection (MTS)

- Disable low frequency non-separable transform (LFNST)

- Disable joint CbCr mode

- Disables de-blocking filer (DF)

- Disables sample adaptive offset (SAO)

- Disables adaptive loop filter (ALF)

- InternalBitDepth=0 (Input=Internal Bit Depth)

- Disable RDOQ and RDOQTS

The design of lossless coding of VTM-7.0 is not efficient for mixed coding scenario where one part of the image is coded as lossless mode and another part of the image is coded as lossy mode because the coding tools mentioned above are disabled/enabled in SPS level. In CE3-2.5[4] , a CU/TB level flag is proposed to indicate the residual coding method (either RRC or TSRC) for lossless coding. However, CE3-2.5 does not support CU/TB level control of in-loop filtering and other lossy tools to achieve local lossless coding. In order to achieve lossless in CE3-2.5, same as VTM-7.0, the above mentioned tools need to be disabled in SPS level.

# Proposed method

## CTU lossless flag

This contribution proposes to signal a flag at each CTU to indicate whether a CTU is coded as either in lossless or in lossy mode. The CTU level local lossless can be achieved with minimal impact on current VVC design since current VVC already supports CTU level control of ALF and SAO.  The semantic of the proposed CTU level lossless flag is defined as follows.

ctu_lossless_flag[ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] equal to 1 specifies that the coding tree unit at luma location ( xCtb, yCtb ) is losslessly coded. ctu_lossless_flag[ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] equal to 0 specifies that the coding tree unit at luma location ( xCtb, yCtb ) is not coded as lossless mode.

When ctu_lossless_flag[ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] is not present, it is inferred to be equal to 0.

Table 1 shows CTU syntax table of the proposed method. The CTU level lossless flag ( ctu_lossless_flag) is signaled at the beginning of each CTU.

Table 1: CTU syntax table with proposed flag

| coding_tree_unit( ) { | Descriptor |
| --- | --- |
| xCtb = CtbAddrX << CtbLog2SizeY |  |
| yCtb = CtbAddrY << CtbLog2SizeY |  |
| ctu_lossless_flag[ CtbAddrX ][ CtbAddrY ] | u(1) |
| ……. |  |
| } |  |

## Interaction with other coding tools

In the proposed method, if a CTU in coded as lossless mode, all of the transform blocks (with tu_cbf flag is equal to 1) within that CTU are coded as transform-skip mode. All lossy coding tools such as DF, ALF, SAO, LMCS, JointCbCr are disabled for the lossless CTU. Some other coding tools such as MTS, SBT, ISP, LFNST are also disabled for lossless CTU in order to remove redundant signalling. Table 2 shows the summary of differences between VTM-7.0 and proposed lossless coding, and a separate spec text change document is included in the package to show the required spec changes.

Table 2: summary of differences between VTM-7.0 and proposed lossless coding

| Coding tools | VTM-7.0 | Proposed CTU level control |
| --- | --- | --- |
| LMCS | Disable in SPS level | Disable if ctu_lossless_flag == 1 |
| DF | Disable in SPS level | Disable if ctu_lossless_flag == 1 |
| SAO | Disable in SPS level | Disable if ctu_lossless_flag == 1 |
| ALF | Disable in SPS level | Disable if ctu_lossless_flag == 1 |
| JointCbCr | Disable in SPS level | Disable if ctu_lossless_flag == 1 |
| SBT | Disable in SPS level | Disable if ctu_lossless_flag == 1 |
| ISP | Disable in SPS level | Disable if ctu_lossless_flag == 1 |
| MTS | Disable in SPS level | Disable if ctu_lossless_flag == 1 |
| LFNST | Disable in SPS level | Disable if ctu_lossless_flag == 1 |
| transform_skip_flag | Signal each TB | if ctu_lossless_flag == 1, transform_skip_flag is inferred to be 1. |

## CTU residual coding flag

In VTM-7.0 lossless coding, all of the coding blocks select TSRC, because lossless coding applies transform skip coding by default. However, it is observed that RRC can achieve more coding gain as compared to TSRC especially in case of camera capture contents, e.g., when lossless coding is used on the current CTU. Therefore, it is proposed to signal another CTU level flag to signal the residual coding method of a CTU. The CTU level residual coding flag may be signaled if (and only if) the ctu_lossless_flag is 1. The semantic of the proposed CTU residual coding flag is defined as follows.

ctu_rrc_flag[ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] equal to 1 specifies that all of the transform blocks of the coding tree unit at luma location ( xCtb, yCtb ) select regular residual coding method.

ctu_rrc_flag[ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] equal to 0 specifies that the transform blocks of the coding tree unit at luma location ( xCtb, yCtb ) select residual coding method based on the value of transform_skip_flag. If transform_skip_flag is equal 1, the transform block selects TSRC. If transform_skip_flag is equal to 0, the transform block selects RRC.

When ctu_lossless_flag [ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] is not present, ctu_rrc_flag [ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] is inferred to be equal to 0.

When ctu_rrc_flag [ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] is not present, it is inferred to be equal to 0.

Table 3 shows the CTU syntax table with proposed CTU level residual coding method. The ctu_rrc_flag is signalled only (and if only) ctu_lossless_flag equal to 1.

Table 3: CTU syntax table with CTU level residual coding flag

| coding_tree_unit( ) { | Descriptor |
| --- | --- |
| xCtb = CtbAddrX << CtbLog2SizeY |  |
| yCtb = CtbAddrY << CtbLog2SizeY |  |
| ctu_lossless_flag[ CtbAddrX ][ CtbAddrY ] | u(1) |
| if (ctu_lossless_flag[ CtbAddrX ][ CtbAddrY ]) |  |
| ctu_rrc_flag[ CtbAddrX ][ CtbAddrY ] | u(1) |
| ……. |  |
| } |  |

# Simulation results

The proposed method is implemented on top of VTM-7.0. In all tests, in the encoder side, ctu_lossless_flag of all CTUs of all sequences are set to 1.

Following tests are conducted

- Test 1: In test 1, all CTUs select VTM-7.0 TSRC ( i.e. ctu_rrc_flag = 0). In this test, there is no change of the VTM-7.0 TSRC.

- Test 2: In test 2, all CTUs select TSRC ( i.e. ctu_rrc_flag = 0), however, TSRC include rice parameter derivation part proposed in JVET-Q0269[3].

- Test 3: In test 3, all CTUs of camera capture content are coded using RRC. However, all CTUs of Class F & TGM sequences are coded in TSRC with rice parameter derivation proposed in JVET-Q0269[3].

- Test 4: Test 3 + Luma BDPCM on

Each test is evaluated under lossless test conditions described in CE3[2]. Table 4, Table 5, Table 6, and Table 7 show the results of test 1, test 2 , test 3 and test 4, respectively.

Table 4: results of test 1

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | Ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM-7.0 | test 1 |  | VTM-7.0 | test 1 |  | VTM-7.0 | test 1 |  |
| Class A1 | 2.2 | 2.2 | -0.01% | 2.2 | 2.2 | -0.01% |  |  |  |
| Class A2 | 1.6 | 1.6 | -0.01% | 1.7 | 1.7 | -0.01% |  |  |  |
| Class B | 2.2 | 2.2 | -0.01% | 2.3 | 2.3 | -0.01% | 2.3 | 2.3 | -0.01% |
| Class C | 1.9 | 1.9 | -0.01% | 2.4 | 2.4 | -0.01% | 2.4 | 2.4 | -0.01% |
| Class D | 1.9 | 1.9 | -0.01% | 2.8 | 2.8 | -0.02% | 2.7 | 2.7 | -0.03% |
| Class E | 2.8 | 2.8 | -0.01% |  |  |  | 3.1 | 3.1 | -0.01% |
| Class F | 5.3 | 5.3 | -0.02% | 33.7 | 33.7 | 0.01% | 50.7 | 50.7 | 0.02% |
| TGM | 11.8 | 11.8 | -0.01% | 107.1 | 107.0 | 0.09% | 124.9 | 125.1 | -0.09% |
| Overall | 2.1 | 2.1 | -0.01% | 2.2 | 2.2 | -0.01% | 2.6 | 2.6 | -0.01% |
| Enc Time[%] | 101% |  |  | 102% |  |  | 102% |  |  |
| Dec Time[%] | 88% |  |  | 92% |  |  | 93% |  |  |

Table 5: results of test 2

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | Ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM-7.0 | test 2 |  | VTM-7.0 | test 2 |  | VTM-7.0 | test 2 |  |
| Class A1 | 2.2 | 2.3 | -3.50% | 2.2 | 2.3 | -3.34% |  |  |  |
| Class A2 | 1.6 | 1.7 | -7.56% | 1.7 | 1.8 | -5.10% |  |  |  |
| Class B | 2.2 | 2.2 | -2.97% | 2.3 | 2.4 | -1.02% | 2.3 | 2.4 | -1.01% |
| Class C | 1.9 | 2.0 | -4.25% | 2.4 | 2.5 | -2.19% | 2.4 | 2.5 | -2.05% |
| Class D | 1.9 | 2.0 | -4.90% | 2.8 | 2.8 | -2.40% | 2.7 | 2.8 | -2.23% |
| Class E | 2.8 | 2.9 | -4.29% |  |  |  | 3.1 | 3.2 | -3.55% |
| Class F | 5.3 | 5.5 | -3.84% | 33.7 | 34.9 | -3.04% | 50.7 | 52.2 | -2.81% |
| TGM | 11.8 | 12.5 | -5.20% | 107.1 | 116.4 | -6.61% | 124.9 | 136.8 | -6.80% |
| Overall | 2.1 | 2.2 | -4.33% | 2.2 | 2.3 | -2.61% | 2.6 | 2.6 | -1.99% |
| Enc Time[%] | 97% |  |  | 99% |  |  | 101% |  |  |
| Dec Time[%] | 90% |  |  | 94% |  |  | 98% |  |  |

Table 6: results of test 3

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | Ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM-7.0 | test 3 |  | VTM-7.0 | test 3 |  | VTM-7.0 | test 3 |  |
| Class A1 | 2.2 | 2.4 | -5.42% | 2.2 | 2.4 | -6.26% |  |  |  |
| Class A2 | 1.6 | 1.7 | -8.86% | 1.7 | 1.9 | -8.13% |  |  |  |
| Class B | 2.2 | 2.3 | -4.02% | 2.3 | 2.4 | -3.84% | 2.3 | 2.4 | -3.85% |
| Class C | 1.9 | 2.0 | -4.98% | 2.4 | 2.6 | -5.37% | 2.4 | 2.6 | -5.30% |
| Class D | 1.9 | 2.0 | -4.64% | 2.8 | 2.9 | -5.29% | 2.7 | 2.9 | -5.27% |
| Class E | 2.8 | 2.9 | -3.83% |  |  |  | 3.1 | 3.3 | -6.43% |
| Class F | 5.3 | 5.5 | -3.84% | 33.7 | 34.9 | -3.04% | 50.7 | 52.2 | -2.81% |
| TGM | 11.8 | 12.5 | -5.20% | 107.1 | 116.4 | -6.61% | 124.9 | 136.8 | -6.80% |
| Overall | 2.1 | 2.2 | -5.24% | 2.2 | 2.3 | -5.59% | 2.6 | 2.7 | -4.98% |
| Enc Time[%] | 95% |  |  | 100% |  |  | 98% |  |  |
| Dec Time[%] | 89% |  |  | 92% |  |  | 98% |  |  |

Table 7: results of test 4

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | Ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM-7.0 | test 3 |  | VTM-7.0 | test 3 |  | VTM-7.0 | test 3 |  |
| Class A1 | 2.2 | 2.4 | -5.82% | 2.2 | 2.4 | -6.52% |  |  |  |
| Class A2 | 1.6 | 1.8 | -10.00% | 1.7 | 1.9 | -8.40% |  |  |  |
| Class B | 2.2 | 2.3 | -5.59% | 2.3 | 2.5 | -4.30% | 2.3 | 2.4 | -4.34% |
| Class C | 1.9 | 2.1 | -6.42% | 2.4 | 2.6 | -5.80% | 2.4 | 2.6 | -5.75% |
| Class D | 1.9 | 2.1 | -7.51% | 2.8 | 2.9 | -5.80% | 2.7 | 2.9 | -5.66% |
| Class E | 2.8 | 3.0 | -6.94% |  |  |  | 3.1 | 3.4 | -7.30% |
| Class F | 5.3 | 5.8 | -7.10% | 33.7 | 35.7 | -4.41% | 50.7 | 53.0 | -4.02% |
| TGM | 11.8 | 12.8 | -7.01% | 107.1 | 117.5 | -7.44% | 124.9 | 137.6 | -7.39% |
| Overall | 2.1 | 2.3 | -6.77% | 2.2 | 2.4 | -5.97% | 2.6 | 2.7 | -5.55% |
| Enc Time[%] | 107% |  |  | 104% |  |  | 103% |  |  |
| Dec Time[%] | 83% |  |  | 91% |  |  | 98% |  |  |

# Conclusion

In this contribution, a CTU level lossless flag is proposed to achieve efficient mixed lossy/lossless coding. In order to improve the coding performance of lossless CTU, another CTU level residual coding flag is also proposed to select the optimal residual coding method for transform blocks in the current CTU.

# References

B. Bross, T. Nguyen, H. Schwarz, D. Marpe, T. Wiegand (HHI), M. Karczewicz, Y.-H. Chao, H. Wang, M. Coban, “ AHG18: Enabling lossless coding with minimal impact on VVC design”, JVET-P0606, Geneva, Oct. 2019.

T. Nguyen, T. C. Ma, A. Nalci, “Description of Core Experiment 3 (CE3): Lossless Coding”, JVET-P2023

M. G. Sarwer, Y. Ye, J. Luo, “ CE3-related: On transform skip residual coding”, JVET-Q0269

T.-C. Ma, H.-J. Jhu, X. Xiu, Y.-W. Chen, X. Wang, Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, A. Nalci, H. E. Egilmez, M. Coban, M. Karczewicz, “ CE3-2.5: Residual coding selection signaling for lossless coding”, JVET-Q0070.

# Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).