| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0300 |
| --- | --- |

| Title: | Unification of bypass coding between TSRC and RRC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungah Choi, Sunmi Yoo, Jin Heo, Jaehyun Lim, Seung Hwan Kim 19 Yangjaedaero-11gil, Seocho-gu, Seoul, 06772 South Korea Yusuke Kato Kiyofumi Abe Tadamasa Toma 3-1-1 Yagumo-naka-machi, Moriguchi, Osaka, Japan | Tel: Email: | +82-2-6912-6561 jungah.choi@lge.com +81-80-3083-1171 kato.yusuke003@jp.panasonic.com |
| Source: | LG Electronics Inc. Panasonic |  |  |

_____________________________

- Abstract

In the current transform skip residual coding, bypass coded bins are coded with the interleaved manner (residual sample by residual sample) after context-coded bin count reaches the maximally allowed number. This contribution proposes to change the coding order of bypass coded bins/syntaxes in transform skip residual coding so that bypass bins are coded together as regular residual coding. The proposed method has no coding efficiency change.

Proposal

As shown in Figure 1, regular residual coding (RRC), after consuming all the allowed context coded bins, each of bypass coded syntaxes such as abs_remainder, dec_abs_level, and coeff_sign_flag are grouped together and coded in regular residual coding (RRC). However, in transform skip residual coding shown in Figure 2, some part of syntaxes (abs_remainder, coeff_sign_flag) are required to be coded as bypass bins with interleaved manner when maximally allowed context coded bins are used. Here, we assume that the number of context coded bin count exceeds at Coeffm+1, and bypass coded syntaxes are coded sample by sample.

Therefore, in order to align the coding order of bypass coded syntaxes in TSRC with that in RRC, and also efficiently code bypass bins, it is proposed to exclude coeff_sign_flag from the conventional pass to another loop. Figure 3 shows the proposed coding order. From this method, we can realize the complete grouping bypass coded syntaxes together. Moreover, the proposed coding order is more unified to that of RRC. Note that there is no coding performance change introduced by the proposed method.

Figure 1. Coding order of bypass coded syntaxes in regular residual coding

Figure 2. Coding order of bypass coded syntaxes in transform skip residual coding

Figure 3. Proposed coding order of bypass coded syntaxes in transform skip residual coding

Spec text

The following table shows the part of the TSRC syntax of the proposed method. Proposed text modifications are based on JVET-P2001-vE.

| residual_ts_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| (…) |  |
| /* remainder scan pass */ |  |
| for( n = 0; n <= numSbCoeff − 1; n++ ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( ( n <= lastScanPosPass2 && AbsLevelPass2[ xC ][ yC ] >= 10 ) ¦ ¦ ( n > lastScanPosPass2 && n <= lastScanPosPass1 && AbsLevelPass1[ xC ][ yC ] >= 2 ) ¦ ¦ ( n > lastScanPosPass1 && coded_sub_block_flag[ xS ][ yS ] ) ) |  |
| abs_remainder[ n ] | ae(v) |
| if( n <= lastScanPosPass2 ) |  |
| AbsLevel[ xC ][ yC ] = AbsLevelPass2[ xC ][ yC ] + 2 * abs_remainder[ n ] |  |
| else if(n <= lastScanPosPass1 ) |  |
| AbsLevel[ xC ][ yC ] = AbsLevelPass1[ xC ][ yC ] + 2 * abs_remainder[ n ] |  |
| else { /* bypass |  |
| AbsLevel[ xC ][ yC ] = abs_remainder[ n ] |  |
| if( abs_remainder[ n ] ) |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| if( BdpcmFlag[ x0 ][ y0 ][ cIdx ] = = 0 && n <= lastScanPosPass1 ) { |  |
| absLeftCoeff = xC > 0 ? AbsLevel[ xC − 1 ][ yC ] ) : 0 |  |
| absAboveCoeff = yC > 0 ? AbsLevel[ xC ][ yC − 1 ] ) : 0 |  |
| predCoeff = Max( absLeftCoeff, absAboveCoeff ) |  |
| if( AbsLevel[ xC ][ yC ] = = 1 && predCoeff > 0 ) |  |
| AbsLevel[ xC ][ yC ] = predCoeff |  |
| else if( AbsLevel[ xC ][ yC ] > 0 && AbsLevel[ xC ][ yC ] <= predCoeff ) |  |
| AbsLevel[ xC ][ yC ] −= 1 |  |
| } |  |
| } |  |
| for( n = 0; n <= numSbCoeff − 1; n++ ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if ( n > lastScanPosPass1 && AbsLevel[ xC ][ yC ] ) |  |
| coeff_sign_flag[ n ] | ae(v) |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( 1 − 2 * coeff_sign_flag[ n ] ) * AbsLevel[ xC ][ yC ] |  |
| } |  |
| } |  |
| } |  |

Conclusion

This contribution proposed to change the coding order of bypass coded syntaxes. There is no coding efficiency change in all classes by the proposed method, and it is more aligned with RRC. It is recommended to adopt the proposed coding order into VVC.

References

- B. Bross, J. Chen, S. Liu, S. Liu, Y. Wang, “Versatile Video Coding (Draft 7),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-P0079, 16th meeting, Gevena, CH, 1-11 Oct. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).