| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0331 |
| --- | --- |

| Title: | AhG8: Constraints on the picture scaling ratios |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang Vadim Seregin Muhammed Coban Marta Karczewicz | Tel: Email: | +1 858 845 1779 yjchang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This proposal provides two constraints on the picture scaling ratios, and is summarized as follows:

- Constraints on the range of the picture scaling ratio to be in the range of 1/8 to 2, inclusive, i.e., 1/8 * PicOutputWidthL <= refPicOutputWidthL <= 2 * PicOutputWidthL

- If subpicture is enabled, the picture scaling ratio shall be 1. This can be fixed by disabling subpicture when RPR is enabled at SPS.

# Problem Statement

The reference picture resampling (RPR) applies a picture scaling ratio to a current picture when the reference picture size and the current picture size are different. The reference picture scaling ratios in VVC WD clause 8.3.2 is extracted as follows:

RefPicScale[ i ][ j ][ 0 ] = ( ( fRefWidth  <<  14 ) + ( PicOutputWidthL  >>  1 ) ) / PicOutputWidthL
RefPicScale[ i ][ j ][ 1 ] = ( ( fRefHeight  <<  14 ) + ( PicOutputHeightL  >>  1 ) ) / PicOutputHeightL

where fRefWidth and fRefHeight are set equal to the scaling window width and height, i.e., PicOutputWidthL and PicOutputHeightL, of the reference picture RefPicList[ i ][ j ] in luma samples, respectively. PicOutputWidthL and PicOutputHeightL are cropped from the picture width and picture height as shown below:

PicOutputWidthL = pic_width_in_luma_samples − ( scaling_win_right_offset + scaling_win_left_offset )

PicOutputHeightL = pic_height_in_luma_samples − ( scaling_win_bottom_offset + scaling_win_top_offset )

Figure 1 shows an example of the scaling windows in the reference picture and the current picture. The range of the scaling ratios should be set properly to be in the range of 1/8 to 2, inclusive. The improper values outside this range could lead to a destructive distortion in visual perception. Since the scaling ratio is derived by the scaling window width and height PicOutputWidthL and PicOutputHeightL, the range of 1/8 to 2 shall be applied to the scaling window widths and heights between the reference and current pictures. However, the current VVC design wrongly apply this range to the picture widths and heights instead of the scaling window widths and heights, and the current design still can derive a scaling factor way outside the range of 1/8 to 2. This proposal would like to resolve this issue.

The design concept of the current VVC specification disallows the reference picture scaling functionality if the RPR is disabled or if the subpicture is enabled, i.e., ref_pic_resampling_enabled_flag is equal to 0 or subpics_present_flag is equal to 1. When ref_pic_resampling_enabled_flag is equal to 0, the picture width and height remain constant in the CVS, and the scaling window offsets are also inferred to be 0. The scaling window width and height, PicOutputWidthL and PicOutputHeightL, are constants in the CVS, and thus the reference picture scaling ratio is constantly set as 1. However, when subpics_present_flag is equal to 1, it only sets the picture width and height to be constants in the CVS, and the scaling window offsets could be signaled with the values larger than 0. It enables the reference picture scaling functionality due to dynamic values of the scaling window width and height. This scaling window offsets in subpicture should be disabled based on the design concept of the current VVC. This proposal would like to address this issue as well.

# Scaling window constraints on the picture scaling ratios

In current VVC specification, the range of the reference picture scaling ratios shall be in the range of 1/8 to 2, inclusive, and there is a bitstream conformance to apply this range to the picture widths and heights between the reference and current pictures as follows:

Let refPicWidthInLumaSamples and refPicHeightInLumaSamples be the pic_width_in_luma_samples and pic_height_in_luma_samples, respectively, of a reference picture of a current picture referring to this PPS. There is a requirement of bitstream conformance that all of the following conditions are satisfied:

–	pic_width_in_luma_samples * 2 shall be greater than or equal to refPicWidthInLumaSamples.

–	pic_height_in_luma_samples * 2 shall be greater than or equal to refPicHeightInLumaSamples.

–	pic_width_in_luma_samples shall be less than or equal to refPicWidthInLumaSamples * 8.

Current picture

Reference picture

Reference scaling window

Current scaling window

–	pic_height_in_luma_samples shall be less than or equal to refPicHeightInLumaSamples * 8.

Figure 1. The scaling windows inside the reference picture and the current picture

Because the reference picture scaling ratios are derived by the scaling window widths and heights, PicOutputWidthL and PicOutputHeightL, the scaling ratios could be outside the range if there is no restriction to PicOutputWidthL and PicOutputHeightL. Therefore, it is proposed to apply this range of 1/8 to 2 to the scaling window widths and heights between the reference and current pictures, i.e., 1/8 * PicOutputWidthL <= refPicOutputWidthL <= 2 * PicOutputWidthL:

Let refPicOutputWidthL and refPicOutputHeightL be the PicOutputWidthL and PicOutputHeightL, respectively, of a reference picture of a current picture referring to this PPS. There is a bitstream conformance that all of the following conditions shall be satisfied:

–	PicOutputWidthL * 2 shall be greater than or equal to refPicOutputWidthL.

–	PicOutputHeightL * 2 shall be greater than or equal to refPicOutputHeightL.

–	PicOutputWidthL shall be less than or equal to refPicOutputWidthL * 8.

–	PicOutputHeightL shall be less than or equal to refPicOutputHeightL * 8.

Another alternative method is that the picture scaling ratios RefPicScale[ i ][ j ] [0] and RefPicScale[ i ][ j ] [1] can be checked or clipped to be within the specified range as follows:

–	RefPicScale[ i ][ j ][ 0 ] shall be smaller than or equal to 2M

–	RefPicScale[ i ][ j ][ 0 ] shall be larger than or equal to 2N
–	RefPicScale[ i ][ j ][ 1 ] shall be smaller than or equal to 2M

–	RefPicScale[ i ][ j ][ 1 ] shall be larger than or equal to 2N

where M = 15 and N = 11.

It is further proposed to remove the constraints on the picture widths and heights that are currently in the VVC spec as mentioned above.

# Subpicture constraint on the picture scaling ratios

In current VVC specification, the design principle disallows the picture scaling functionality, i.e., the scaling ratio shall be 1, if one of the following conditions is satisfied:

- RPR is disabled, i.e, ref_pic_resampling_enabled_flag is equal to 0

- Subpicture is enabled, i.e., subpics_present_flag is equal to 1

The related specification in current VVC is extracted as follows:

Clause 7.3.2.4 in VVC Draft 7 version 14: pic_parameter_set_rbsp( ) {; Descriptor. …. pic_width_in_luma_samples; ue(v). pic_height_in_luma_samples; ue(v). …. scaling_window_flag; u(1). if( scaling_window_flag ) {. scaling_win_left_offset; ue(v). scaling_win_right_offset; ue(v). scaling_win_top_offset; ue(v). scaling_win_bottom_offset; ue(v). }. …. Clause 7.4.3.4 in VVC Draft 7 version 14: When subpics_present_flag is equal to 1 or ref_pic_resampling_enabled_flag equal to 0, the value of pic_width_in_luma_samples shall be equal to pic_width_max_in_luma_samples. When subpics_present_flag is equal to 1 or ref_pic_resampling_enabled_flag equal to 0, the value of pic_height_in_luma_samples shall be equal to pic_height_max_in_luma_samples. When ref_pic_resampling_enabled_flag is equal to 0, the value of scaling_window_flag shall be equal to 0.

However, there is an error which could activate the picture scaling functionality when scaling_window_flag is equal to 1 in the subpicture use case. This disobeys the design principle.

The issue can be fixed by simply disabling subpicture when RPR is enabled at SPS. The proposed change is as follows with yellow change mark:

ref_pic_resampling_enabled_flag; u(1). if( !ref_pic_resampling_enabled_flag ). subpics_present_flag; u(1). subpics_present_flag equal to 1 specifies that subpicture parameters are present in in the SPS RBSP syntax. subpics_present_flag equal to 0 specifies that subpicture parameters are not present in the SPS RBSP syntax. When not present, the value of subpics_present_flag is inferred to be equal to 0.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).