| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0335 |
| --- | --- |

| Title: | AhG9: On the wraparound offsets |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang Vadim Seregin Muhammed Coban Marta Karczewicz | Tel: Email: | +1 858 845 1779 yjchang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This proposal would like to fix the bugs of the wraparound offsets in reference picture resampling (RPR). The modifications on top of the current VVC include:

- Because the reference picture resampling (RPR) design would renew the picture width and height in PPS, it is proposed that the wraparound offsets shall be signaled in PPS instead of SPS to align the design of RPR.

  - The value of ref_wraparound_enabled_flag shall be the same for all PPSs that are referred to by coded pictures in a CLVS. This constraint aligns the current VVC design which keeps the wraparound enabled flag unchanged in CLVS.

- Motion compensation is fixed by replacing the wraparound offset of the current picture with the wraparound offset of the reference picture.

- Wraparound support on the RPR and subpictures at the same time

Figure 1. Wraparound offset in the reference picture with PERP format

# Problem Statement

A horizontal wraparound offset in motion compensation was adopted into VVC to reduce the subjectively visual artifacts for the 360 use case using Padded Equirectangular Projection (PERP) format, in which the left picture boundaries and the right picture boundaries are padded to be the same. The reference pixels that extend outside the border of the picture are wrapped around by moving the position with a distance of ERP width as shown in Figure 1. The wraparound enabled flag and the wraparound offset, e.g. ERP width, are signaled in SPS in the current VVC:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_ref_wraparound_enabled_flag | u(1) |
| if( sps_ref_wraparound_enabled_flag ) |  |
| sps_ref_wraparound_offset_minus1 | ue(v) |
| … |  |

- sps_ref_wraparound_enabled_flag equal to 1 specifies that horizontal wrap-around motion compensation is applied in inter prediction. sps_ref_wraparound_enabled_flag equal to 0 specifies that horizontal wrap-around motion compensation is not applied. When the value of ( CtbSizeY / MinCbSizeY + 1) is less than or equal to ( pic_width_in_luma_samples / MinCbSizeY − 1 ), where pic_width_in_luma_samples is the value of pic_width_in_luma_samples in any PPS that refers to the SPS, the value of sps_ref_wraparound_enabled_flag shall be equal to 0.

- sps_ref_wraparound_offset_minus1 plus 1 specifies the offset used for computing the horizontal wrap-around position in units of MinCbSizeY luma samples. The value of ref_wraparound_offset_minus1 shall be in the range of ( CtbSizeY / MinCbSizeY ) + 1 to ( pic_width_in_luma_samples / MinCbSizeY ) − 1, inclusive, where pic_width_in_luma_samples is the value of pic_width_in_luma_samples in any PPS that refers to the SPS.

The wraparound offset shall be smaller than or equal to the picture width. If we don’t disallow the reference picture resampling (RPR) functionality and wraparound offset functionality are enabled at the same time, the wraparound offsets should be different for the pictures referring to different PPS because the RPR design would renew the picture width and height in PPS. However, the current VVC design keeps the wraparound offset as a constant in CLVS. This would wrongly shift the position of a pixel to another position outside the reference picture as shown in Figure 2, where the reference picture width and the current picture width are different in the RPR. This proposal would like to fix the issues.

# Moving the signaling of the wraparound offsets to the PPS from SPS

The wraparound offset shall be smaller than or equal to the picture width. Because the reference picture resampling (RPR) design would renew the picture width and height in PPS, it is proposed that the wraparound offsets shall be signaled in PPS instead of SPS to align the design of RPR.

It is further proposed that the value of ref_wraparound_enabled_flag shall be the same for all PPSs that are referred to by coded pictures in a CLVS. This constraint can align the current VVC design which keeps the wraparound enabled flag unchanged in CLVS.

The changes to the current VVC specification are shown as follows while the yellow parts are the changes relative to the current VVC specification:

Clause 7.3.2.3 in VVC Draft 7 version 14: seq_parameter_set_rbsp( ) {; Descriptor. …. sps_ref_wraparound_enabled_flag; u(1). if( sps_ref_wraparound_enabled_flag ). sps_ref_wraparound_offset_minus1; ue(v). …. Clause 7.4.3.3 in VVC Draft 7 version 14: sps_ref_wraparound_enabled_flag equal to 1 specifies that horizontal wrap-around motion compensation is applied in inter prediction. sps_ref_wraparound_enabled_flag equal to 0 specifies that horizontal wrap-around motion compensation is not applied. When the value of ( CtbSizeY / MinCbSizeY + 1) is larger than or equal to ( pic_width_in_luma_samples / MinCbSizeY − 1 ), where pic_width_in_luma_samples is the value of pic_width_in_luma_samples in any PPS that refers to the SPS, the value of sps_ref_wraparound_enabled_flag shall be equal to 0. sps_ref_wraparound_offset_minus1 plus 1 specifies the offset used for computing the horizontal wrap-around position in units of MinCbSizeY luma samples. The value of ref_wraparound_offset_minus1 shall be in the range of ( CtbSizeY / MinCbSizeY ) + 1 to ( pic_width_in_luma_samples / MinCbSizeY ) − 1, inclusive, where pic_width_in_luma_samples is the value of pic_width_in_luma_samples in any PPS that refers to the SPS. Clause 7.3.2.4 in VVC Draft 7 version 14: pic_parameter_set_rbsp( ) {; Descriptor. …. ref_wraparound_enabled_flag; u(1). if( ref_wraparound_enabled_flag ). ref_wraparound_offset_minus1; ue(v). …. Clause 7.4.3.4 in VVC Draft 7 version 14: ref_wraparound_enabled_flag equal to 1 specifies that horizontal wrap-around motion compensation is applied in inter prediction. ref_wraparound_enabled_flag equal to 0 specifies that horizontal wrap-around motion compensation is not applied. When the value of ( CtbSizeY / MinCbSizeY + 1) is larger than or equal to ( pic_width_in_luma_samples / MinCbSizeY − 1 ), where pic_width_in_luma_samples is the value of pic_width_in_luma_samples in any PPS that refers to the SPS, the value of ref_wraparound_enabled_flag shall be equal to 0. It is a requirement of bitstream conformance that the value of ref_wraparound_enabled_flag shall be the same for all PPSs that are referred to by coded pictures in a CLVS. ref_wraparound_offset_minus1 plus 1 specifies the offset used for computing the horizontal wrap-around position in units of MinCbSizeY luma samples. The value of ref_wraparound_offset_minus1 shall be in the range of ( CtbSizeY / MinCbSizeY ) + 1 to ( pic_width_in_luma_samples / MinCbSizeY ) − 1, inclusive.

Reference picture

Current picture

wraparound_offset_curr

wraparound_offset_curr

Figure 2. The position of a reference pixel could be wrongly shifted to an unknown position, i.e., outside of the reference picture, by the wraparound offset of the current picture

# Bugfixes in motion compensation

In the current VVC design, the wraparound offset of the current picture is used to locate the position of a reference pixel. This could wrongly shift this reference pixel to another unknown position outside the reference picture as shown in Figure 2, where the reference picture width and the current picture width are different.

The issue can be fixed by replacing the wraparound offset of the current picture with the wraparound offset of the reference picture in motion compensation. The changes to the current VVC specification are shown as follows while the yellow parts are the changes relative to the current VVC specification:

Clause 8.5.3.2.2 in VVC Draft 7 version 14: ref_wraparound_offset_minus1 is the ref_wraparound_offset_minus1 parsed in PPS that is referred to by the reference picture. xInti = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ? ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, ( xIntL + i ) ) : (644) xIntL + i ) yInti = Clip3( 0, picH − 1, yIntL + i ) (645) Clause 8.5.6.3.2 in VVC Draft 7 version 14: ref_wraparound_offset_minus1 is the ref_wraparound_offset_minus1 parsed in PPS that is referred to by the reference picture. xInti = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ? ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, xIntL + i − 3 ) : (957) xIntL + i − 3 ) yInti = Clip3( 0, picH − 1, yIntL + i − 3 ) (958) Clause 8.5.6.3.3 in VVC Draft 7 version 14: ref_wraparound_offset_minus1 is the ref_wraparound_offset_minus1 parsed in PPS that is referred to by the reference picture. xInt = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ? (968) ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, xIntL ) : xIntL ) yInt = Clip3( 0, picH − 1, yIntL ) (969) Clause 8.5.6.3.4 in VVC Draft 7 version 14: ref_wraparound_offset_minus1 is the ref_wraparound_offset_minus1 parsed in PPS that is referred to by the reference picture. The variable xOffset is set equal to ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY ) / SubWidthC. xInti = Clip3( 0, picWC − 1, sps_ref_wraparound_enabled_flag ? ClipH( xOffset, picWC, xIntC + i − 1 ) : xIntC + i − 1 ) yInti = Clip3( 0, picHC − 1, yIntC + i − 1 ) (974)

An alternative method to fix the issue in motion compensation is to use the scaling ratios between the reference picture and the current picture to scale the wraparound offset that is derived from the PPS referred to by the current picture. A general formula to derive the wraparound offset of the reference picture, defined as ref_wraparound_offset, is as follows:

ref_wraparound_offset = curr_wraparound_offset * ref_pic_width ÷ curr_pic_width,

where curr_wraparound_offset is the wraparound offset of the current picture, ref_pic_width is defined as the pic_width_max_in_luma_samples of the reference picture, and curr_pic_width is defined as the pic_width_max_in_luma_samples of the current picture.

# Wraparound for subpictures

In JVET-P0127, it was proposed to support the wraparound motion compensation for subpictures. This contribution would to apply the proposed methods to subpicture wraparound. Note that the proposed subpicture wraparound supports the RPR functionality. The modifications would be as follows:

- Signal the wraparound flag and the wraparound offsets in PPS to align RPR design, which would change subpicture width and height at PPS:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_num_subpics_minus1 | ue(v) |
| for( i = 0; i <= pps_num_subpics_minus1; i++ ) { |  |
| ref_wraparound_enabled_flag[ i ] | u(1) |
| if( ref_wraparound_enabled_flag[ i ] ) |  |
| ref_wraparound_offset_minus1[ i ] | ue(v) |
| } |  |
| … |  |

- Add the subpicture index RefSubPicIdx to the wraparound parameters in motion compensation on top of the proposed changes in Section 3, where RefSubPicIdx is derived to be such that SubpicIdList[ RefSubPicIdx ] of the reference picutre is equal to slice_subpic_id of the current slice.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).