| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0336-r1 |
| --- | --- |

| Title: | AHG9: Cleanup for the maximum number of triangle merging candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang Vadim Seregin Muhammed Coban Adarsh Ramasubramonian Marta Karczewicz | Tel: Email: | +1 858 845 1779 yjchang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

The contribution is summarized as follows:

- If the value of MaxNumMergeCand varies in picture by picture (i.e., if it is signaled in PH instead of PPS), the value of MaxNumTriangleMergeCand shall be signaled in PH instead of PPS as well.

- MaxNumTriangleMergeCand is inferred to be equal to 2 in picture header without signaling in either PPS or picture headers if MaxNumMergeCand is equal to 2.

- In response to the comments in spec ticket #691 of the JVET VVC bug tracker, this contribution provides an alternative method which applies JVET-N0851 and JVET-N0447 into the current VVC specification.

# Introduction

There are PPS and PH constant parameters design to signal a parameter in either PPS or PH in the current VVC. The maximum number of triangle merging candidates is one of the parameters within this PPS/PH signaling structure. The specification is extracted as follows:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( constant_slice_header_params_enabled_flag ) { |  |
| … |  |
| pps_six_minus_max_num_merge_cand_plus1 | ue(v) |
| pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 | ue(v) |
| } |  |

pps_six_minus_max_num_merge_cand_plus1 equal to 0 specifies that pic_six_minus_max_num_merge_cand is present in PHs referring to the PPS. pps_six_minus_max_num_merge_cand_plus1 greater than 0 specifies that pic_six_minus_max_num_merge_cand is not present in PHs referring to the PPS. The value of pps_six_minus_max_num_merge_cand_plus1 shall be in the range of 0 to 6, inclusive.

pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 equal to 0 specifies that pic_max_num_merge_cand_minus_max_num_triangle_cand is present in PHs of slices referring to the PPS. pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 greater than 0 specifies that pic_max_num_merge_cand_minus_max_num_triangle_cand is not present in PHs referring to the PPS. The value of pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 shall be in the range of 0 to MaxNumMergeCand − 1.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( !pps_six_minus_max_num_merge_cand_plus1 ) |  |
| pic_six_minus_max_num_merge_cand | ue(v) |
| … |  |
| if( sps_triangle_enabled_flag && MaxNumMergeCand >= 2 && !pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 ) |  |
| pic_max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |

pic_six_minus_max_num_merge_cand specifies the maximum number of merging motion vector prediction (MVP) candidates supported in the slices associated with the PH subtracted from 6. The maximum number of merging MVP candidates, MaxNumMergeCand is derived as follows:

MaxNumMergeCand = 6 − picsix_minus_max_num_merge_cand	(85)

The value of MaxNumMergeCand shall be in the range of 1 to 6, inclusive. When not present, the value of pic_six_minus_max_num_merge_cand is inferred to be equal to pps_six_minus_max_num_merge_cand_plus1 − 1.

pic_max_num_merge_cand_minus_max_num_triangle_cand specifies the maximum number of triangular merge mode candidates supported in the slices associated with the pictur header subtracted from MaxNumMergeCand.

When pic_max_num_merge_cand_minus_max_num_triangle_cand is not present, and sps_triangle_enabled_flag is equal to 1 and MaxNumMergeCand greater than or equal to 2, pic_max_num_merge_cand_minus_max_num_triangle_cand is inferred to be equal to pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 − 1.

The maximum number of triangular merge mode candidates, MaxNumTriangleMergeCand is derived as follows:

MaxNumTriangleMergeCand = 
	MaxNumMergeCand − pic_max_num_merge_cand_minus_max_num_triangle_cand	(87)

When pic_max_num_merge_cand_minus_max_num_triangle_cand is present, the value of MaxNumTriangleMergeCand shall be in the range of 2 to MaxNumMergeCand, inclusive.

When pic_max_num_merge_cand_minus_max_num_triangle_cand is not present, and (sps_triangle_enabled_flag is equal to 0 or MaxNumMergeCand is less than 2), MaxNumTriangleMergeCand is set equal to 0.

When MaxNumTriangleMergeCand is equal to 0, triangle merge mode is not allowed for the slices associated with the PH.

# Problem statement

Issue 1: MaxNumMergeCand varies in picture by picture if it is not signaled in PPS, i.e., if it is signaled in PH, and thus it is difficult to constrain the value of the PPS syntax element pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 as follows:

The value of pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 shall be in the range of 0 to MaxNumMergeCand − 1.

Issue 2: Another issue is the redundant signaling of MaxNumTriangleMergeCand if MaxNumMergeCand is equal to 2. Because MaxNumTriangleMergeCand shall be in the range of 2 to MaxNumMergeCand, inclusive, MaxNumTriangleMergeCand must be 2 if MaxNumMergeCand is equal to 2.

Issue 3: In Spec ticket #691 of the JVET VVC bug tracker, i.e., https://jvet.hhi.fraunhofer.de/trac/vvc/ticket/691#comment:9, another concern was raised to look for a cleaner design and suggested to re-evaluate the ideas discussed during the March'19 Geneva meeting, that is, the proposal JVET-N0851 and N0447 as follows:

- Having the regular merge list derived for 6 candidates all the time regardless of signaled max_num_merge_cand (JVET-N0851)

- Having independent signaling of max_num_merge_cand and max_num_triangle_cand (JVET-N0447)

# The bugfixes for MaxNumTriangleMergeCand

Issue 1: MaxNumMergeCand is a variable in picture by picture if it is not signaled in PPS, and thus it is difficult to constrain the value of the PPS syntax element pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 as follows:

The value of pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 shall be in the range of 0 to MaxNumMergeCand − 1.

Issue 2: Another issue is the redundant signaling of MaxNumTriangleMergeCand if MaxNumMergeCand is equal to 2. Because MaxNumTriangleMergeCand shall be in the range of 2 to MaxNumMergeCand, inclusive, MaxNumTriangleMergeCand must be 2 if MaxNumMergeCand is equal to 2.

Proposed 1 for Issue 1:  If MaxNumMergeCand varies in picture by picture (i.e., if it is not signaled in PPS), the value of MaxNumTriangleMergeCand shall not be signaled in PPS as well.

Proposed 2 for Issue 2: MaxNumTriangleMergeCand is inferred to be equal to 2 in picture header without signaling in either PPS or picture headers if MaxNumMergeCand is equal to 2.

- MaxNumMergeCand > 2 implies:

  - If MaxNumMergeCand is signaled in PPS, i.e., pps_six_minus_max_num_merge_cand_plus1 > 0:

    - MaxNumMergeCand = 6 - (pps_six_minus_max_num_merge_cand_plus1 - 1)  > 2

The syntax changes to VVC Draft 7 are as described in the following table with yellow change mark:

Section 7.3.2.4 in VVC Draft 7 version 14: pic_parameter_set_rbsp( ) {; Descriptor. …. if ( pps_six_minus_max_num_merge_cand_plus1 && pps_six_minus_max_num_merge_cand_plus1 < 5 ). pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1; ue(v). }. …. Section 7.4.3.4 in VVC Draft 7 version 14: pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 equal to 0 specifies that pic_max_num_merge_cand_minus_max_num_triangle_cand is present in PHs of slices referring to the PPS. pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 greater than 0 specifies that pic_max_num_merge_cand_minus_max_num_triangle_cand is not present in PHs referring to the PPS. The value of pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 shall be in the range of 0 to MaxNumMergeCand − 1. If not present, the value of pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 is inferred to be equal to 0. Section 7.3.2.6 in VVC Draft 7 version 14: picture_header_rbsp( ) {; Descriptor. …. if( sps_triangle_enabled_flag && MaxNumMergeCand >= > 2 && !pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 ). pic_max_num_merge_cand_minus_max_num_triangle_cand; ue(v). Section 7.4.3.6 in VVC Draft 7 version 14: pic_max_num_merge_cand_minus_max_num_triangle_cand specifies the maximum number of triangular merge mode candidates supported in the slices associated with the pictur header subtracted from MaxNumMergeCand. When pic_max_num_merge_cand_minus_max_num_triangle_cand is not present, and sps_triangle_enabled_flag is equal to 1 and MaxNumMergeCand greater than or equal to 2, pic_max_num_merge_cand_minus_max_num_triangle_cand is inferred to be equal to pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 − 1. The maximum number of triangular merge mode candidates, MaxNumTriangleMergeCand is derived as follows: MaxNumTriangleMergeCand = MaxNumMergeCand − pic_max_num_merge_cand_minus_max_num_triangle_cand (87) When pic_max_num_merge_cand_minus_max_num_triangle_cand is present, the value of MaxNumTriangleMergeCand shall be in the range of 2 to MaxNumMergeCand, inclusive. When pic_max_num_merge_cand_minus_max_num_triangle_cand is not present, and the following applies: - If sps_triangle_enabled_flag is equal to 1 and MaxNumMergeCand is equal to 2, MaxNumTriangleMergeCand is set equal to 2. - Otherwise, if (sps_triangle_enabled_flag is equal to 0 or MaxNumMergeCand is less than 2), MaxNumTriangleMergeCand is set equal to 0. When MaxNumTriangleMergeCand is equal to 0, triangle merge mode is not allowed for the slices associated with the PH.

# Re-evaluation for the design of MaxNumTriangleMergeCand

In Spec ticket #691 of the JVET VVC bug tracker, i.e., https://jvet.hhi.fraunhofer.de/trac/vvc/ticket/691#comment:9, another concern was raised to look for a cleaner design and suggested to re-evaluate the ideas discussed during the March'19 Geneva meeting, that is, the proposal JVET-N0851 and N0447 as follows:

- Having the regular merge list derived for 6 candidates all the time regardless of signaled max_num_merge_cand (JVET-N0851)

- Having independent signaling of max_num_merge_cand and max_num_triangle_cand (JVET-N0447)

Proposed 3: Apply JVET-N0851 and JVET-N0447 into the current VVC specification as follows:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_six_minus_max_num_merge_cand_plus1 | ue(v) |
| pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 pps_six_minus_max_num_triangle_merge_cand_plus1 | ue(v) |

pps_six_minus_max_num_merge_cand_plus1 equal to 0 specifies that pic_six_minus_max_num_merge_cand is present in PHs referring to the PPS. pps_six_minus_max_num_merge_cand_plus1 greater than 0 specifies that pic_six_minus_max_num_merge_cand is not present in PHs referring to the PPS. The value of pps_six_minus_max_num_merge_cand_plus1 shall be in the range of 0 to 6, inclusive.

pps_six_minus_max_num_triangle_merge_cand_plus1 equal to 0 specifies that pic_six_minus_max_num_triangle_merge_cand is present in PHs referring to the PPS. pps_six_minus_max_num_triangle_merge_cand_plus1 greater than 0 specifies that pic_six_minus_max_num_triangle_merge_cand is not present in PHs referring to the PPS. The value of pps_six_minus_max_num_merge_cand_plus1 shall be in the range of 0 to 5, inclusive.

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| if( !pps_six_minus_max_num_merge_cand_plus1 ) |  |
| pic_six_minus_max_num_merge_cand | ue(v) |
| if( sps_triangle_enabled_flag && MaxNumMergeCand >= 2 && !pps_six_minus_max_num_triangle_merge_cand_plus1 !pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 ) |  |
| pic_six_minus_max_num_triangle_merge_cand pic_max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |

pic_six_minus_max_num_merge_cand specifies the maximum number of merging motion vector prediction (MVP) candidates supported in the slices associated with the PH subtracted from 6. The maximum number of merging MVP candidates, MaxNumMergeCand is derived as follows:

MaxNumMergeCand = 6 − picsix_minus_max_num_merge_cand	(85)

The value of MaxNumMergeCand shall be in the range of 1 to 6, inclusive. When not present, the value of pic_six_minus_max_num_merge_cand is inferred to be equal to pps_six_minus_max_num_merge_cand_plus1 − 1.

pic_six_minus_max_num_triangle_merge_cand specifies the maximum number of triangle merging candidates supported in the slices associated with the PH subtracted from 6. The maximum number of triangle merging candidates, MaxNumMergeCand is derived as follows:

MaxNumTriangleMergeCand = 6 – pic_six_minus_max_num_triangle_merge_cand	(85)

When pic_max_num_merge_cand_minus_max_num_triangle_cand is present, the value of MaxNumTriangleMergeCand shall be in the range of 2 to 6, inclusive.

When not present and sps_triangle_enabled_flag is equal to 1, the value of pic_six_minus_max_num_triangle_merge_cand is inferred to be equal to pps_six_minus_max_num_triangle_merge_cand_plus1 − 1.

When sps_triangle_enabled_flag is equal to 0, the value of MaxNumTriangleMergeCand is inferred to be equal to 0.

When MaxNumTriangleMergeCand is equal to 0, triangle merge mode is not allowed for the slices associated with the PH.

Besides, MaxNumMergeCand should be replaced with 6 in the mergeing candidates derivation process.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).