| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0341 |
| --- | --- |

| Title: | Non-CE4: Range of the motion vector |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Naeri Park Junghak Nam Hyeongmun Jang Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 {naeri.park, junghak.nam, hm.jang, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In current VVC draft text, the motion vector range is defined as [-2^17, 2^17 – 1]. If a motion vector exceed the range, two different operations are applied according to coding mode. Specifically, wrapping operation is applied for INTER mode, and clipping operation is for MERGE mode. In this contribution, unified MVD range control method is proposed as follow:

Method #1: Use always the clipping operation

Method #2: Use always the wrapping operation

The experimental results reportedly show no changes in coding performance and run-time compared to VTM7.0 as anchor in CTC[2].

Introduction

In HEVC, INTER mode and MERGE mode are used for inter prediction. In INTER mode, motion vector is calculated with sum of motion vector predictor (mvpLX) and motion vector difference (mvdLX) and motion vector range is [-2^15, 2^15 – 1] by wrapping operation as follows. In HEVC, since the motion vector of the MERGE mode is just using the derived motion vector predictor, wrapping operation for final motion vector is not necessary.

uLX[ 0 ] = (mvpLX[ 0 ] + mvdLX[ 0 ] + 216) % 216 mvLX[ 0 ] = ( uLX[ 0 ] >= 215 ) ? ( uLX[ 0 ] − 216 ) : uLX[ 0 ] uLX[ 1 ] = (mvpLX[ 1 ] + mvdLX[ 1 ] + 216) % 216 mvLX[ 1 ] = ( uLX[ 1 ] >= 215 ) ? ( uLX[ 1 ] − 216 ) : uLX[ 1 ]

In the current VVC 7.0, there are many modes for inter prediction. In INTER mode, motion vector is calculated with sum of motion vector predictor (mvpLX) and motion vector difference (mvdLX) and motion vector range is [-2^17, 2^17 – 1] by wrapping operation as follows. The range of the motion vector was extended according to increasing of the picture resolution.

uLX[ 0 ] = ( mvpLX[ 0 ] + mvdLX[ 0 ] + 218 ) % 218 mvLX[ 0 ][ 0 ][ 0 ] = ( uLX[ 0 ] >= 217 ) ? ( uLX[ 0 ] − 218 ) : uLX[ 0 ] uLX[ 1 ] = ( mvpLX[ 1 ] + mvdLX[ 1 ] + 218 ) % 218 mvLX[ 0 ][ 0 ][ 1 ] = ( uLX[ 1 ] >= 217 ) ? ( uLX[ 1 ] − 218 ) : uLX[ 1 ]

But, in current VVC, there are various modes in general merge mode, e. g. MMVD, DMVR. For example, motion vector of the MMVD is calculated with sum of the motion vector predictor and motion vector difference for MMVD (mmvd offset). Therefore, motion vector of the MMVD mode can be exceed given range. To prevent motion vector exceeding permitted range, motion vector is clipped within [-2^17, 2^17 – 1]. Following table shows the methods of the motion vector range control for each mode.

Table 1 Motion vector range control for each mode on HEVC and VVC

| Category | Mode | HEVC | VVC |
| --- | --- | --- | --- |
| INTER | IBC |  | Wrapping |
|  | AFFINE |  | Wrapping (CPMV) Clipping (Sub-block MV) |
|  | INTER | Wrapping | Wrapping |
| MERGE | IBC |  | N/A |
|  | AFFINE |  | Clipping (Sub-block MV) |
|  | Regular merge | N/A | N/A |
|  | MMVD |  | Clipping |
|  | CIIP |  | N/A |
|  | TPM |  | N/A |
|  | DMVR |  | Clipping |

Proposed method

In this proposal, the methods of the range control are unified. In the method #1, clipping operation is used for all cases. In the method #2, wrapping operation is used for all cases.

Method #1 Use always the clipping operation

In generally, affine, MMVD and DMVR mode is targeting motion vector refinement within given small range. Even though extreme case, motion vector with inverted sign is not proper for these modes. To use the unified range control method, clipping operation of the merge mode is extended to inter mode.

Table 2 Motion vector range control of the Method #1

| Category | Mode | VVC | Method #1 |
| --- | --- | --- | --- |
| INTER | IBC | Wrapping | Clipping |
|  | AFFINE | Wrapping (CPMV) Clipping (Sub-block MV) | Clipping (CPMV) Clipping (Sub-block MV) |
|  | INTER | Wrapping | Clipping |
| MERGE | IBC | N/A | N/A |
|  | AFFINE | Clipping (Sub-block MV) | Clipping (Sub-block MV) |
|  | Regular merge | N/A | N/A |
|  | MMVD | Clipping | Clipping |
|  | CIIP | N/A | N/A |
|  | TPM | N/A | N/A |
|  | DMVR | Clipping | Clipping |

Method #2 Use always the wrapping operation

In generally, wrapping operation is preferable than clipping operation because modulo is more simple in implementation. Since exceeding the motion vector range is very rare, clipping operation for extreme case is burden in hardware implementation. To use the unified range control method, wrapping operation is used for inter mode and merge mode.

Table 3 Motion vector range control of the Method #2

| Category | Mode | VVC | Method #2 |
| --- | --- | --- | --- |
| INTER | IBC | Wrapping | Wrapping |
|  | AFFINE | Wrapping (CPMV) Clipping (Sub-block MV) | Wrapping (CPMV) Wrapping (Sub-block MV) |
|  | INTER | Wrapping | Wrapping |
| MERGE | IBC | N/A | N/A |
|  | AFFINE | Clipping (Sub-block MV) | Wrapping (Sub-block MV) |
|  | Regular merge | N/A | N/A |
|  | MMVD | Clipping | Wrapping |
|  | CIIP | N/A | N/A |
|  | TPM | N/A | N/A |
|  | DMVR | Clipping | Wrapping |

Experimental results

The proposed method has been implemented on top of the VTM7.0 with default setting and tested using JVET SDR common test condition [2]. The experimental results reportedly show no changes in coding performance and run-time compared to VTM7.0 as anchor in CTC.

Method #1 Use always the clipping operation

|  | Over VTM 7.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |  |  |  |  |  |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 98% | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class E |  |  |  |  |  | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 99% | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 101% | 98% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 101% | 0.00% | 0.00% | 0.00% | 101% | 100% |

Method #2 Use always the wrapping operation

|  | Over VTM 7.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 99% |  |  |  |  |  |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 99% |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 98% | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 99% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 99% | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 99% | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 99% |

Conclusion

In current VVC 7.0, the motion vector range is defined as [-2^17, 2^17 – 1]. If the motion vector exceed the range, wrapping operation and clipping operation is used for INTER and MERGE, respectively. In this contribution, unified range control methods are proposed. In the method #1, clipping operation is used for all cases. In the method #2, wrapping operation is used for all cases. The experimental results reportedly show no changes in coding performance and run-time compared to VTM7.0 as anchor in CTC. It is recommended to consider this method in the next VTM.

Reference

- B. Bross, et al., “Versatile Video Coding (Draft 7)”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-P2001(JVET-P2001-vE), 16th Meeting: Geneva, CH, 1–11 Oct. 2019.

- Frank Bossen, et al., “JVET common test conditions and software reference configurations for SDR video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N1010, 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).