| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 Jan. 2020 | Document: JVET-Q0345-v1 |
| --- | --- |

| Title: | AHG6/AHG9: Signalling EAP via the ERP SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ya-Hsuan Lee, Jian-Liang Lin, Ying-Jui Chen, Chi-Cheng Ju (MediaTek) No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Jian-Liang Lin +886-3-5670766 ext. 25250 { louise.lee, jl.lin, yr.chen, cc.ju}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to enable the use of equal-area projection (EAP) for remapping 360 videos via the ERP SEI message. The EAP format is similar to the ERP format that the same guard band and wrap-around motion compensation can be applied. The only difference is the mapping function for the latitude. Therefore, to use an EAP, there is no need to define a new SEI message, and only one flag is added in the ERP SEI message to indicate the use of EAP. Moreover, PEAP has better performance for viewports that nearly 35% BD-rate reduction compared to PERP can be observed in the dynamic viewport.

# Introduction

ERP is the most general and common projection used in 360 videos. But oversampling near the poles can be observed in ERP. On the other hand, EAP [1] is also one of the common projections that regions on the sphere can maintain the same ratio of their sizes on the projected picture. Both of the ERP and EAP formats have a rectangular layout, and the mapping functions along the longitude are the same. The only difference is that EAP use a Sin function to adjust the sampling of the latitude to achieve the equal-area distribution. With the same size of the projected picture, EAP has better performance than ERP for the viewports near the equator.

# Proposed syntax and semantics

To use EAP, only one flag, erp_equal_area_projection_flag, is added in the equirectangular projection SEI message, and only the remapping equation for the latitude is modified.

The proposed syntax elements are shown highlighted compared to VVC SEI messages (Draft 2) [2].

##### 8.14.1.1.2 Projection for one sample location

Inputs to this process are:

–	pictureWidth and pictureHeight, which are the width and height, respectively, of a monoscopic projected luma picture, in relative projected picture sample units, and

–	the centre point of a sample location (hPos, vPos) along the horizontal and vertical axes, respectively, in relative projected picture sample units, where hPos and vPos may have non-integer real values.

Outputs of this process are:

–	sphere coordinates (ϕ, θ) for the sample location in degrees relative to the local coordinate axes.

The projection for a sample location is derived as follows:

–	If ErpFlag is equal to 1, the following applies:

–	If RegionWisePackingFlag is equal to 0 and erp_guard_band_flag is equal to 1, the following applies:

hPos′ = hPos − erp_left_guard_band_width	(40)
pictureWidth = pictureWidth − erp_left_guard_band_width − erp_right_guard_band_width

–	Otherwise, the following applies:

hPos′ = hPos		(41)

–	If erp_equal_area_projection_flag is equal to 0, tThe following applies:

ϕ = 180 − hPos′ * ( 360 ÷ pictureWidth )		(42)
θ = 90 − vPos * ( 180 ÷ pictureHeight )

–	Otherwise (erp_equal_area_projection_flag is equal to 1), the following applies:

ϕ = 180 − hPos′ * ( 360 ÷ pictureWidth )		(43)
θ = Asin( 1 – 2 * vPos ÷ pictureHeight )

…

#### Equirectangular projection SEI message syntax

| equirectangular_projection( payloadSize ) { | Descriptor |
| --- | --- |
| erp_cancel_flag | u(1) |
| if( !erp_cancel_flag ) { |  |
| erp_persistence_flag | u(1) |
| erp_equal_area_projection_flag | u(1) |
| erp_guard_band_flag | u(1) |
| erp_reserved_zero_2bits | u(2) |
| erp_reserved_zero_1bits | u(1) |
| if( erp_guard_band_flag = = 1 ) { |  |
| erp_guard_band_type | u(3) |
| erp_left_guard_band_width | u(8) |
| erp_right_guard_band_width | u(8) |
| } |  |
| } |  |
| } |  |

#### 8.14.2.2 Equirectangular projection SEI message semantics

…

erp_equal_area_projection_flag equal to 0 specifies that the projection used to remap the colour samples of the projected pictures onto a sphere coordinate space is an equirectangular projection. erp_equal_area_projection_flag equal to 1 specifies that the projection used to remap the colour samples of the projected pictures onto a sphere coordinate space is an equal-area projection.

erp_guard_band_flag equal to 1 indicates that the constituent picture contains guard band areas for which the sizes are specified by the syntax elements erp_left_guard_band_width and erp_right_guard_band_width. erp_guard_band_flag equal to 0 indicates that the constituent picture does not contains guard band areas for which the sizes are specified by the syntax elements erp_left_guard_band_width and erp_right_guard_band_width.

erp_reserved_zero_2bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for erp_reserved_zero_2bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of erp_reserved_zero_2bits.

erp_reserved_zero_1bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for erp_reserved_zero_1bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of erp_reserved_zero_1bits.

…

# Simulation results

The experiments are conducted on the 360Lib-10.0 [3] and the VTM-7.0 [4] followed by the JVET common test conditions (CTC) and evaluation procedures [5]. Compared to PERP, Table I demonstrates the average BD-rate reduction of the two dynamic viewports defined in the CTC as listed in Table II.

When the viewport is near the equator, PEAP has much better performance than PERP that nearly 35% BD-rate saving can be observed. For the BD-rate loss cases, they usually happen when the viewports are near the poles, but they can be improved by rotating the sphere to make the viewports close to the equator.

Table I BD-rate performance of the two dynamic viewports for PEAP compared to PERP

|  | Viewport #1 PSNR |  |  | Viewport #2 PSNR |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| SkateboardInLot | -30.46% | -26.95% | -24.94% | -34.98% | -29.07% | -29.31% |
| ChairLift | -25.28% | -18.21% | -17.47% | 29.98% | 34.68% | 28.07% |
| KiteFlite | -12.88% | -4.78% | -5.14% | -18.28% | -11.50% | -13.31% |
| Harbor | 8.15% | 1.85% | -0.65% | -25.66% | -6.08% | -5.71% |
| Trolley | -25.62% | -13.45% | -12.84% | -17.17% | -2.98% | -3.85% |
| GasLamp | -25.88% | -10.13% | -9.38% | -22.40% | -12.51% | -11.77% |
| Balboa | -13.33% | -9.47% | -9.87% | -17.70% | -13.70% | -14.48% |
| Broadway | -16.22% | -12.12% | -12.33% | -13.72% | -9.11% | -8.51% |
| Landing2 | -12.04% | -6.40% | -7.18% | -14.73% | -7.76% | -6.51% |
| BranCastle2 | -23.65% | -21.80% | -21.00% | -25.32% | -21.22% | -20.52% |
| Class S1 | -18.66% | -11.94% | -11.74% | -14.75% | -4.58% | -5.98% |
| Class S2 | -16.31% | -12.45% | -12.59% | -17.87% | -12.95% | -12.50% |
| Overall | -17.72% | -12.14% | -12.08% | -16.00% | -7.93% | -8.59% |

Table II Dynamic viewport parameters

| Class | Sequence name | V1 start (yaw, pitch) | V1 end (yaw, pitch) | V2 start (yaw, pitch) | V2 end (yaw, pitch) |
| --- | --- | --- | --- | --- | --- |
| S1 | SkateboardInLot | -180, 35 | -270, 5 | -65, -5 | 25, 25 |
| S1 | ChairliftRide | -45, -15 | 45, 15 | -201, -73 | -111, -43 |
| S1 | KiteFlite | -114, -53 | -24, -23 | 13, -41 | 103, -11 |
| S1 | Harbor | -83, -56 | 7, -26 | -140, 12 | -50, 42 |
| S1 | Trolley | 210, -18 | 300, 12 | 30, -44 | 120, -14 |
| S1 | GasLamp | -51, -11 | 39, 19 | 61, -15 | 151, 15 |
| S2 | Balboa | -4, 3 | 86, 33 | -116, -24 | -26, 6 |
| S2 | Broadway | 50, -20 | 140, 10 | -111, -41 | -21, -11 |
| S2 | Landing2 | -45, 0 | 45, 30 | -225, 0 | -135, 30 |
| S2 | BranCastle2 | 116, -15 | 206, 15 | -145, 0 | -55, 30 |

# Conclusion

In this contribution, it is proposed to signal a syntax element, erp_equal_area_projection_flag, in the ERP SEI message to make use of EAP as an alternative. Compared to the PERP, the simulation results of the viewports reportedly show that, in terms of PSNR-Y, PSNR-U, PSNR-V, PEAP achieves the average -17.72%, -12.14%, and -12.08% BD-rate performance, respectively.

# References

- Y. Ye, J. Boyce, “Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 9)”, Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-M1004, Jan. 2018, Marrakech, MA.

- J. Boyce, G. J. Sullivan, Y.-K. Wang, “Video usability information and supplemental enhancement information for coded video bitstreams (Draft 2),” Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-P2007, Oct. 2019, Geneva, CH.

- 360Lib, https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/

- VTM, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0

- P. Hanhart, J. Boyce, K. Choi, “JVET common test conditions and evaluation procedures for 360° video”, JVET-L1012, Oct. 2018, Macao, CN

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).