| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0352 |
| --- | --- |

| Title: | AHG9/AHG12: On subpicture boundary |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Junghak Nam Naeri Park Seung-hwan Kim Jaehyun Lim 19, Ynagjae-daero 11-gil, Seocho-gu, Seoul, 06772, Korea | Tel: Email: | +82-10-8505-0793 hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In the VVC draft specification, in-loop filter could be applicable on the subpicture boundary even when the subpicture is treated as a picture. If subpictures from different bitstreams are merged, undesirable mismatch could occur at the subpicture boundary. This error could be propagated into the following subpictures. Therefore, this contribution proposes three methods to prevent error propagation.

- loop_filter_across_subpic_enabled_flag[ i ] signaling depend on subpic_treated_as_pic_flag[ i ]

- Restrict pixel modification by the deblocking operation of following subpictures

- Restrict the motion compensation range within the subpicture except subpicture boundary area where reconstructed pixels could be modified by in-loop filter.

Introduction

The subpicture is proposed to support bitstream merging [1]. In current VVC specification, subpicture information is transmitted in SPS and also in-loop filtering is controlled by the transmitted syntax at the subpicture boundary. The flag which controls in-loop filter on subpicture boundary is signaled independently even when the subpicture is treated as a picture. Hence it is possible that the subpicture treated as a picture is extracted from the bitstream and merged with different bitstream. This case could cause reconstructed pixel mismatch at the subpicture boundary when the in-loop filter is enabledas described in Figure 1.

Figure 1 Mismatch caused by in-loop filtering at the subpicture boundary

Proposed method

Proposed method 1

The proposed method 1 suggest to signal loop_filter_across_subpic_enabled_flag[ i ] only when a subpicture is not treated as a picture.

| subpics_present_flag | u(1) |
| --- | --- |
| if( subpics_present_flag ) { |  |
| sps_num_subpics_minus1 | u(8) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) { |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| subpic_ctu_top_left_y[ i ] | u(v) |
| subpic_width_minus1[ i ] | u(v) |
| subpic_height_minus1[ i ] | u(v) |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| if( ! subpic_treated_as_pic_flag[ i ] ) |  |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |

loop_filter_across_subpic_enabled_flag[ i ] equal to 1 specifies that in-loop filtering operations may be performed across the boundaries of the i-th subpicture in each coded picture in the CVS. loop_filter_across_subpic_enabled_flag[ i ] equal to 0 specifies that in-loop filtering operations are not performed across the boundaries of the i-th subpicture in each coded picture in the CVS. When not present, the value of loop_filter_across_subpic_enabled_pic_flag[ i ] is set equal to 0.

Proposed method 2

The proposed method suggests to prevent pixel modification by deblock filtering operation of following subpicture at bottom and right subpicture boundary.

- 8.8.3.2 Deblocking filter process for one direction

…

- The variable filterEdgeFlag is derived as follows:

- If edgeType is equal to EDGE_VER and one or more of the following conditions are true, filterEdgeFlag is set equal to 0:

- The left boundary of the current coding block is the left boundary of the picture.

- The left boundary of the current coding block is the left or right boundary of the subpicture and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

- The left boundary of the current coding block is the left boundary of the tile and loop_filter_across_tiles_enabled_flag is equal to 0.

- The left boundary of the current coding block is the left boundary of the slice and loop_filter_across_slices_enabled_flag is equal to 0.

- The left boundary of the current coding block is one of the vertical virtual boundaries of the picture and VirtualBoundariesDisabledFlag is equal to 1.

- Otherwise, if edgeType is equal to EDGE_HOR and one or more of the following conditions are true, the variable filterEdgeFlag is set equal to 0:

- The top boundary of the current luma coding block is the top boundary of the picture.

- The top boundary of the current coding block is the top or bottom boundary of the subpicture and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

- The top boundary of the current coding block is the top boundary of the tile and loop_filter_across_tiles_enabled_flag is equal to 0.

- The top boundary of the current coding block is the top boundary of the slice and loop_filter_across_slices_enabled_flag is equal to 0.

- The top boundary of the current coding block is one of the horizontal virtual boundaries of the picture and VirtualBoundariesDisabledFlag is equal to 1.

- Otherwise, filterEdgeFlag is set equal to 1.

        - Filtering process for a luma sample using short filters

When nDp is greater than 0 and pred_mode_plt_flag of the coding unit that includes the coding block containing the sample p0 is equal to 1, nDp is set equal to 0

When nDq is greater than 0 and pred_mode_plt_flag of the coding unit that includes the coding block containing the sample q0 is equal to 1, nDq is set equal to 0:

When loop_filter_across_subpic_enabled_flag[ subPicIdxP ] is equal to 0, wherein subPicIdxP is the subpicture index containing the sample pi and the sample pi belongs to different subpicture. the filtered sample value, pi′ is substituted by the corresponding input sample value pi with i = 0..maxFilterLengthP − 1.

- 8.8.3.6.7 Filtering process for a luma sample using long filters

…

When pred_mode_plt_flag of the coding unit that includes the coding block containing the sample pi is equal to 1, the filtered sample value, pi′ is substituted by the corresponding input sample value pi with i = 0..maxFilterLengthP − 1.

When pred_mode_plt_flag of the coding unit that includes the coding block containing the sample qi is equal to 1, the filtered sample value, qi′ is substituted by the corresponding input sample value qj with j = 0..maxFilterLengthQ − 1.

When loop_filter_across_subpic_enabled_flag[ subPicIdxP ] is equal to 0, wherein subPicIdxP is the subpicture index containing the sample pi and the sample pi belongs to a different subpicture. the filtered sample value, pi′ is substituted by the corresponding input sample value pi with i = 0..maxFilterLengthP − 1.

- 8.8.3.6.9 Filtering process for a chroma sample

…

When pred_mode_plt_flag of the coding unit that includes the coding block containing the sample pi is equal to 1, the filtered sample value, pi′ is substituted by the corresponding input sample value pi with i = 0..maxFilterLengthP − 1.

When pred_mode_plt_flag of the coding unit that includes the coding block containing the sample qi is equal to 1, the filtered sample value, qi′ is substituted by the corresponding input sample value qi with i = 0..maxFilterLengthQ − 1:

When loop_filter_across_subpic_enabled_flag[ subPicIdxP ] is equal to 0, wherein subPicIdxP is the subpicture index containing the sample pi and the sample pi belongs to a different subpicture. the filtered sample value, pi′ is substituted by the corresponding input sample value pi with i = 0..maxFilterLengthP − 1.

Proposed method 3

The proposed method suggests that the motion compensation to be applied within the area, which is not affected (or modified) by loop filtering when the subpicture is treated as a picture itself.

Figure 2. Proposed motion compensation range for subpicture which is treated as a picture

The proposed motion compensation ranges is defined with considering modified pixels range from long-tap deblocking filter as Figure 3 and Figure 4.

Figure 3. worst case luma pixel modification by long-tap deblocking filter at subpicture boundary

Figure 4. worst case chroma pixel modification by long-tap deblocking filter at subpicture boundary for 4:2:0 color format

It is from subpictureLeftBoundaryPos + 7 to subpictureRightBoudnaryPos – 7 horizontally and subpictureTopBoundaryPos + 7 to subpictureBottomBoundaryPos – 3 vertically.

Figure 5. the proposed restricted motion compensation range for subpicture

The variables SubPicLeftBoundaryPos, SubPicTopBoundaryPos, SubPicRightBoundaryPos, and SubPicBotBoundaryPos are derived as follows:

if( subpic_treated_as_pic_flag[ SubPicIdx ] ) {
      SubPicLeftBoundaryPos = subpic_ctu_top_left_x[ SubPicIdx ] * CtbSizeY
      SubPicRightBoundaryPos = Min( pic_width_max_in_luma_samples − 1,
            ( subpic_ctu_top_left_x[ SubPicIdx ] + subpic_width_minus1[ SubPicIdx ] + 1 ) * CtbSizeY − 1)
      SubPicTopBoundaryPos = subpic_ctu_top_left_y[ SubPicIdx ] *CtbSizeY                                                (116)
      SubPicBotBoundaryPos = Min( pic_height_max_in_luma_samples − 1,
            ( subpic_ctu_top_left_y[ SubPicIdx ] + subpic_height_minus1[ SubPicIdx ] + 1 ) * CtbSizeY − 1)
      if( loop_filter_across_subpic_enabled_flag[ SubPicIdx ] ) {
            if( SubPicLeftBoundaryPos > 0 )
                  SubPicLeftBoundaryPos += offsetLeft
            if( SubPicRightBoundaryPos < pic_width_max_in_luma_samples − 1 )
                  SubPicRightBoundaryPos −= offsetRight
            if( SubPicTopBoundaryPos > 0 )
                  SubPicTopBoundaryPos += offsetTop
            if( SubPicBotBoundaryPos < pic_height_max_in_luma_samples − 1 )
                  SubPicBotBoundaryPos −= offsetBottom
      }
}

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).