| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0363 |
| --- | --- |

| Title: | On transform skip residual coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | J. Gan, C. Rosewarne Address line 1 Building A, 5 Talavera Rd, Macquarie Park, 2113, NSW, Australia | Tel: Email: | +61 2 9805 2844 jonathan.gan@canon.com.au chris.rosewarne@canon.com.au |
| Source: | Canon Australia Pty Ltd |  |  |

_____________________________

# Abstract

This contribution proposes removing the second pass of context-coded syntax elements from TSRC. The remaining four context-coded syntax elements are decoded in a single pass, requiring only one CABAC budget check per coefficient. It is asserted that the proposed simplification aligns the CABAC utilisation of RRC and TSRC. Losses relative to VTM-7.0 are 0.01% luma in CTC RA, and 0.37% luma in Class F RA.

# Introduction

In the VVC WD7 [1] regular residual coding (RRC) process, residual coefficients are signalled by up to four context-coded syntax elements: sig_coeff_flag, abs_level_gtx_flag[ ][ 0 ], par_level_flag, and abs_level_gtx_flag[ ][ 1 ]. These syntax elements are decoded in a first scan pass. Use of CABAC is prohibited if a bin budget of 1.75 bins/coefficient is exhausted, and the CABAC budget is checked once per coefficient.

In the transform skip residual coding (TSRC) process, residual coefficients are signalled by up to eight context-coded syntax elements: sig_coeff_flag, coeff_sign_flag, abs_level_gtx_flag[ ][ 0 ], par_level_flag, abs_level_gtx_flag[ ][ 1 ] , abs_level_gtx_flag[ ][ 2 ] , abs_level_gtx_flag[ ][ 3 ], and abs_level_gtx_flag[ ][ 4 ]. Moreover, these eight syntax elements are decoded in two scan passes. Because of the two scan passes, the CABAC budget is checked twice per coefficient.

# Proposed method

This contribution proposes removing the second pass of context-coded syntax elements from the TSRC process. The remaining four context-coded syntax elements are: sig_coeff_flag, coeff_sign_flag, abs_level_gtx_flag[ ][ 0 ], and par_level_flag. These four are decoded in a single pass, with the CABAC budget checked once per coefficient.

The proposed simplified TSRC changes the statistics of abs_remainder. Therefore, a second aspect of the proposed method changes the Rice parameter derivation for abs_remainder. A fixed Rice parameter of 0 is used.

It is asserted that the proposed simplification aligns the CABAC utilisation of RRC and TSRC.

# Experimental results

Results were obtained under the common test conditions (CTC) defined in JVET-N1010 [2] and are shown below in Table 1.

Table 1: Result of proposed method over CTC

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.02% | 0.09% | 100% | 101% |
| Class A2 | 0.01% | 0.07% | -0.02% | 100% | 101% |
| Class B | 0.02% | 0.05% | -0.01% | 100% | 101% |
| Class C | 0.05% | 0.13% | 0.10% | 100% | 100% |
| Class E | 0.05% | -0.03% | 0.15% | 101% | 100% |
| Overall | 0.03% | 0.05% | 0.06% | 100% | 101% |
| Class D | 0.04% | -0.02% | 0.02% | 100% | 101% |
| Class F | 0.47% | 0.28% | 0.40% | 100% | 101% |
|  |  |  | Random access |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.06% | 0.05% | 99% | 101% |
| Class A2 | -0.01% | 0.02% | -0.07% | 99% | 100% |
| Class B | 0.01% | 0.07% | 0.17% | 99% | 100% |
| Class C | 0.03% | -0.06% | 0.05% | 99% | 88% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.00% | 0.07% | 99% | 97% |
| Class D | 0.08% | 0.05% | -0.18% | 96% | 79% |
| Class F | 0.37% | 0.51% | 0.46% | 99% | 86% |
|  |  |  | Low delay B |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.41% | -0.20% | 109% | 143% |
| Class C | 0.01% | -0.20% | -0.17% | 110% | 135% |
| Class E | -0.25% | 0.44% | -0.17% | 106% | 168% |
| Overall | -0.07% | -0.13% | -0.19% | 109% | 146% |
| Class D | 0.09% | -0.13% | 0.02% | 110% | 129% |
| Class F | 0.39% | -0.05% | 0.96% | 108% | 167% |

Results obtained under low QP conditions as defined in JVET-P2023 [3] are shown below in Table 2.

Table 2: Result of proposed method over low QPs

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.02% | 0.03% | 121% | 124% |
| Class A2 | 0.01% | 0.00% | 0.00% | 121% | 125% |
| Class B | 0.01% | 0.00% | -0.03% | 120% | 123% |
| Class C | 0.00% | -0.05% | -0.01% | 119% | 119% |
| Class E | -0.08% | -0.16% | -0.16% | 121% | 123% |
| Overall | -0.01% | -0.03% | -0.03% | 120% | 122% |
| Class D | 0.03% | -0.05% | -0.07% | 120% | 120% |
| Class F | 0.93% | 0.58% | 0.63% | 113% | 122% |
| Class TGM | 2.72% | 2.44% | 2.49% | 114% | 123% |
|  |  |  | Random access |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.05% | 0.04% | 113% | 124% |
| Class A2 | 0.00% | 0.00% | 0.00% | 112% | 127% |
| Class B | 0.01% | -0.01% | -0.04% | 113% | 125% |
| Class C | 0.01% | -0.05% | -0.03% | 114% | 118% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.00% | -0.01% | 113% | 123% |
| Class D | 0.05% | 0.09% | 0.00% | 113% | 113% |
| Class F | 0.91% | 0.65% | 0.50% | 111% | 134% |
| Class TGM | 2.57% | 1.82% | 1.87% | 111% | 152% |
|  |  |  | Low delay B |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.01% | -0.01% | 115% | 128% |
| Class C | 0.02% | -0.01% | 0.00% | 115% | 125% |
| Class E | -0.01% | -0.03% | 0.04% | 115% | 133% |
| Overall | 0.01% | -0.01% | 0.01% | 115% | 128% |
| Class D | 0.01% | 0.04% | 0.02% | 115% | 121% |
| Class F | 0.43% | 0.45% | 0.37% | 110% | 144% |
| Class TGM | 1.45% | 1.17% | 1.18% | 110% | 167% |

# Additional results (informative)

It is asserted that a fixed Rice parameter of 0 is most suitable for binarizing abs_remainder. Tables 3 and 4 show coding results obtained with fixed Rice parameters of 1 and 2 respectively.

Table 3: Result of proposed method with Rice parameter 1 over CTC

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.06% | 0.11% | 99% | 99% |
| Class A2 | 0.03% | 0.04% | -0.02% | 100% | 99% |
| Class B | 0.05% | 0.06% | -0.04% | 99% | 100% |
| Class C | 0.14% | 0.21% | 0.25% | 98% | 98% |
| Class E | 0.11% | -0.02% | 0.19% | 99% | 99% |
| Overall | 0.08% | 0.08% | 0.09% | 99% | 99% |
| Class D | 0.14% | 0.12% | 0.17% | 99% | 97% |
| Class F | 0.97% | 0.54% | 0.64% | 99% | 100% |
|  |  |  | Random access |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.11% | 0.00% | 99% | 100% |
| Class A2 | 0.00% | 0.01% | 0.04% | 99% | 100% |
| Class B | 0.03% | -0.06% | 0.07% | 99% | 99% |
| Class C | 0.12% | 0.01% | 0.13% | 98% | 87% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.01% | 0.07% | 99% | 96% |
| Class D | 0.18% | -0.11% | 0.19% | 95% | 79% |
| Class F | 0.51% | 0.56% | 0.29% | 98% | 85% |

Table 4: Result of proposed method with Rice parameter 2 over CTC

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.05% | 0.13% | 100% | 99% |
| Class A2 | 0.03% | 0.04% | -0.04% | 100% | 100% |
| Class B | 0.06% | 0.06% | 0.01% | 100% | 101% |
| Class C | 0.23% | 0.33% | 0.31% | 100% | 102% |
| Class E | 0.16% | 0.01% | 0.23% | 100% | 99% |
| Overall | 0.11% | 0.11% | 0.13% | 100% | 100% |
| Class D | 0.22% | 0.21% | 0.29% | 100% | 101% |
| Class F | 2.06% | 1.49% | 1.68% | 100% | 101% |
|  |  |  | Random access |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.05% | 0.03% | 99% | 100% |
| Class A2 | 0.00% | 0.05% | 0.06% | 98% | 98% |
| Class B | 0.04% | 0.06% | 0.19% | 99% | 99% |
| Class C | 0.20% | 0.16% | 0.27% | 98% | 87% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | 0.08% | 0.15% | 99% | 96% |
| Class D | 0.29% | 0.09% | 0.32% | 95% | 77% |
| Class F | 1.17% | 1.07% | 1.00% | 98% | 86% |

As shown above, changing the Rice parameter for abs_remainder from 1 to 0 results in a coding improvement of approximately -0.04% in luma RA. Table 5 below shows the result of changing the Rice parameter from 1 to 0 for the current TSRC method (ie, eight context-coded syntax elements). There are only minor coding improvements (-0.01% luma RA).

Table 5: Result of current TSRC with Rice parameter 0 over CTC

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.04% | 0.05% | 100% | 100% |
| Class A2 | 0.00% | -0.01% | -0.02% | 101% | 100% |
| Class B | 0.00% | 0.03% | -0.08% | 100% | 100% |
| Class C | -0.03% | 0.04% | 0.09% | 100% | 100% |
| Class E | 0.01% | -0.07% | -0.06% | 100% | 100% |
| Overall | -0.01% | 0.00% | -0.01% | 100% | 100% |
| Class D | -0.05% | 0.04% | -0.06% | 100% | 101% |
| Class F | -0.08% | -0.02% | 0.13% | 100% | 101% |
|  |  |  | Random access |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | 0.01% | 100% | 100% |
| Class A2 | -0.04% | 0.03% | 0.02% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.07% | 100% | 100% |
| Class C | -0.01% | 0.01% | -0.01% | 100% | 87% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.01% | 0.03% | 100% | 97% |
| Class D | -0.05% | -0.28% | 0.23% | 97% | 79% |
| Class F | 0.04% | 0.20% | 0.18% | 99% | 85% |

Another possibility for simplification is to use the RRC process everywhere, including transform skip blocks. However, Table 6 shows significant coding losses even in (camera-captured content) CTC, with 0.15% loss in luma RA.

Table 6: Result of removal of TSRC over CTC

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.51% | -0.36% | -0.56% | 100% | 100% |
| Class A2 | 0.04% | 0.13% | 0.03% | 100% | 100% |
| Class B | 0.11% | 0.07% | 0.04% | 100% | 101% |
| Class C | 0.49% | 0.40% | 0.45% | 101% | 101% |
| Class E | 0.25% | 0.04% | 0.52% | 100% | 100% |
| Overall | 0.27% | 0.08% | 0.11% | 100% | 100% |
| Class D | 0.37% | 0.26% | 0.42% | 101% | 101% |
| Class F | 3.81% | 3.08% | 3.22% | 100% | 104% |
|  |  |  | Random access |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | -0.05% | -0.26% | 100% | 100% |
| Class A2 | -0.04% | 0.14% | 0.15% | 100% | 100% |
| Class B | 0.08% | 0.01% | 0.23% | 100% | 100% |
| Class C | 0.39% | 0.19% | 0.04% | 99% | 88% |
| Class E |  |  |  |  |  |
| Overall | 0.15% | 0.07% | 0.07% | 100% | 97% |
| Class D | 0.54% | -0.19% | 0.19% | 96% | 79% |
| Class F | 2.82% | 2.37% | 2.48% | 98% | 86% |

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 7)”, JVET-P2001, Geneva, CH, October 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, March 2019.

- T. Nguyen, T. C. Ma, A. Nalci, “Description of Core Expeirment 3 (CE3): Lossless Coding”, JVET-P2023, Geneva, CH, October 2019.

# Patent rights declaration(s)

Canon Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Australia Pty Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).